Customizing the Diskette Configuration

Normally, TeleDisk can determine a system's configuration without

intervention.  However, there are cases where add-on diskette

adapters make it impossible to accurately sense the system setup.

To determine if TeleDisk has correctly sensed your system's con-

figuration, use the VIEWCONF command that is included with

TeleDisk.

If necessary, a text file, named DISKETTE.CFG can be prepared

using any text editor or word processing program in non-document

mode.  This file can be placed anywhere in the DOS command search

path and will be automatically located by TeleDisk.

The layout of the configuration file is straightforward.  Each

line in the file begins with the letter of the diskette drive

being described, followed by a colon (:).  The rest of the line

contains keywords and numeric values which specify the configur-

ation for that drive.  Each item on the line is separated by one

or more spaces.  In most cases, keywords can be abbreviated.

A configuration file line can contain any of the following key-

words.  The minimum abbreviation for each keyword is shown in

parentheses:

360K (360K) specifies a 5.25" 360K drive.

720K (720K) specifies a 3.5" 720K drive.  Most utilities also

support the less common 5.25" 720K quad-density drive.

1.2M (1.2) specifies a 5.25" 1.2M drive.

1.44M (1.4) specifies a 3.5" 1.44M drive.

2.88M (2.8) specifies a 3.5" 2.88M drive.

8INCH (8I) specifies an 8" drive.

Note that the drive type must be specified.

BIOS (B) specifies the BIOS drive number for the drive. The

word BIOS is followed by a  a number between 0 and 127.  This

number is also the same one used for the /D: parameter when

1

DRIVER.SYS is used to support the drive in the system's

CONFIG.SYS file.

PORT (P) specifies the base I/O port of the adapter connected

to the drive.  The first or primary adapter is always des-

cribed by the number 3F0H (observe the "H" at the end of a

hexadecimal number.)  Secondary adapters are usually ad-

dressed by the number 370H and sometimes by 360H or 3E0H.  If

not specified, a PORT value of 3F0H is assumed.

UNIT (U) specifies the physical unit number of the drive on

the adapter.  This unit number must be within the range of 0

to 3 and should not be confused with the BIOS unit number.

DMA (DM) specifies the Direct Memory Access channel which

services the adapter.  If omitted, DMA channel 2 is assumed.

INTERRUPT or IRQ (I) specifies the system interrupt which

services the adapter. If omitted, interrupt 6 is assumed.

Note that disk controller boards jumpered for IRQ 2 shoudl be

specified as INTERRUPT 9 for best performance.

SEEK (S) specifies the track-to-track stepping time of the

drive in milliseconds.  If omitted, 6 milliseconds is

assumed.

CHANGE (CH) specifies that the drive possesses diskette

change detection circuitry. If this term is omitted, the

drive is assumed not to support change detection.  Usually,

1.2M, 1.44M and 2.88M drives  have change detection

circuitry; 360K, 720K and 8 inch drives do not.

DUAL (DU) is applicable for 1.2M 5.25"drives only. Some in-

expensive secondary adapters require that 1.2M drives be

configured for dual-speed operation in which the spindle

motor spins at 360 revolutions per minute for high-density

operation and 300 revolutions per minute for low-density

operation. If omitted, normal single-speed operation is

assumed.

COMPATICARD (CO) specifies that the drive is connected to a

MicroSolutions Compaticard I(tm) or Compaticard II(tm) adap-

ter.  If the Compaticard IV(tm) is being used, this term

should not be specified.

2

DAA (DA) indicates that the drive is connected to an IBM PS/2

Diskette Adapter/A. This adapter is present on external

drives on Microchannel systems only and is assumed not to be

present.

A number is required to follow the following words:

BIOS

PORT

UNIT

DMA

INTERRUPT

SEEK

A number is assumed to be a decimal value unless suffixed by the

letter "H" which indicates a hexadecimal quantity.  Thus, 1000

and 3E8H refer to the same quantity.

Here is an example of a DISKETTE.CFG file:

A: BIOS 0 UNIT 0 1.44M CHANGE

B: BIOS 1 UNIT 1 1.2M CH

Q: BIOS 4 PORT 370H UNIT 0 360K

There are two other items which may start a configuration line.

DEFAULT: initializes the configuration to the values that

would be obtained if automatic configuration sensing were to

be performed.  Use of DEFAULT is useful if automatic sensing

has correctly sensed all but one or two drives. For example,

suppose that the configuration given above is correctly

sensed, with the exception of drive Q:. The DISKETTE.CFG file

could be stated as:

DEFAULT:

Q: BIOS 4 PORT 370H UNIT 0 360K

DEFAULT must appear as the only item on a line.

NOTE: causes the remainder of a line to be ignored. Comments

may be inserted into the DISKETTE.CFG file in this manner.

For example:

3

NOTE:  Automatic sensing gets everything but

NOTE:  drive Q: connected to a special second

NOTE:  adapter.

DEFAULT:

Q: BIOS 4 PORT 370H UNIT 0 360K

NOTE:  This is the end of the configuration file.

(end)

4