MOUSE.DOC - Programming Interface

The following provides information to programmers who want to

include the AT&T MOUSE 6300 in their application programs. This

document includes descriptions of the graphics and text cursors,

and descriptions of the mouse functions and extended mouse

functions.

This protocol is compatible with the protocol of Microsoft Corp.

as it is documented in the installation and operation manual

published by Microsoft Corp. entitled Microsoft Mouse for the

IBM Personal Computer.

These notes apply to the Mouse Driver (mouse.com) release 1.04a.

The programming interface uses the 8086 software interrupt 51.

Parameter values are passed and returned via 8086 registers.

This interface can be used from assembly language programs or

from high level language programs such as C, Modula-2, or

Pascal.

Programming Interface from the GWBASIC Interpreter

It is possible to make a mouse system call from a BASIC program

running under the GWBASIC interpreter.  The following shows how

to make these calls.

Insert an initialization sequence such as:

10  DEF SEG=0

20  MSEG=256*PEEK(51*4+3)+PEEK(51*4+2)

30  MOUSE=256*PEEK(51*4+1)+PEEK(51*4)+2

40  DEF SEG=MSEG

Be sure that the statements appear before any calls to mouse

functions.  Then use the CALL statement to make the call.

CALL MOUSE(M1%,M2%,M3%,M4%)

where MOUSE is the variable containing the entry offset of the

mouse software, and M1%, M2%, M3%, and M4% are the names of the

integer variables you have chosen for parameters in this call.

They correspond to the values for AX, BX, CX, and DX which are

described in this document.  As an example:

100  'Set minimum and maximum horizontal

position to'(320,100)

200  M1%=7 'function number is 7

300  M3%=0 'minimum coordinate

400  M4%=639 'maximum coordinate

500  CALL MOUSE(M1%,M2%,M3%,M4%)

**Cursor Descriptions - Text and Graphics Cursors**

The characteristics of the cursor are determined by Functions 9

and 10. MOUSE 6300 has both text and graphics cursors available.

The graphics cursor is a shape that moves over the images on the

screen. The shape can include an arrow, a sight mark, or others.

The software text cursor is a character attribute that moves

from character to character on the screen. The hardware text

cursor is a moving, sometimes flashing block which also moves

from character to character on the screen. The three cursors are

exclusive - only one can be on the screen at one time. You have

the option of choosing which cursor will appear on the screen

and of switching from one cursor to the other.

Note that when a mouse function refers to the graphics cursor

location it is referring to the point on the screen that lies

directly under the cursor's target area. The target area is the

point in the cursor block that the mouse software uses to

determine the cursor coordinates. The upper left corner of the

cursor block is used to determine the coordinates.

**Notice to System Programmers**

System programmers who intercept the keyboard hardware interrupt

(int 9) or the video i/o software interrupt (int 10h) should

exercise utmost care, and note the following:

The mouse driver itself intercepts these two interrupts.

The mouse driver expects to receive all the codes which are

relevant for the mouse from the keyboard.  If a program which

intercepts the keyboard interrupt is loaded after the driver the

following codes from the keyboard should fall through and be

left available for the mouse driver:

-2-

LOGITECH - mouse.doc for AT&T                      v.1.04a

77h, 78h, 79h, 75h, 76h, 7ch, 7dh,

F7h, F8h, F9h,

FEh and the next 2 codes which follow an

occurrence of FEh (FEh is the prefix character

for mouse movement reports).

Codes 74h and F4h are reserved for future use by mouse

software.

All other codes are left by the mouse driver to the next

interrupt handler on the chain (presumably the ROM resident

keyboard interrupt handler).

With regard to video (int 10h) the program should guarantee that

video mode changes (function 0) are intercepted by the mouse

driver. The mouse driver guarantees consistency of the screen

contents to programs that write on the screen via int 10h (or

via DOS).

Some programs write directly on the video memory.  In this case

the application program should use function 2 (hide cursor) or

16 (conditional hide cursor) before writing, then function 1

(show cursor) after writing.  GWBasic is an example of such a

program.

To check for the presence in memory of the LOGITECH mouse driver

one should check for a "signature" at the offset 10h from the

entry of software interrupt 51.  The signature is the string

"LOGITECH MOUSE DRIVER".

-3-

LOGITECH - mouse.doc for AT&T                      v.1.04a

List of Mouse Functions

The following functions apply to MOUSE 6300. These functions

will be described in greater detail in subsequent sections.

Microsoft Compatible Functions

Functions                           Function Number

Mouse Initialization                     0

Show Cursor                              1

Hide Cursor                              2

Get Mouse Position & Button Status       3

Set Mouse Cursor Position                4

Get Button Press Information             5

Get Button Release Information           6

Set Minimum & Maximum X Position         7

Set Minimum & Maximum Y Position         8

Define Graphics Cursor Block             9

Define Text Cursor                       10

Read Mouse Motion Counters               11

Define Event Handler                     12

Light Pen Emulation Mode On              13

Light Pen Emulation Mode Off             14

Set Mouse Motion/Pixel Ratio             15

Conditional Hide Cursor                  16

LOGITECH Specific Functions

Get Acceleration                         29

Set Acceleration                         30

Set Grid                                 31

Get Mouse Mode                           32

Set Mouse Mode                           33

Get Keyboard Emulation Configuration     34

Set Keyboard Emulation Configuration     35

Note: the LOGITECH Specific Functions are not compatible

with Microsoft mouse drivers, and their use should be

avoided in applications that need to be able to work with

other mouse drivers.

-4-

LOGITECH - mouse.doc for AT&T                      v.1.04a

**Description of the Mouse Functions**

The following descriptions of the mouse functions specify the

required input and the expected output of each function. The

descriptions also include any special circumstances involved

with each function. The input and output parameters are

specified as values for 8086 registers.

**Mouse Initialization - Function 0**

This function describes whether the mouse hardware and software

are installed. The mouse status is 0 if the mouse hardware and

software are not installed and -1 if the hardware and software

are installed.

Input                   Output

AX = 0                  AX = mouse status

BX = number of buttons

**Show Cursor - Function 1**

This function increments the internal cursor flag. If the flag

is 0 it displays the cursor on the screen. The cursor tracks the

motion of the mouse, changing position as the mouse changes

position.

Input                   Output

AX = 1                  ----

**Hide Cursor - Function 2**

This function removes the cursor from the screen and decrements

the internal cursor flag. Although the cursor is hidden it still

tracks the motion of the mouse, changing position as the mouse

changes position.

Input                   Output

AX = 2                  ----

-5-

LOGITECH - mouse.doc for AT&T                      v.1.04a

**Get Mouse Position & Button Status - Function 3**

This function reports the status of the buttons. It also reports

the position of the cursor horizontally and vertically. The

button status is a single integer value with bit 0 representing

the left button and bit 1 the right. When the button is down a

bit is 1 and when the button is up a bit is 0.

Input                   Output

AX = 3                  BX = button status

CX = horizontal cursor

position

DX = vertical cursor

position

**Set Mouse Cursor Position - Function 4**

This function sets the cursor to the specified horizontal and

vertical positions on the screen. The new values must be within

the specified ranges of the virtual screen. If you are not using

a high resolution screen the values are rounded to the nearest

values permitted by the screen for horizontal and vertical

positions.

Input                   Output

AX = 4                  ----

CX = new cursor position

(horizontal)

DX = new cursor position

(vertical)

**Get Button Press Information - Function 5**

This function reports on the current button status.  It gives a

count of button presses since the last call to Function 5, and

it gives the horizontal and vertical position of the cursor the

last time the button was pressed.

The BX parameter indicates which button was pressed. If BX is 0,

the left button is checked.  If BX is 1, the right button is

checked. The button status is a single integer

-6-

LOGITECH - mouse.doc for AT&T                      v.1.04a

value. Bit 0 represents the left button and bit 1 the right. If

the button is down a bit is 1 and if it is up a bit is 0.

Input                   Output

AX = 5                  AX = button status

BX = button             BX = number of button

presses

CX = cursor at last

press

(horizontal)

DX = cursor at last

press

(vertical)

**Get Button Release Information - Function 6**

This function reports on the current button status. It reports a

count of the button releases since the last call to Function 6,

and it gives the horizontal and vertical position of the cursor

since the last time the button was released.

The BX parameter specifies which button is checked. If BX is 0,

the left button is checked. If BX is 1, the right button is

checked. The button status is a single integer value. Bit 0

represents the left button and bit 1 represents the right

button. If a button is down a bit is 1 and if a bit is 0 a

button is up.

**Set Minimum & Maximum X Position - Function 7**

This function sets the minimum and maximum possible cursor

positions on the screen. All cursor movement is restricted to

this area once the area is set. The virtual screen determines

the maximum and minimum values.

If the cursor is outside the defined area when the call is made,

it moves to just inside the area. If the minimum value is

greater than the maximum the two values are exchanged.

Input                   Output

AX = 7                  ----

CX = minimum position

DX = maximum position

-7-

LOGITECH - mouse.doc for AT&T                      v.1.04a

**Set Minimum & Maximum Y Position - Function 8**

This function sets the minimum and maximum possible cursor

positions on the screen. All cursor movement is restricted to

this area once the area is set. The virtual screen determines

the maximum and minimum values.

If the cursor is outside the defined area when the call is made,

it moves to just inside the area. If the minimum value is

greater than the maximum the two values are switched.

Input                   Output

AX = 8                  ----

CX = minimum position

DX = maximum position

**Define Graphics Cursor Block - Function 9**

This function determines what the shape and color of the cursor

will be when it is in graphics mode, and it identifies the

center of the cursor. The cursor hot spot values must be within

the range of -16 to 16. They define one pixel within the cursor.

Input                   Output

AX = 9                  ----

BX = horizontal cursor hot spot

CX = vertical cursor hot spot

DX = pointer to screen and cursor masks

**Define Text Cursor - Function 10**

This function selects the software or hardware cursor. When the

hardware cursor is selected this function sets the first and

last scan lines which will be shown on the screen. The value of

the parameter BX selects the cursor type. If the value is 0, the

software text cursor is selected. If the value is 1, the

hardware text cursor is selected. When the software text cursor

is selected, the values of parameters CX and DX specify the

screen and cursor masks. When the hardware cursor is selected,

the parameters of CX and DX must contain the line numbers of the

first and last scan line in the cursor to be shown on the

screen.

-8-

LOGITECH - mouse.doc for AT&T                      v.1.04a

Input                   Output

AX = 10                 ----

BX = select cursor

CX = screen mask value/scan line start

DX = cursor mask value/scan line stop

**Read Mouse Motion Counters - Function 11**

NOTE: In mouse.com there is an accelerator which affects fast

mouse movement. Cursor movement is expanded by the accelerator

if hand movement is over a certain speed threshold. With slow

mouse movement the mouse resolution is 200 Steps per Inch (SPI).

With fast mouse movement it is 400 SPI. Information in the

following sections changes according to whether the accelerator

is in effect or not.

This function returns the horizontal and vertical step count,

which is the distance the mouse has moved in 1/200 inch

increments, since the last call to this function.  With the

Default Accelerator installed the ratio of mouse to cursor

movement is variable between 1/200 and 1/400.  The step count is

always within the range -32768 to 32767. A positive horizontal

count specifies a motion to the right while a positive vertical

count specifies a motion to the bottom of the screen. The step

count is set to 0 after the call is completed.

Input                   Output

AX = 11                 CX = horizontal count

DX = vertical count

**Define Event Handler - Function 12**

This routine sets the call mask and subroutine address for the

mouse software interrupts. The software interrupts stop

execution of your program and call the specified subroutine

whenever one or more of the conditions defined by the call mask

occur. The call mask is a single integer value which defines the

conditions which will cause an interrupt. Each

bit in the call mask corresponds to a specific condition:

-9-

LOGITECH - mouse.doc for AT&T                      v.1.04a

Mask Bit                Condition

0                       change cursor position

1                       press left button

2                       release left button

3                       press right button

4                       release right button

5-15                    not used

Input                   Output

AX = 12                 ----

CX = call mask

DX = address offset to subroutine

**Light Pen Emulation Mode On - Function 13**

This function enables light pen emulation by the mouse. When the

mouse emulates the light pen, calls to the pen function will

return the cursor position at the last pen down which is

controlled by the mouse buttons. The pen is down when both

buttons are down. The pen is off the screen when both buttons

are up.

Input                   Output

AX = 13                 ----

**Light Pen Emulation Mode Off - Function 14**

This function disables the light pen emulation mode. When light

pen simulation is disabled, calls to the pen function return

information about the light pen only.

Input                   Output

AX = 14                 ----

**Set Mouse Motion/Pixel Ratio - Function 15**

This function sets the mouse motion to screen pixel ratio.

-10-

LOGITECH - mouse.doc for AT&T                      v.1.04a

The horizontal and vertical ratios specify the amount of mouse

motion at 8 pixels with the values falling within the range of 1

to 32767. The default values are 8 steps to 8 pixels

horizontally and 16 steps to 8 pixels vertically. This is

equivalent to 3.2 inches of horizontal mouse movement and 2.0

inches of vertical mouse movement.

Input                   Output

AX = 15                 ----

CX = horizontal step to pixel ratio

DX = vertical step to pixel ratio

**Conditional Hide Cursor - Function 16**

This function allows the user to define an area on the screen

within which the mouse will turn off. Performing the Show Cursor

Function - Function 1 - resets the region and turns the mouse

back on. This function is used to guard a portion of the screen

which your program is about to update.

Input                   Output

AX = 16                 ----

CX = left margin

DX = top margin

SI = right margin

DI = bottom margin

**Get Acceleration - Function 29**

This mode returns the acceleration of the mouse in real mouse

mode.

Input                   Output

AX = 29                 BX = acceleration type

ES:DX = acceleration

vector

address

**Set Acceleration - Function 30**

This mode establishes the acceleration of the mouse in real

mouse mode.

-11-

LOGITECH - mouse.doc for AT&T                      v.1.04a

Input                   Output

AX = 30                 ----

BX = acceleration type

ES:DX = acceleration vector

address

Acceleration type

BX = 0   default accelerator

BX = 1   use the given accelerator vector

BX = 2   no accelerator

Acceleration is an array of 128 bytes. Each element contains the

value to be substituted as relative mouse movements count when

the value used as index for the array is received from the mouse

The Default Acceleration Table is built up in the following

way:

Displacements 0,1,2 and 3 are left unchanged.

Displacements 4 through 9 are mapped into 5

through 15.

Displacements 10 through 14 are mapped into 18

through 26.

Displacements 15 through 19 are mapped into 29

through 37.

Displacements 20 through 127 are mapped into 40

through 254.

**Set Grid - Function 31**

This function allows the user to set the cursor movements so

that they move on the points of a grid on the screen.

Input                   Output

AX = 30

BX = horizontal grid value

CX = vertical grid value

-12-

LOGITECH - mouse.doc for AT&T                      v.1.04a

Grid Value:             0 = 1 = no grid

distance of points

Implementation restriction: Only values which are powers of 2

(2,4,8,16...) are allowed as grid values.

The set grid function does not apply in text cursor mode.

**Get Mouse Mode - Function 32**

This function distinguishes between realmouse mode and keyboard

emulation mode. The value returned in BX is 0 when the mode is

mousebase and 1 when it is keybase.

Input                   Output

AX = 32                 BX = mode

**Set Mouse Mode - Function 33**

This function distinguishes between real mouse mode and keyboard

emulation mode. A bit is 0 when real mouse mode is set and 1

when keyboard emulation mode is set.

Input                   Output

AX = 33                 ----

BX = mode

**Get Keyboard Emulation Configuration - Function 34**

This mode gets the configuration for keyboard emulation mode.

Input                   Output

AX = 34                 ES:DX = parameter

ES:DX = address of      structure loaded with

parameter structure     config-

uration values

-13-

LOGITECH - mouse.doc for AT&T                      v.1.04a

Parameter Structure

WORD    length of left button press string

DWORD   pointer to left button press string

WORD    length of left button release string

DWORD   pointer to left button release string

WORD    length of middle button press string

DWORD   pointer to middle button press string

WORD    length of middle button release string

DWORD   pointer to middle button release string

WORD    length of right button press string

DWORD   pointer to right button press string

WORD    length of right button release string

DWORD   pointer to right button release string

WORD    length of forward movement string

DWORD   pointer to forward movement string

WORD    length of backward movement string

DWORD   pointer to backward movement string

WORD    length of left movement string

DWORD   pointer to left movement string

WORD    length of right movement string

DWORD   pointer to right movement string

WORD    x scale

WORD    y scale

Pointers are offset, segment in the 8086 conventions (offset

lower word, segment upper word).

Lengths are interpreted as buffer length in input.  They will

contain actual string lengths in the output.

Length of the button strings cannot exceed 30.

Length of the movement strings cannot exceed 4.

A length of 0 is legal. In this case the corresponding pointer

will not be used. The value for the corresponding item will not

be returned.

-1 will be returned in length if the buffer is shorter than the

actual string.

**Set Keyboard Emulation Configuration - Function 35**

This mode sets the configuration for keyboard emulation mode.

-14-

LOGITECH - mouse.doc for AT&T                      v.1.04a

Input

AX = 35

BX = configuration type

ES:DX = address of parameter structure

containing configuration values

Configuration Type

0 = use specified configuration

1 = use default configuration; parameter

structure is not used

2 = use "reserved" configuration (for LOGITECH

use only); parameter structure is not used.

Parameter Structure

(See chart for Function 34 - Get Keyboard Emulation Config.)

Pointers are offset, segment in the 8086 conventions (offset

lower word, segment upper word).

The length of the button strings cannot exceed 30.

The length of the movement strings cannot exceed 4.

Longer strings will be truncated.

A length of 0 is legal. The corresponding pointer will not be

used and no string will be attached to the specific button or

movement.

Function 35 will not alter the mouse mode. If a new

configuration is issued while the mouse is in keyboard emulation

mode, the new configuration will be in effect immediately. If a

new configuration is issued while the mouse is in real mouse

mode, the new configuration will be in effect at the next

setmouse mode which specifies keyboard emulation mode.

The default configuration in keybase mode is keyboard cursor

keys for mouse movement, <CR> for the left mouse button, <esc>

for the right mouse button, and 5 for both the horizontal and

vertical mouse movement scale.