Softpaq:  SP1454

Part Number: N/A

Title: Fix Partition Utility (FixPart.EXE)  This program updates a corrupted

file allocation table (FAT) using the backup copy of the FAT stored on the

hard drive.  The Operating System files are also replaced with the

appropriate set for DOS 6.2 or Windows 95 systems.

Version:  N/A

CATEGORY:  Drivers / Miscellaneous

EFFECTIVE DATE:  15 December, 1995

PRODUCTS AFFECTED:

All Compaq products with a serial number range between x502xxxxxxxxx and

x547xxxxxxxxx, except the Presario 7100 Family and the LTE 5000 Family.

OPERATING SYSTEM:  DOS/Windows95

SYSTEM CONFIGURATION:  Windows 3.1X/Windows95

SUPERSEDES: None

DESCRIPTION:

This utility will correct any changes made to the file allocation table (FAT)

and/or the operating system files as a result of updating the System Partition

via Compaq Diagnostics or Configuration Utility. The utility is designed for use

on units operating under DOS 6.2 or Windows 95, only.  The utility will

autoexecute upon booting to the diskette created from the SoftPaq utility.

HOW TO USE:

1.  Obtain 1 formatted 1.44 MB diskette.

2.  Download the SoftPaq to a directory on your hard drive and change to

that directory.  The file that is downloaded is SP1454.EXE.

3.  From that drive and directory, execute the SoftPaq file:

SP1454 [ENTER]

You will be prompted to place the formatted 1.44 MB diskette into a

diskette drive to create the diskette.

4.  After the diskette is created, you may delete the downloaded file

SP1454.EXE if you wish.

5.  Insert the diskette into the computer to be tested and reboot the

computer.  The utility will automatically execute and provide the

required information on screen.  After completing the process,

remove the diskette and power cycle the computer to resume normal

operations.

Copyright 1995, Compaq Computer Corporation.  All rights reserved.

Product names mentioned herein may be trademarks and/or registered trademarks of their respective companies.