�����������������������������������������������Ŀ

�  README.DOC for DOS version 2.11 release #2   �

�������������������������������������������������

1. AT&T 640 * 400 GRAPHIC MODE

This system supports	AT&T 640 * 400	high resolution graphics mode.

On this diskette you will find the following files.

������������������������������������������������������������������Ŀ

� HIGRP.COM  �	Sets AT&T CGA 640 * 400 graphics 80 * 25 text mode �

������������������������������������������������������������������Ĵ

� TEXT50.COM �	Sets AT&T CGA 640 * 400 graphics 80 * 50 text mode �

��������������������������������������������������������������������

To use the AT&T 640 * 400 graphics mode with popular software packages,

FRAMEWORK, AUTOCAD, etc., it is necessary to configure the software from

one of their own configuration menus.

2. REAL TIME CLOCK

CLOCKDEV.SYS is DOS installable device driver which supports a clock/calendar

chip. When installed, all time and date functions are directed to the driver

which reads and writes date to the clock/calendar chip.

Install: Include the line

�����������������������Ŀ

� DEVICE = CLOCKDEV.SYS �

�������������������������

in the system CONFIG.SYS file.

Use	 : Use the normal DOS date and time commands.

The clock/calendar will be automatically read and set.

- end -