**********************************************

***            Documentation for           ***

***     The Speech Thing Device Driver     ***

***       STDRIVER.SYS Version 2.60        ***

***      Copyright 1988 by COVOX INC.      ***

***            February 9, 1989            ***

***      Program and instructions by       ***

***           Kevin C. Gevatosky           ***

**********************************************

///////////////////////////////////////////////////////////////////////////////

INTRODUCTION

///////////////////////////////////////////////////////////////////////////////

The Speech Thing device driver supports playback of all Voice Master*

digitized data files including, 'bit-wise', 'nybble-wise' and 'byte-wise' data

types. The driver also supports synthesized speech output generated by the

Smooth Talker* programs from 'First Byte'. Although LOADPLAY and STALK are the

primary interface programs used to playback data through the Speech Thing,

they are mainly designed for use with interpreted BASIC and are difficult, at

best, to use with other languages or application programs. The device driver

program, "STDRIVER.SYS", was designed to overcome this difficulty and allow

the Speech Thing to be employed in a wide variety of languages and operating

environments. A device driver can also be accessed directly from DOS in much

the same way as disk files. For instance, ASCII files can be sent to the

device driver using DOS's 'TYPE' or 'COPY' command and Smooth Talker will then

promptly speak them back. Even a Voice Master pre-digitized data file can be

copied directly to the driver for playback. Well, one might ask, "Since

STDRIVER.SYS can do all this, why bother with the LOADPLAY or STALK programs?"

There are a couple of reasons,  but mainly, LOADPLAY and STALK are easier to

use from BASIC than the device driver and BASIC is the computer language that

is most used by microcomputerists, especially novices. In fact, as will soon

be seen, programs which use the device driver can be rather complex and we

recommend that only experienced programmers ever attempt such feats. So please

note: If you do not understand the examples given in this documentation, then

DO NOT CALL US FOR ASSISTANCE BECAUSE WE DO NOT OFFER COURSES IN PROGRAMMING.

On the other hand, if you are an experienced programmer and are having

difficulty because of some fault or 'bug' inherent to the STDRIVER.SYS program,

then contact us and we will try to assist you.

The Speech Thing device driver is composed of two logical divisions.

One section for playback of pre-digitized Voice Master data files and the

other section which works in conjunction with the Smooth Talker programs

(SPEECHV2.EXE and SPEECHV3.EXE) for playback of synthesized speech. And, like

the device driver program itself, this documentation is also divided into

logical sections. The first section describes the operation and functions of

the driver as it pertains to the 'Voice Master Mode' and the second section

describes the same for 'Smooth Talker Mode'. Finally, the third section will

describe how too install the STDRIVER.SYS driver program. Since many of the

functions supported by the device driver are also supported by LOADPLAY and

STALK, it is suggested that you read the documentation for those programs

first.

--- Device driver IOCTL commands ---

The driver supports three distinct functions, WRITE, READ and IOCTL.

Input Output ConTroL or IOCTL is function 44H of DOS interrupt 21H and is used

to send commands to the Speech Thing device driver. Some languages like GW-

BASIC* and QuickBasic* specifically support the IOCTL function and commands

can easily be passed to the driver in the form of a three character ASCII

string. In cases where the IOCTL function is not specifically supported, such

as with Assembly language, Turbo PASCAL*, or QuickC*, then commands must be

sent to the driver using the DOS interrupt directly. Some of the IOCTL

commands are used to pass a numerical value to the driver program. This is

done by immediately following the three character command with an equal sign,

'=', and then the decimal number for the value to pass. For example, sending

the IOCTL string 'SPE=8' would tell the driver to set the playback speed to a

value of eight.

IMPORTANT: When sending a parameter with an IOCTL command, this exact format,

within the quotes, must be adhered to: 'CMD=XXXX'. Where CMD is the IOCTL

command, which can be either upper or lower case letters, followed immediately

by the equal sign and finally the decimal value. Note that no space characters

or commas are allowed.

///////////////////////////////////////////////////////////////////////////////

SECTION I

VOICE MASTER MODE

///////////////////////////////////////////////////////////////////////////////

There are basically two methods for sending a Voice Master data file

to the driver from within an application program. One method uses DOS's 'COPY'

command which can be executed from the application via a 'SHELL' command. The

advantage of using this method is that it is simpler because DOS is given the

responsibility for memory allocation/de-allocation and details involved with

writing the data out to the driver. However, the SHELL command is not

universally available and requires a large amount of overhead memory for

loading in DOS's command processor. The other available method is more direct

and uses the 'WRITE' command which is accessed through function 40H of DOS

interrupt 21H. Using this method is more complex because the application

program is responsible for memory allocation/de-allocation and other details.

However, it is more versatile and has the advantage of not requiring

additional overhead memory.

In Voice Master Mode, a data file is passed to the driver as a single

block rather than a byte (or character) at a time. This is because DOS cannot

transfer single bytes of data to the driver fast enough to satisfy playback

requirements which are typically above 12,000 bytes per second. Therefore, it

is necessary to first read the data file into a buffer so that the driver can

access the playback data as fast as it needs to. When DOS is told to 'COPY' a

data file to the driver, it first checks to ascertain if there is enough

memory available to allocate buffer space for the file. If so, the buffer is

allocated, the entire data file is read into the buffer, and control is passed

to the driver. The driver, in turn, reads and sends the data, one byte at a

time, out to the Speech Thing device. Finally, control is returned back to DOS

and the buffer is de-allocated. A similar situation occurs when a data file is

sent to the driver with a 'WRITE' command, with the main difference being that

the application program is responsible for buffering the data.

Once a data file has been read into a buffer and sent to the driver

for playback, the file can be played over again as many times as desired

by simply issuing a 'PLY' command to the driver. This will work as long as the

buffer is not subsequently corrupted by some activity which puts data in the

same memory area. If this happens, playback may become garbled, perhaps beyond

recognition. The 'PLY' command uses values stored in lower RAM to determine

the location of the buffer and the start and end addresses of the data to play

back. The values in these RAM locations are initialized whenever a data file is

sent to the driver with a 'WRITE' command (or indirectly by DOS's 'COPY'

command). By default, the driver will play back the entire data file from

start to end, which may consist of up to 64,000 bytes of playback data. The

128 byte 'VMF file header' is not sent out to the Speech Thing and is only

used by the driver to determine the file type and set certain parameters. The

driver always considers the first 128 bytes of the buffer to be the file

header and byte 129 is then the first byte of playback data. By default, the

following 'word' size addresses are established for use by the driver:

LOCATION                DESCRIPTION                     RANGE

0:1AAH      Segment value for start of data buffer.     0 - 65,520

0:1ACH      Starting address of playback data.          0 - 64,000

0:1AEH      Ending address of playback data.            0 - 64,000

(NOTE: See the 'USR' command for more information on these locations.)

These locations can be altered prior to issuing a 'PLY' command in order to

cause playback of any desired portion of the buffer. The start and end

addresses refer only to the playback data of the file and are referenced as an

offset+128 bytes to the value specified by the starting buffer segment

(location 1AAH). Thus a value of '0' for the start address will cause playback

starting with byte 129 of the buffer which corresponds to the first byte of

playable data. This method for setting the start and end address is completely

compatible with that used for the LOADPLAY program and data files should be

interchangeable between LOADPLAY and the   driver. By using the 'PLY' command

in conjunction with the above RAM locations, it is possible to play back any

amount of data up to the size of system memory. For instance, to playback a

576,128 byte data file: First allocate a buffer large enough to accommodate the

file, then copy the file to the buffer. Next send an 'RDH' IOCTL command to

the driver followed by sending the 128 byte header with the 'WRITE' command.

Then, use the 'WRITE' command again to playback the first 64,000 bytes data.

Finally, add 0FA0H (64,000/16) to the buffer segment value and 'WRITE' the

next 64,000 bytes, then repeat this step until all the data is played back.

Nothing to it, right?  Well, like anything else, a little practice ....

NOTE: When setting the start address for 'bit-wise' or 'nybble-wise' data

files, the address values must begin on the 'amplitude byte' boundary. In

other words, a 'bit-wise' start address must be evenly divisible by 16 and a

'nybble-wise' start address must be evenly divisible by 64. See the editor

documentation for an explanation of file types and structures.

In order to playback data files directly from the DOS prompt, it is

necessary to use the 'binary switch' with DOS's 'COPY' command. For example:

A>COPY SAMPLE.VMF/B ST

This would cause the driver to play back the above file. If you forget the

binary switch,'/B', then the driver will respond with a write error. The above

command could just as well have been executed via a 'SHELL' command as

follows:

SHELL"COPY SAMPLE.VMF/B ST"

Remember, the driver will only play back data files created with the Voice

Master and they must have a header (VMF type).

--- IOCTL Commands in Voice Master Mode ---

COMMAND                            FUNCTION

EVM     - Enable 'Voice Master Mode'. This tells the device driver that

pre-digitized data files will be passed to it. This is the mode

to use when it is desired to playback speech files created with

the Covox Voice Master. This mode automatically disables the

'Smooth Talker Mode'. When in this mode, the driver expects data

to be sent to it as a single block rather than a byte at a time.

RDH     - This command causes the 'VMF' file header to be read by the

driver at the next 'COPY' command from DOS, or 'WRITE' command

from an application program, however, actual playback of the data

is supressed. See example 2b (below) for further clarification.

PLY     - This causes playback of the data file in the buffer. This

function assumes that the data file is already buffered in

memory and that a previous 'RDH', 'WRITE' or DOS 'COPY' command

has been used to set initialization parameters. Playback range

can be varied by altering RAM addresses 0:1AAH through 0:1AFH

prior to issuing this command.

SPE     - Sets the playback speed. Range: SPE=0 to SPE=255

VOL     - Sets the playback volume. Range: VOL=1 to VOL=8

CKE     - Use the 8253 timer for control of playback speed. This is the

default mode.

CKD     - Disable use of the 8253 timer and use software timing for

control of playback speed.

DAC     - Enable an alternate port address for output. By default the

driver uses the port number at LPT1 (0:408 & 0:409) for sending

data to a DAC (Digital to Analog Converter). This command allows

an alternate port address to be specified as the DAC port.

Range: DAC=0 to DAC=65535.

WRA     - Set the number of times the data is to played back.

Until a key is pressed: WRA=0.  RANGE: WRA=1 to WRA=255 for

number of times to repeat playback.

USR     - Establish an alternate set of user address locations. By default,

three 'word' size locations at 0:1AAH through 0:1AFH are used for

storing the SEGMENT plus start and end OFFSET values for playing

back a data file. This command can be used to specify alternate

memory locations to store these values. Any locations from 0:100H

through 0:1FFH can be selected. However, memory locations must

be specified in decimal notation. For example: USR=256 will

establish user address locations 0:100H through 0:105H and

USR=426 would restore the default locations to 0:1AA through

0:1AF. As can be seen, a segment value of zero is always

assumed. This command is provided so that the device driver can

be compatible with other applications that might also use the

default locations.

--- Programming in 'Voice Master Mode'---

There are two ways that a pre-digitized Voice Master file can be sent

to the device driver from within a program. One way is to use a SHELL command

which does not require the use of a protected buffer area. Since DOS will

automatically buffer and write the file, the programmer does not have to be

concerned with these functions. We recommend using the SHELL command with the

BASIC programming languages. As an example, consider the following QuickBASIC

program:

Example 1

----------------------------------QuickBASIC----------------------------------

ON ERROR GOTO PRINT.MESSAGE     'Handle any I/O errors.

CMD$="EVM"                      'First enable Voice Master Mode.

CALL DEVICE.IOCTL

SHELL "COPY filename.vmf/b st"  'Now playback the file.

TERMINATE: END                  'End this program.

SUB DEVICE.IOCTL STATIC

SHARED CMD$                 '(allow access to an external variable)

OPEN "ST" FOR OUTPUT AS #1  'First open device for I/O-WRITE access.

IOCTL #1,CMD$               'Send the command to the driver.

CLOSE #1                    'Close the device for I/O-write access.

END SUB

PRINT.MESSAGE:                 'If an error, then print message and

PRINT "AN I/O ERROR HAS OCCURRED"

RESUME TERMINATE               'terminate program.

------------------------------------------------------------------------------

That is really all there is to it. Other parameters like speed, volume and the

number of times to playback the file (wrap) could also have been specified

simply by making additional calls to the 'DEVICE.IOCTL' routine.

As a further example consider the following program which is very

similar to the above, but instead of playing back the entire data file only

the portion from 5,000 to 10,000 is played back and is repeated three times.

Pay particular attention to the use of the 'RDH' command which allows buffering

of the data file, but suppresses playback until a 'PLY' command is sent. This

allows for the oppurtunity to set or change the start and end address range

before actualy playing back the data.

Example 1b

----------------------------------QuickBASIC----------------------------------

ON ERROR GOTO PRINT.MESSAGE    'Handle any I/O errors.

CMD$ = "EVM"                   'First enable Voice Master Mode.

CALL DEVICE.IOCTL

CMD$ = "RDH"                   'Send the "read file header" command.

CALL DEVICE.IOCTL

SHELL "COPY c:\covox\calc.vmf/b st"  'Now DOS will copy the data file into

'a buffer for later playback.

CALL SET.ADDRESSES             'Set playback range for data file.

FOR REPITITION = 1 TO 3        'Playback the data three times.

CMD$ = "PLY"                'Send the playback command.

CALL DEVICE.IOCTL

NEXT REPITITION

TERMINATE: END                 'End this program.

PRINT.MESSAGE:                 'If an error, then print message and

PRINT "AN I/O ERROR HAS OCCURRED"

RESUME TERMINATE               'terminate program.

SUB DEVICE.IOCTL STATIC

SHARED CMD$                 '(allow access to an external variable)

OPEN "ST" FOR OUTPUT AS #1  'First open device for I/O-WRITE access.

IOCTL #1, CMD$              'Send the command to the driver.

CLOSE #1                    'Close the device for I/O-write access.

END SUB

SUB SET.ADDRESSES STATIC

DEF SEG = 0               'Set segment to zero for following POKE's.

SAL = &H1AC: SAH = &H1AD  'Locations to POKE low/high bytes of start address.

EAL = &H1AE: EAH = &H1AF  'Locations to POKE low/high bytes of end address.

POKE SAL, 136: POKE SAH, 19  'Set start address for playback to 5,000.

POKE EAL, 32: POKE EAH, 78   'Set end address for playback to 20,000.

END SUB

------------------------------------------------------------------------------

The other method for sending pre-digitized data to the Speech Thing

requires the program itself to allocate a buffer, read the data file into it

and finally write the data out to the driver. The following examples will

demonstrate how this is done in other language environments:

Example 2

----------------------------- Assembly Language -----------------------------

CSEG            SEGMENT 'CODE'

ASSUME  CS:CSEG,SS:CSEG,DS:CSEG,ES:CSEG

ORG     100H

ASM_PLAY:       MOV     SP,OFFSET STK_SIZE  ;Set-up the program stack

MOV     AH,4AH       ;and modify DOS's memory allocation to

MOV     BX,SP        ;the size of the program plus

MOV     CL,4

SHR     BX,CL

ADD     BX,1

ADD     BX,20H       ;20H paragraphs for the PSP.

INT     21H

CALL    PLAY_FILE    ;Go playback a Voice Master data file.

MOV     AX,4C00H     ;Exit back to DOS.

INT     21H

;EQUATES AND VARIABLES

BUFF_SIZE       EQU     64000     ;Size of data buffer (in bytes).

READ            EQU     0

WRITE           EQU     1

CR              EQU     0DH       ;Carriage return character.

BUFF_SEG        DW      0         ;Segment location of data buffer.

FILE_SIZE       DW      0         ;Number of bytes in data file.

CMD$            DB      3,'EVM'   ;Command string to enable 'Voice Master mode.

FILENAME$       DB      'file.VMF',0  ;ASCIIZ specification for the data file.

DEVICE_NAME     DB      'ST',0 ;ASCIIZ specification for the driver.

HANDLE          DW      ?      ;File or driver handle will be saved here.

ERR_MSG         DB      'An error has occurred!',07H,CR,'$'

PLAY_FILE       PROC    NEAR   ;Procedure to playback a Voice Master file.

;First reserve buffer space for data file

CALL    ALLOCATE

JC      ERROR

;Read data file into allocated buffer.

MOV     AL,READ   ;Open the Voice Master data file for a read.

MOV     DX,OFFSET FILENAME$

CALL    OPEN

JC      ERROR

CALL    FILE_READ ;Read file into data buffer.

JC      ERROR

CALL    CLOSE     ;Close the file and release handle.

JC      ERROR

;Send driver command to enable 'Voice Master Mode', if necessary.

MOV     AL,WRITE  ;First open the driver for a write access.

MOV     DX,OFFSET DEVICE_NAME

CALL    OPEN

JC      ERROR

CALL    DEVICE_IOCTL  ;Now send the command to the driver.

JC      ERROR

;Send the data file to the driver for playback.

CALL    DEVICE_WRITE  ;Send the contents of the buffer to the

JC      ERROR         ;driver for playback.

CALL    CLOSE         ;Close driver access and release handle.

;Release buffer space since it is no longer needed.

CALL    DE_ALLOCATE

JNC     EXIT_PLAY_FILE

ERROR:          MOV     DX,OFFSET ERR_MSG

CALL    PRINT

CALL    CLOSE

EXIT_PLAY_FILE: RET                   ;Exit this procedure.

PLAY_FILE       ENDP

;MEMORY ALLOCATION & INPUT/OUTPUT PROCEDURES

ALLOCATE        PROC    NEAR        ;Reserve buffer space for data file.

MOV     AX,4800H

MOV     BX,BUFF_SIZE/16 ;Number of paragraphs to reserve.

INT     21H

MOV     BUFF_SEG,AX ;Save start segment of buffer.

RET

ALLOCATE        ENDP

DE_ALLOCATE     PROC    NEAR        ;Release reserved buffer space.

MOV     AX,4900H

MOV     ES,BUFF_SEG ;Start segment of area to release.

INT     21H

RET

DE_ALLOCATE     ENDP

OPEN            PROC    NEAR        ;Open driver or data file for access.

MOV     AH,3DH

INT     21H

MOV     HANDLE,AX   ;Save handle for the data file or device.

RET

OPEN            ENDP

CLOSE           PROC    NEAR        ;Close access to driver or data file.

MOV     AH,3EH

MOV     BX,HANDLE

INT     21H

RET

CLOSE           ENDP

FILE_READ       PROC    NEAR        ;Read data file into buffer.

PUSH    DS          ;Save data segment register.

MOV     AH,3FH

MOV     BX,HANDLE   ;Get the file handle.

MOV     CX,BUFF_SIZE ;Fill entire buffer if possible.

MOV     DX,BUFF_SEG ;Start segment of buffer.

MOV     DS,DX

MOV     DX,0        ;Offset start address of buffer.

INT     21H

POP     DS          ;Restore data segment register.

MOV     FILE_SIZE,AX ;Get and save actual number of bytes read.

RET

FILE_READ       ENDP

DEVICE_IOCTL    PROC    NEAR        ;Send an IOCTL command to the 'ST' device.

MOV     AX,4403H    ;Use IOCTL to write to control channel.

MOV     BX,HANDLE

MOV     CL,CMD$     ;Get length of command.

MOV     CH,0

MOV     DX,OFFSET CMD$+1 ;Get address of command string and

INT     21H         ;send it to the driver.

RET

DEVICE_IOCTL    ENDP

DEVICE_WRITE    PROC    NEAR

PUSH    DS          ;Save data segment register.

MOV     AX,4400H    ;Use IOCTL to get device information.

MOV     BX,HANDLE

INT     21H         ;Go get the device information.

MOV     AX,4401H    ;Use IOCTL to set device information.

MOV     BX,HANDLE

MOV     DH,0        ;Set high byte to zero.

OR      DL,20H      ;Toggle bit #5 to indicate 'raw' data mode.

INT     21H         ;Send the modified device information.

MOV     AH,40H      ;Send data file to driver for playback.

MOV     BX,HANDLE

MOV     CX,FILE_SIZE ;Number of data bytes to playback.

MOV     DX,BUFF_SEG ;Start segment of data buffer.

MOV     DS,DX

MOV     DX,0        ;Starting byte of buffer.

INT     21H         ;Write the data to the driver.

POP     DS          ;Restore data segment register.

RET

DEVICE_WRITE    ENDP

PRINT           PROC    NEAR        ;Print a string to the screen.

MOV     AH,09H

INT     21H

RET

PRINT           ENDP

STACK           DW      20H DUP(?)  ;Stack space for COM file.

STK_SIZE        EQU     $-2

CSEG            ENDS

END     ASM_PLAY

------------------------------------------------------------------------------

Example 3

---------------------------------Turbo PASCAL---------------------------------

program pascal_play;

type

data_str = string[16];

long_str = string[255];

regs = record

ax,bx,cx,dx,bp,si,di,ds,es,flags: integer;

end;

const

read = 0;

write = 1;

buff_size = 32767;    {Size, in bytes, of data buffer}

var

device_name, command_str, filename: data_str;

result: regs;

file_size, buff_seg, handle, carry: integer;

buffer: array [1..buff_size] of byte; {Declare an array as the data buffer}

procedure open(open_file: data_str; access: byte);

begin

with result do

begin

ax := $3D00 + access;

dx := ofs(open_file)+1;

ds := seg(open_file);

end;

msdos(result);               {Open file/device to get handle}

with result do

begin

carry := flags and 1;

handle := ax;

end;

end;

procedure close;

begin

with result do

begin

ax := $3E00;

bx := handle;

end;

msdos(result);               {Close device}

with result do

begin

carry := flags and 1;

end;

end;

procedure device_ioctl;

begin

with result do

begin

ax := $4403;

bx := handle;

cx := length(command_str);

dx := ofs(command_str)+1;

ds := seg(command_str);

end;

msdos(result);               {Send I/O command}

with result do

begin

carry := flags and 1;

end;

end;

procedure print_error;

begin

writeln('An error has occurred!');

end;

procedure file_read;

begin

with result do

begin

ax := $3F00;

bx := handle;

cx := buff_size;

dx := ofs(buffer);

ds := seg(buffer);

end;

msdos(result);               {Read file}

with result do

begin

file_size := ax;

carry := flags and 1;

end;

end;

procedure device_write;

begin

with result do

begin

ax := $4400;

bx := handle;

end;

msdos(result);               {Get device information}

with result do

begin

ax := $4401;

bx := handle;

dx := (dx or $20) and $00FF;  {Toggle bit #5 on for 'RAW' mode}

end;

msdos(result);               {Set device information}

with result do

begin

ax := $4000;

bx := handle;

cx := file_size;

dx := ofs(buffer);

ds := seg(buffer);

end;

msdos(result);               {Send data to driver}

with result do

begin

carry := flags and 1;

end;

end;

procedure play_file;

begin

open(filename,read);                    {Open file for a read access}

if carry=0 then

begin

file_read;                          {Read file into buffer array}

if carry=0 then

begin

close;

if carry=0 then

begin

open(device_name,write);    {Open device for a write access}

if carry=0 then

begin

device_ioctl;           {Send the command to the driver}

if carry=0 then

begin

device_write;       {Send data to driver for playback}

if carry=0 then

begin

close;

end;

end;

end;

end;

end;

end;

if carry=1 then    {If an I/O error occurs then print a message}

begin

print_error;

close;

end;

end;

begin

device_name := 'ST' + chr(0);

command_str := 'EVM';  {Command for device to use 'Voice Master Mode'}

filename := 'file.VMF' + chr(0);  {ASCIIZ filename for the data file}

play_file;             {Go playback data file}

end.

------------------------------------------------------------------------------

Example 4

------------------------------------QuickC -----------------------------------

/*

Using: MicroSoft Quick C ver 1.0 compiled with EXE option set.

*/

#include <stdio.h>

#include <dos.h>

#define DOS_INT   0x21

#define READ  0x00

#define WRITE  0x01

#define BUFF_SIZE 64000  /* Size, in bytes, of file buffer */

char far *disk_file_name = "file.VMF";

/* procedure area */

int device_open(int *handle,char far *file_name,int mode)

/* open device */

{

union REGS regs;

struct SREGS segregs;

regs.h.ah = 0x3D;

regs.h.al = (unsigned char) mode;

segregs.ds = FP_SEG(file_name);

regs.x.dx = FP_OFF(file_name);

intdosx(&regs,&regs,&segregs);

if(regs.x.cflag) return(-1);

*handle =  regs.x.ax;

return(0);

}

int device_close(int handle)

/* Close access to device */

{

union REGS regs;

regs.x.bx = handle;

regs.x.ax = 0x3E00;

intdos(&regs,&regs);

return(0);

}

int device_ioctl(int handle,char far *cmd_string)

/* Write an IOCTL command to the device */

{

union REGS regs;

struct SREGS segregs;

regs.x.ax = 0x4403;

regs.x.bx = handle;

regs.x.cx = strlen(cmd_string);

segregs.ds = FP_SEG(cmd_string);

regs.x.dx = FP_OFF(cmd_string);

int86x(DOS_INT,&regs,&regs,&segregs);

if(regs.x.cflag) return(-1);

return(0);

}

int device_write(int handle,char far *pointer,unsigned int length)

/* Write to a device */

{

union REGS regs;

struct SREGS segregs;

regs.x.ax = 0x4000;

regs.x.bx = handle;

regs.x.cx =  length;

regs.x.dx =  FP_OFF(pointer);

segregs.ds = FP_SEG(pointer);

int86x(DOS_INT,&regs,&regs,&segregs);

if(regs.x.cflag) return(-1);

return(0);

}

int device_read(int handle,void far *pointer,unsigned max_length,

unsigned int *length)

/* Read from a device */

{

union REGS regs;

struct SREGS segregs;

regs.x.ax = 0x3F00;

regs.x.bx = handle;

regs.x.cx =  max_length;

regs.x.dx =  FP_OFF(pointer);

segregs.ds = FP_SEG(pointer);

int86x(DOS_INT,&regs,&regs,&segregs);

if(regs.x.cflag) return(-1);

*length = regs.x.ax;

return(0);

}

int device_set_raw(int handle)

/* Set the device to raw I/O mode (i.e. not cooked) */

{

union REGS regs;

regs.x.ax = 0x4400;

regs.x.bx = handle;

intdos(&regs,&regs);

if(regs.x.cflag) return(-1);

regs.x.ax = 0x4401;

regs.x.bx = handle;

regs.h.dh = 0;

regs.h.dl |= 0x20;

intdos(&regs,&regs);

if(regs.x.cflag) return(-1);

return(0);

}

int allocate(unsigned int buf_size,unsigned int *segment)

/* Get allocated memory from dos */

{

union REGS regs;

regs.x.ax = 0x4800;

regs.x.bx = buf_size/16;

intdos(&regs,&regs);

if(regs.x.cflag) return(-1);

*segment = regs.x.ax;

return(0);

}

int de_allocate(int buf_seg)

/* Get allocated memory from dos */

{

union REGS regs;

struct SREGS segregs;

regs.x.ax = 0x4900;

segregs.es = buf_seg;

int86x(DOS_INT,&regs,&regs,&segregs);

if(regs.x.cflag) return(-1);

return(0);

}

main ()

{

unsigned int segment,length_read;

void far *data_pointer;

int input_handle,st_handle;

allocate(BUFF_SIZE,&segment);

data_pointer = (void far *)(segment * 0x10000);

if(device_open(&input_handle,disk_file_name,READ)){

printf("Error opening file: %s",disk_file_name);

exit(-1);

}

if(device_read(input_handle,data_pointer,BUFF_SIZE,&length_read)){

printf("Error reading disk file");

exit(-1);

}

device_close(input_handle);

if(device_open(&st_handle,(char far *)"ST",WRITE)){

printf("Error opening ST");

exit(-1);

}

if(device_ioctl(st_handle,(char far *)"EVM")){

printf("Error writing I/O command");

exit(-1);

}

if(device_set_raw(st_handle)){

printf("Error setting ST raw mode");

exit(-1);

}

if(device_write(st_handle,data_pointer,length_read)){

printf("Error writing to ST");

exit(-1);

}

device_close(st_handle);

de_allocate(segment);

return(0);

}

------------------------------------------------------------------------------

Example 5

--------------------------QuickC (large data files)---------------------------

/*

Demonstration program to play LARGE VMF files (larger then 64,000 bytes)

from C by manipulating far pointers.

Compilation requirements:

Requires Microsoft Quick C version 1.0.

Requires LARGE memory model libraries.

Compiled with qcl:

qcl large.c

qc defaults to the MEDIUM memory module, so in-memory compilation

is not possible.

*/

#include <stdio.h>

#include <dos.h>

#define DOS_INT   0x21

#define READ      0x00

#define WRITE     0x01

#define BUFF_SIZE 192128L  /* Size, in bytes, of file buffer (must be  */

/* greater than or equal to 64128 bytes)    */

#define TRUE      -1

#define FALSE     0

main (int argc,char *argv[])

{

int i;

int input_handle, st_handle, quit_flag=FALSE;

long loop, total_length = 0L;

unsigned int read_length;

unsigned int write_length;

unsigned int segment;

unsigned long segment_ptr, offset_ptr;

unsigned int far *seg_start = (unsigned int far *) 0x000001aa;

unsigned int far *play_start = (unsigned int far *) 0x000001ac;

unsigned int far *play_end = (unsigned int far *) 0x000001ae;

void far *data_pointer;

void far *temp_pointer;

int array_index;

unsigned short seg_array[10];

unsigned short len_array[10];

/*  Allocating the buffer to be used for loading the files and  */

/*   playback. BUFFSIZE is a defined constant (see above)  */

if (allocate((unsigned long) BUFF_SIZE, &segment))

{

printf("Allocate ERROR\n");

exit(TRUE);

}

/*  Converting the SEGMENT address to a far pointer  */

data_pointer = (void far *)(segment * 0x10000);

if (device_open(&input_handle, (char far *) argv[1], READ))

{

printf("Error opening file: %s", argv[1]);

exit(TRUE);

}

/*  Reading the 128 byte VMF header  */

if (device_read(input_handle, data_pointer, (unsigned int) 128,

&read_length ))

{

printf("Error reading disk file");

exit(TRUE);

}

if (read_length < 128)

{

printf("File to short to be a VMF file, aborting...\n");

exit(TRUE);

}

/*  We must keep track of each block of 64,000 bytes; therefore, */

/*  we load in 64,000 bytes, set up the pointers, then see if    */

/*  another block is needed, then repeat the process.            */

temp_pointer = data_pointer;

segment_ptr = ((unsigned long) temp_pointer & 0xffff0000L);

seg_array[0] = (unsigned short) (segment_ptr >> 16);

array_index = 1;

/*  Calculating segment after header.  Far pointers require  */

/*  special handling, as every 16 bytes of offset requires   */

/*  the segment to be incremented by 1.  In this instance,   */

/*  we are adding 128/16=8 to the segment section of the far */

/*  pointer.   */

segment_ptr += (unsigned long) ((128 / 16) * 0x10000L);

offset_ptr = ((unsigned long) temp_pointer & 0x0000ffffL);

temp_pointer = (void far *) (segment_ptr + offset_ptr);

/*  A loop to read the entire VMF file into memory, 64,000 */

/*  bytes at a time.  The loop continues until either the  */

/*  allocated space is used up, or the file has been read. */

/*  We start from 128 because of the previously read in    */

/*  header.  */

for (loop=128 ; loop < BUFF_SIZE ; loop += 64000L)

{

if (device_read(input_handle, temp_pointer, (unsigned int) 64000L,

&(read_length) ))

{

printf("Error reading disk file");

exit(TRUE);

}

total_length += (long) read_length;

len_array[array_index] = read_length;

/*  See the above comment on far pointers   */

segment_ptr = ((unsigned long) temp_pointer & 0xffff0000L);

seg_array[array_index++] = (unsigned short) (segment_ptr >> 16);

if (read_length != (unsigned int) 64000L)

break;              /*  The whole file has been read  */

else

{

/*  Calculating next segment  */

segment_ptr += (unsigned long) ((64000 / 16) * 0x10000L);

offset_ptr = ((unsigned long) temp_pointer & 0x0000ffffL);

temp_pointer = (void far *) (segment_ptr + offset_ptr);

}

}

device_close(input_handle);

if (len_array[array_index - 1] > 128)

len_array[array_index - 1] -= 128;

else

{

len_array[array_index - 2] -= (128 - len_array[array_index - 1]);

--array_index;

}

if (read_length == (unsigned int) 64000L)

{

printf("File is to large for memory, but we'll play the first ");

printf("%ld bytes\n", BUFF_SIZE);

}

if (device_open(&st_handle, (char far *) "ST", WRITE))

{

printf("Error opening ST");

exit(TRUE);

}

if (device_set_raw(st_handle))

{

printf("Error setting ST raw mode");

exit(TRUE);

}

/*  Enable Voice Master Mode  */

if(device_ioctl(st_handle, (char far *) "EVM"))

{

printf("Error writing I/O command 'EVM'");

exit(TRUE);

}

/*  Sending the 128 byte header to the VMF file  */

if(device_ioctl(st_handle, (char far *) "RDH"))

{

printf("Error writing I/O command 'RDH'");

exit(TRUE);

}

if (device_write(st_handle, data_pointer, 128))

{

printf("Error writing to ST");

exit(TRUE);

}

for (i=1 ; i<array_index ; ++i)

{

*seg_start = seg_array[i];

*play_start = 0;

*play_end = len_array[i];

if(device_ioctl(st_handle,(char far *)"PLY"))

{

printf("Error writing I/O command");

exit(TRUE);

}

}

device_close(st_handle);

de_allocate(segment);

exit(FALSE);

}

/* procedure area */

int device_open(int *handle, char far *stream_name, int mode)

/* open device */

{

union REGS regs;

struct SREGS segregs;

regs.h.ah = 0x3D;

regs.h.al = (unsigned char) mode;

segregs.ds = FP_SEG(stream_name);

regs.x.dx = FP_OFF(stream_name);

intdosx(&regs,&regs,&segregs);

if(regs.x.cflag)

return(-1);

*handle =  regs.x.ax;

return(0);

}

int device_close(int handle)

/* Close access to device */

{

union REGS regs;

regs.x.bx = handle;

regs.x.ax = 0x3E00;

intdos(&regs,&regs);

return(0);

}

int device_ioctl(int handle,char far *cmd_string)

/* Write an IOCTL command to the device */

{

union REGS regs;

struct SREGS segregs;

regs.x.ax = 0x4403;

regs.x.bx = handle;

regs.x.cx = strlen(cmd_string);

segregs.ds = FP_SEG(cmd_string);

regs.x.dx = FP_OFF(cmd_string);

int86x(DOS_INT,&regs,&regs,&segregs);

if(regs.x.cflag)

return(-1);

return(0);

}

int device_write(int handle,unsigned char far *pointer,unsigned int length)

/* Write to a device */

{

union REGS regs;

struct SREGS segregs;

regs.x.ax = 0x4000;

regs.x.bx = handle;

regs.x.cx =  length;

regs.x.dx =  FP_OFF(pointer);

segregs.ds = FP_SEG(pointer);

int86x(DOS_INT,&regs,&regs,&segregs);

if(regs.x.cflag)

return(-1);

return(0);

}

int device_read(int handle, unsigned char far *pointer,

unsigned int max_length, unsigned int *length)

/* Read from a device */

{

union REGS regs;

struct SREGS segregs;

regs.x.ax = 0x3F00;

regs.x.bx = handle;

regs.x.cx = max_length;

regs.x.dx =  FP_OFF(pointer);

segregs.ds = FP_SEG(pointer);

int86x(DOS_INT,&regs,&regs,&segregs);

if(regs.x.cflag)

return(-1);

*length = regs.x.ax;

return(0);

}

int device_set_raw(int handle)

/* Set the device to raw I/O mode (i.e. not cooked) */

{

union REGS regs;

regs.x.ax = 0x4400;

regs.x.bx = handle;

intdos(&regs,&regs);

if(regs.x.cflag)

return(-1);

regs.x.ax = 0x4401;

regs.x.bx = handle;

regs.h.dh = 0;

regs.h.dl |= 0x20;

intdos(&regs,&regs);

if(regs.x.cflag)

return(-1);

return(0);

}

int allocate(unsigned long buf_size, unsigned int *segment)

/* Get allocated memory from dos */

{

union REGS regs;

regs.x.ax = 0x4800;

regs.x.bx = buf_size/16;

intdos(&regs,&regs);

if(regs.x.cflag)

return(-1);

*segment = regs.x.ax;

return(0);

}

int de_allocate(int buf_seg)

/* Get allocated memory from dos */

{

union REGS regs;

struct SREGS segregs;

regs.x.ax = 0x4900;

segregs.es = buf_seg;

int86x(DOS_INT,&regs,&regs,&segregs);

if(regs.x.cflag) return(-1);

return(0);

}

------------------------------------------------------------------------------

///////////////////////////////////////////////////////////////////////////////

SECTION II

SMOOTH TALKER MODE

///////////////////////////////////////////////////////////////////////////////

This mode of operation causes the device driver to pass data from DOS

or the application program to the Speech Thing after the data has been

translated by Smooth Talker. Therefore it is mandatory that either

SPEECHV2.EXE or SPEECHV3.EXE be resident in memory when using this mode. When

in this mode, the driver reads and writes data one byte or character at a time

which means that an intermediate buffer between the driver and DOS, or an

application program, is not required. Data bytes are passed to and from the

driver as a stream of ASCII characters representing either English or phonetic

structures. Data is passed in several ways, from DOS by using its 'COPY' or

'TYPE' command or from an application program using a WRITE or READ function.

The ability to Send a string of characters directly from DOS to the driver is

one advantage that STDRIVER.SYS has over the STALK program. On the other hand,

STDRIVER.SYS does not support any of Smooth Talker's dictionary functions.

Smooth Talker can continuously playback an English or phonetic string

of up to 255 characters before it must pause to reset and read in the next

string. If this pause occurs in the middle of a sentence, then the speech will

sound discontinuous. For this reason, the driver program breaks up the string

of input characters into grammatical sentences with natural pauses and then

passes each sentence on to Smooth Talker for interpretation. A natural pause

is considered by the driver to be a period, colon, question or exclamation

mark. After Smooth Talker interprets the sentence, it sends the result to the

Speech Thing for playback and then resets itself to receive another 255

characters. If the driver is sent a string that cannot be divided into

sentences, then it will send the maximum of 255 characters to Smooth Talker

and then pause. There is one exception to the above which is the 'carriage

return' character. The driver can optionally be set to either ignore or

acknowledge carriage returns. If acknowledged, then a carriage return

character has exactly the same effect as a period or other sentence

terminator.

Causing smooth talker to speak directly from the DOS prompt can be

done using one of two DOS commands as follows:

A>COPY SAMPLE ST

or

A>TYPE SAMPLE > ST

Both methods have the same effect and will cause Smooth Talker to playback

the file. Also the 'SHELL' command could be used with equal effect:

SHELL "COPY SAMPLE ST"

or

SHELL "TYPE SAMPLE > ST"

The above file, "SAMPLE", could be any ASCII text file of any length

and can be created with a word processor, EDLIN, or directly from the DOS

prompt. Here is an example of creating a file from the DOS prompt:

A>COPY CON: SAMPLE

This is a sample text file.

^Z

Enter the above text exactly as shown and press ENTER after each line. The

CONTROL-Z character on the last line is the 'end of file' marker and instructs

DOS to create the file.

IOCTL Commands in Smooth Talker Mode

COMMAND                      FUNCTION

SV2     - Enable the 'Smooth Talker Mode' for use with the SPEECHV2.EXE

program. This tells the device driver that all data passed to

and from it will be sent one byte at a time as a stream of ASCII

characters. This mode automatically disables the 'Voice Master

Mode' (EVM command).

SV3     - Same as SV2 except that the SPEECHV3.EXE program is be used

instead.

ENG     - This instructs the driver that ASCII characters passed to it

for playback will be sent as English strings. After issuing this

command, all subsequent WRITEs by DOS or the application program

will cause smooth talker to interpret the string as English and

speak it. Once an 'ENG' command is sent, the driver will

continue to interpret strings as English until specifically told

to do otherwise by issuing the 'PHO' command. If a phonetic

string is sent to the driver while it is in English mode, then

it will attempt to speak the string as English. Also, the

default mode for the driver is English and the 'ENG' command

need not be sent if the driver was not previously in phonetic

mode.

PHO     - Same as for 'ENG' above, but instructs the driver that phonetic

strings will be passed instead.

XEP     - This instructs the driver that the next string sent to it is to

be converted from English to phonetics. Immediately after

issuing this command the application program should send an

English string via a WRITE command to the driver, then the

phonetic result can be obtained by sending the driver a READ

command. Once the the 'XEP' command has been issued, first a

WRITE, then a subsequent READ must be performed before the

driver will return to its normal I/O state. #Also, the driver

expects that the string written to it will be an English string,

even if the driver is currently in phonetics mode. There is no

need to first issue an 'ENG' command before sending the English

string for the 'XEP' function. After issuing a READ to get the

phonetic result, the driver will automatically be reset to its

previous English or phonetic mode.

TTE     - This enables the teletype mode which causes string data passed

to the driver to be echoed onto the screen.

TTD     - This disables the teletype mode.

CRE     - Causes the driver to acknowledge carriage returns. This feature

should be enabled in applications where it is desired to have

Smooth Talker speak the displayed string when an ENTER or

RETURN key is pressed.

CRD     - Causes the driver to ignore carriage returns. This is useful

when Smooth Talker is being used to speak back ASCII text files

where carriage returns occur after every screen line and cause

unwanted pauses.

FLU     - Flushes out the 255 byte string buffer that the device driver

uses for passing strings to the SPEECHV2 or SPEECHV3 program.

Sometimes it is necessary to clear out an English or phonetic

string that was previously sent to the driver but never spoken

by Smooth Talker. This situation can occur when carriage returns

have been disabled and a string without a sentence terminator

(a period, colon, etc.) is sent to the driver.

TON     - Sets the tone that smooth talker speaks with. The tone control

will only work the SPEECHV3 program and has no effect if changed

for SPEECHV2. Range: TON=0 for low tone or TON=1 for high tone.

SPE     - Sets the speed of playback. Range: SPE=0 to SPE=9.

PIT     - Sets the pitch of playback. Range: PIT=0 to PIT=9.

VOL     - Sets the volume of playback. Range: VOL=0 to VOL=9.

--- Programming in Smooth Talker Mode ---

Besides the IOCTL function, two other main I/O functions are also

supported by the device driver. They are READ and WRITE which correspond,

respectively, to functions 3FH and 40H of DOS interrupt 21H. In 'Smooth Talker

Mode', data is passed to and from the driver as a string of ASCII characters

whenever a WRITE or READ is performed. The maximum length of a string that is

written to the driver will depend on the limitations of the language being

used. For example, GW-BASIC only allows strings with a maximum length of 255

characters whereas QuickBasic strings can be up to 32,767 characters in

length. On the other hand, the READ function always returns a string of 255

characters or less. In this present implementation of the driver program, the

READ function is only used in association with the 'eXchange English for

Phonetics' or 'XEP' command. In order to convert an English string to a

phonetic string, the 'XEP' command is sent using the IOCTL function followed

by sending an English string with a WRITE command. Finally, the phonetic

result is obtained by using the READ function. Because the 'XEP' command

employs all three of the available driver I/O functions, it will be used below

as the primary example for passing string data to and from the driver.

Example A

----------------------------------GW-BASIC-----------------------------------

100 ON ERROR GOTO 430          'Handle any I/O errors.

110 CMD$="XEP"                 'Assign the device command to a string.

130 ENGLISH$="This is a sample string." 'Assign the string to write.

150 GOSUB 180                  'Go do the conversion.

160 END

170 '

180 'ENGLISH TO PHONETICS

190 GOSUB 240                  'Send the command.

200 GOSUB 300                  'Send the English string.

210 GOSUB 360                  'Get phonetic string.

220 RETURN

230 '

240 'DEVICE IOCTL

250 OPEN "ST" FOR OUTPUT AS #1 'First open device for IOCTL write access.

260 IOCTL #1,CMD$              'Send the command to the driver.

270 CLOSE #1                   'Close device for IOCTL write access.

280 RETURN

290 '

300 'DEVICE WRITE

310 OPEN "ST" FOR OUTPUT AS #1 'Open device for write access.

320 PRINT #1,ENGLISH$          'Send the English string to the driver.

330 CLOSE #1                   'Close device for write access

340 RETURN

350 '

360 'DEVICE READ

370 OPEN "ST" FOR INPUT AS #1  'Open device for read access.

380 INPUT #1,PHONETIC$         'Read the phonetic conversion from the

390 PRINT PHONETIC$            'driver and display it to the screen.

400 CLOSE #1

410 RETURN                     'Exit from routine and return to caller.

420 '

430 PRINT "AN I/O ERROR HAS OCCURED"

440 GOTO 160

-----------------------------------------------------------------------------

Example B

---------------------------------QuickBASIC----------------------------------

ON ERROR GOTO PRINT.MESSAGE          'Handle any I/O errors.

CMD$="XEP"                           'Assign the device command to a string.

ENGLISH$="This is a sample string."  'Assign the English string to translate.

CALL ENGLISH.TO.PHONETICS            'Go do conversion.

TERMINATE: END                       'End this program.

SUB ENGLISH.TO.PHONETICS STATIC

CALL DEVICE.IOCTL           'Send the command.

CALL DEVICE.WRITE           'Send the English string.

CALL DEVICE.READ            'Get and print the phonetic equivalent.

END SUB

SUB DEVICE.IOCTL STATIC

SHARED CMD$                 '(allow access to an external variable)

OPEN "ST" FOR OUTPUT AS #1  'First open device for I/O-WRITE access.

IOCTL #1,CMD$               'Send the command to the driver.

CLOSE #1                    'Close the device for I/O-write access.

END SUB

SUB DEVICE.WRITE STATIC

SHARED ENGLISH$             '(allow access to an external variable)

OPEN "ST" FOR OUTPUT AS #1  'Open the device for a WRITE access.

PRINT #1,ENGLISH$           'Send the English string to the driver.

CLOSE #1                    'Close the device for write access

END SUB

SUB DEVICE.READ STATIC

OPEN "ST" FOR INPUT AS #1   'and re-open it for a READ access.

INPUT #1,PHONETIC$          'Read the phonetic conversion from the

PRINT:PRINT PHONETIC$       'driver and display it to the screen.

CLOSE#1                     'Close read access and release file handle.

END SUB

PRINT.MESSAGE:                 'If an error, then print message and

PRINT "AN I/O ERROR HAS OCCURED"

RESUME TERMINATE               'terminate program.

-----------------------------------------------------------------------------

Example C

----------------------------- Assembly Language -----------------------------

CSEG            SEGMENT 'CODE'

ASSUME  CS:CSEG,SS:CSEG,DS:CSEG,ES:CSEG

ORG     100H

ASM_XEP_SAMPLE: CALL    ENGLISH_TO_PHON

RET     ;Return to DOS or calling routine.

CMD$            DB      3,'XEP'

ENGLISH$        DB      25,'This is a sample string.',CR

;EQUATES AND VARIABLES

READ            EQU     0

WRITE           EQU     1

CR              EQU     0DH    ;Carriage return character.

DEVICE_NAME     DB      'ST',0 ;ASCIIZ file specification for the driver.

HANDLE          DW      0      ;File handle will be saved here.

WRITE_STRING    DW      0      ;This holds the address of the string to send.

WRITE_COUNT     DW      0      ;Total number of characters in the above string.

READ_STRING     DB      100H DUP(0) ;Buffer for the returned string.

READ_COUNT      DB      0           ;Length of the returned string.

ERR_MSG         DB      'An error has occured!',07H,CR,'$'

;ENGLISH TO PHONETICS PROCEDURE

ENGLISH_TO_PHON PROC    NEAR

;Open the driver for a write access.

MOV     AL,WRITE

CALL    DEVICE_OPEN

JC      ERROR

;Send a command string to the driver.

CALL    DEVICE_IOCTL

JC      ERROR

;Send the English string to be translated.

MOV     AL,ENGLISH$      ;Get number of characters in string

MOV     AH,0

MOV     WRITE_COUNT,AX   ;and save it for write.

MOV     AX,OFFSET ENGLISH$+1 ;Get address of the English

MOV     WRITE_STRING,AX  ;string and save it also.

CALL    DEVICE_WRITE     ;Go translate English to phonetics.

JC      ERROR

;Terminate the write access to the driver.

CALL    DEVICE_CLOSE

JC      ERROR

;Open the driver for a read access.

MOV     AL,READ

CALL    DEVICE_OPEN

JC      ERROR

;Get the phonetic translation from the driver.

CALL    DEVICE_READ

JC      ERROR

MOV     DX,OFFSET READ_STRING  ;Get address of phonetic string.

MOV     BL,READ_COUNT   ;Get length of string (always < 255).

MOV     BH,0

MOV     READ_STRING[BX],'$' ;Add a DOS string terminator and

CALL    PRINT               ;and go display the string.

;Terminate the read access to the driver and release the handle.

CALL    DEVICE_CLOSE

JC      ERROR

JMP     SHORT E_TO_P_DONE

;If an error occurs then notify user.

ERROR:          MOV     DX,OFFSET ERR_MSG

CALL    PRINT

CALL    DEVICE_CLOSE

E_TO_P_DONE:    RET     ;Exit this procedure.

ENGLISH_TO_PHON ENDP

;INPUT/OUTPUT PROCEDURES

DEVICE_OPEN     PROC    NEAR       ;Open access to the 'ST' device.

MOV     AH,3DH

MOV     DX,OFFSET DEVICE_NAME

INT     21H

MOV     HANDLE,AX  ;Get and save handle for further access.

RET

DEVICE_OPEN     ENDP

DEVICE_CLOSE    PROC    NEAR       ;Close access to the 'ST' device.

MOV     AH,3EH

MOV     BX,HANDLE

INT     21H

RET

DEVICE_CLOSE    ENDP

DEVICE_IOCTL    PROC    NEAR       ;Write an IOCTL command to the 'ST' device.

MOV     AH,44H     ;DOS's 'DEVICE DRIVER CONTROL' function.

MOV     AL,3       ;Indicate a 'write command' sub-function.

MOV     BX,HANDLE

MOV     CL,CMD$    ;Get length of command to driver.

MOV     CH,0

MOV     DX,OFFSET CMD$+1 ;Get address of command

INT     21H        ;string and send it to driver.

RET

DEVICE_IOCTL    ENDP

DEVICE_WRITE    PROC    NEAR       ;Write a string to the 'ST' device.

MOV     AH,40H     ;DOS's 'WRITE to DEVICE' function.

MOV     BX,HANDLE

MOV     CX,WRITE_COUNT ;Tell driver number of characters in

MOV     DX,WRITE_STRING ;string and address of string.

INT     21H        ;Send the string to the driver.

RET

DEVICE_WRITE    ENDP

DEVICE_READ     PROC    NEAR       ;Read a string from the 'ST' device.

MOV     AH,3FH     ;DOS's 'READ from DEVICE' function.

MOV     BX,HANDLE

MOV     CX,0FFH    ;Maximum number of characters to read.

MOV     DX,OFFSET READ_STRING  ;Address of string buffer.

INT     21H        ;Tell driver to return a string.

MOV     READ_COUNT,AL ;Save length of string returned.

RET

DEVICE_READ     ENDP

PRINT           PROC    NEAR       ;Print a string to the screen.

MOV     AH,09H

INT     21H

RET

PRINT           ENDP

CSEG            ENDS

END     ASM_XEP_SAMPLE

-----------------------------------------------------------------------------

Example D

------------------------------- Turbo PASCAL -------------------------------

program pascal_XEP_sample;

type

data_str = string[16];

long_str = string[255];

regs = record

ax,bx,cx,dx,bp,si,di,ds,es,flags: integer;

end;

var

device, cmd_str: data_str;

english_str, phonetic_str: long_str;

result: regs;

handle, carry: integer;

work_var, read_len: byte;

phonetic_array: array[1..256] of char;

const

write = 1;

read = 2;

procedure device_open(access : byte);

begin

device := 'ST' + chr(0);

with result do

begin

ax := $3D00 + access;

dx := ofs(device)+1;

ds := seg(device);

end;

msdos(result);               {Open device to get handle}

with result do

begin

carry := flags and 1;

handle := ax;

end;

end;

procedure device_close;

begin

with result do

begin

ax := $3E00;

bx := handle;

end;

msdos(result);               {Close device}

with result do

begin

carry := flags and 1;

end;

end;

procedure device_ioctl;

begin

with result do

begin

ax := $4403;

bx := handle;

cx := length(cmd_str);

dx := ofs(cmd_str)+1;

ds := seg(cmd_str);

end;

msdos(result);               {Send I/O command}

with result do

begin

carry := flags and 1;

end;

end;

procedure device_write;

begin

with result do

begin

ax := $4000;

bx := handle;

cx := length(english_str);

dx := ofs(english_str)+1;

ds := seg(english_str);

end;

msdos(result);               {Write string to device}

with result do

begin

carry := flags and 1;

end;

end;

procedure device_read;

begin

with result do

begin

ax := $3F00;

bx := handle;

cx := $00FF;

dx := ofs(phonetic_array);

ds := seg(phonetic_array);

end;

msdos(result);               {Read string from device}

with result do

begin

carry := flags and 1;

read_len := ax;

end;

work_var := read_len;

phonetic_str := '';

while read_len > 0 do        {Read the result into an array}

begin                      {PASCAL programmers: Is their another way?}

read_len := read_len - 1;

phonetic_str := phonetic_str + phonetic_array[work_var - read_len];

end;

end;

procedure print_error;

begin

writeln('An error has occurred!');

end;

procedure english_to_phon;

begin

device_open(write);

if carry=0 then

begin

device_ioctl;

if carry=0 then

begin

device_write;

if carry=0 then

begin

device_close;

device_open(read);

if carry=0 then

begin

device_read;

if carry=0 then

begin

writeln(phonetic_str);

device_close;

end;

end;

end;

end;

end;

if carry=1 then

print_error;

device_close;

end;

begin

cmd_str := 'XEP';      {IOCTL command to send to driver}

english_str := 'This is a sample string.' + chr(13);  {String to convert}

english_to_phon;      {Go convert string to phonetics}

end.

-----------------------------------------------------------------------------

Example E

------------------------------------QuickC------------------------------------

/*

Sample program for using voice master device driver with

the C programming language.

Using: MicroSoft Quick C ver 1.0 compiled with EXE option set.

*/

#include <stdio.h>

#include <dos.h>

#define READ  0x00

#define WRITE  0x01

char far *english = "This is a sample string.\r";

char phonetic[256];

char far *cmd[] = {"SV2","XEP",""};

/* procedure area */

int device_open(int *handle,int mode)

/* open ST device */

{

union REGS regs;

struct SREGS segregs;

static char far *file_name = "ST";

regs.h.ah = 0x3D;

regs.h.al = (unsigned char) mode;

segregs.ds = FP_SEG(file_name);

regs.x.dx = FP_OFF(file_name);

intdosx(&regs,&regs,&segregs);

if(regs.x.cflag) return(-1);

*handle =  regs.x.ax;

return(0);

}

int device_close(int handle)

/* Close access to the 'ST' device */

{

union REGS regs;

regs.x.bx = handle;

regs.x.ax = 0x3E00;

intdos(&regs,&regs);

return(0);

}

int device_ioctl(char far *cmd_string)

/* Write an IOCTL command to the ST device */

{

union REGS regs;

struct SREGS segregs;

int handle;

if(device_open(&handle,WRITE)) return(-1);

regs.x.ax = 0x4403;

regs.x.bx = handle;

regs.x.cx = strlen(cmd_string);

segregs.ds = FP_SEG(cmd_string);

regs.x.dx = FP_OFF(cmd_string);

intdosx(&regs,&regs,&segregs);

device_close(handle);

if(regs.x.cflag) return(-1);

return(0);

}

int device_write(char far *string)

/* Write string to the 'ST' device */

{

union REGS regs;

struct SREGS segregs;

int handle;

if(device_open(&handle,WRITE)) return(-1);

regs.x.ax = 0x4000;

regs.x.bx = handle;

regs.x.cx = strlen(string);

regs.x.dx =  FP_OFF(string);

segregs.ds = FP_SEG(string);

intdosx(&regs,&regs,&segregs);

if(regs.x.cflag) return(-1);

device_close(handle);

return(0);

}

int device_read(char far *string,int max_length)

/* Read a string from the 'ST' device */

{

union REGS regs;

struct SREGS segregs;

int handle;

if(device_open(&handle,READ)) return(-1);

regs.x.ax = 0x3F00;

regs.x.bx = handle;

regs.x.cx = max_length;

regs.x.dx =  FP_OFF(string);

segregs.ds = FP_SEG(string);

intdosx(&regs,&regs,&segregs);

device_close(handle);

if(regs.x.cflag) return(-1);

return(0);

}

main ()

{

int i;

i = 0;

/* keep sending command until a zero length string */

while(strlen(cmd[i])){

if(device_ioctl(cmd[i++])){

printf("An I/O command error has occurred");

exit(-1);

}

}

if(device_write(english)){

printf("A device write error");

exit(-1);

}

if(device_read((char far *)phonetic,sizeof(phonetic))){

printf("A device read error");

exit(-1);

}

printf("\n%s\n",phonetic);

return(0);

}

------------------------------------------------------------------------------

The above programming examples contain all the procedures necessary

for sending any command or string of data to the driver program. In order to

send a different command to the driver, simply change the string assigned to

the variable 'CMD$'. Likewise, 'ENGLISH$' can also be changed as desired. In

the program example that follows, procedures and variables that have already

been defined will be assumed. Here is a QuickBASIC example that sets the

playback speed to '8' and causes Smooth Talker to speak the greeting, 'Hello,

how are doing?'. Simply make the following changes to the above QuickBASIC

program (EXAMPLE B):

Example F

------------------------------------------------------------------------------

CMD$="SPE=8"                         'The IOCTL command to set playback speed.

ENGLISH$="Hello, how are you doing?" 'Assign the English string to speak.

CALL SPEAK.ENGLISH                   'Go speak the greeting.

SUB SPEAK.ENGLISH

CALL DEVICE.IOCTL                 'Send the speed command.

CALL DEVICE.WRITE                 'Send the English string.

END SUB

------------------------------------------------------------------------------

Because the programming examples given for the other languages also use the

string variables 'CMD$' and 'ENGLISH$' and contain I/O procedures with the

same labels, the above logic can be applied to them as well. If this is not

yet apparent, then go back and study the examples very carefully.

NOTE: All the program examples given in this documentation have been checked

for accuracy and should be executable. We hope that they will provide the

programmer with adequate information for interfacing with our device driver.

However, some of the examples could probably be written more effectively. So,

if you have written a program (or re-written one of the above examples) that

you believe better demonstrates the use of the STDRIVER.SYS device driver,

then send us a copy and if we like it we will include it in this documentation

along with full credit to you for your efforts. We would also be interested in

seeing examples from other languages and application programs, like ADA, LISP,

DBASE or whatever.

///////////////////////////////////////////////////////////////////////////////

SECTION III

INSTALLING STDRIVER.SYS

///////////////////////////////////////////////////////////////////////////////

The device driver STDRIVER.SYS will work with any PC or MS DOS version

3.0 or higher. It can be installed by including it in a 'CONFIG.SYS' file

which must reside in the root directory of the drive that is used to boot the

system. If a CONFIG.SYS file already exists, then just add the STDRIVER.SYS

driver program to it by including the following line:

device=STDRIVER.SYS

This will cause the STDRIVER.SYS device driver to be installed the next time

the system is booted up. If a CONFIG.SYS file does not already exist, then you

will have to make one as follows:

A>COPY CON: CONFIG.SYS   <press RETURN>

device=STDRIVER.SYS      <press RETURN>

^Z                       <CONTROL-Z>

Please refer to a standard DOS manual for more details on CONFIG.SYS files.

Optional parameters may also be specified by including them with the

device name according to the following format:

device=STDRIVER.SYS /P/P/P/etc.

^ a mandatory space must be here!

Where the 'P' parameter represents any legal STDRIVER.SYS device command. For

example:

device=STDRIVER.SYS /EVM/SPE=200/VOL=7

Including the above line in the CONFIG.SYS file will cause the device driver

to install itself in 'Voice Master Mode' (EVM), with playback speed set to 200

and playback volume set to 7. These settings will remain in effect until a

command is sent to the driver which changes one or more of them. As another

example:

device=STDRIVER.SYS /SV3/PHO/TTD

The above commands would cause STDRIVER.SYS to use the 'SPEECHV3.EXE' Smooth

Talker program (SV3). Data sent to the driver would be interpreted as phonetic

strings (PHO) and the string would not be echoed to the screen when spoken

(TTD).

Finally, if no parameters are entered, the device driver will install itself in

the default mode which would be equivalent to the following:

device=STDRIVER.SYS /SV2/ENG/CRE/TTE/TON=0/SPE=5/PIT=5

Keep in mind that the parsing routine will not accept an illegal parameter and

if one is found then parsing will stop where the error occurs and default

parameters will be used for the remaining parameters. Of course, the driver

will notify you of any illegal parameters when it installs itself at boot-up

time, but it will not specify exactly where the error occurred. You must

determine this yourself. For example, the following would cause an error:

device=STDRIVER.SYS /SV3/SPE=150

The error occurs because setting the playback speed to 150 is not allowed when

the driver is in 'Smooth Talker Mode'.

NOTE: The Speech Thing device driver will work correctly only with DOS

versions 3.0 or higher. Although the driver program can be installed with DOS

2.X versions, only Voice Master functions work properly and any attempt to

access Smooth Talker functions will cause a 'lock-up'.

Also, don't forget that STDRIVER.SYS requires either SPEECHV2.EXE (for PC's &

XT's) or SPEECHV3.EXE (for AT's only) to be loaded in memory before any

strings can be sent to the driver. Probably the best way to do this is to

include one of the two programs in an 'AUTOEXEC.BAT' file and then it will

automatically be loaded into memory upon boot-up.

* 'Microsoft','QuickC',GW-BASIC and 'QuickBASIC' are trademarks of Microsoft

Corp.

* 'Turbo Pascal' is a trademark of Borland International.

* 'Smooth Talker' is a trademark of First Byte Inc.

* 'Speech Thing' and 'Voice Master' are trademarks of Covox Inc.