A d v a n c e d

����  �����    ����  ��    �� �  �����

�  ��  �    �  ��  �� ��    �� � ������

�    � �����  ��    �� ��  ��  �       �

����  �   �  �      �  ����   �  �����

Advanced Gravis Computer Technology Ltd.

Fax (604) 435-9358                   BBS  (604) 431-5927 V32bis

Compuserve: GO PCVENB (71333,350)    InterNET EMail: tech@gravis.com

FidoNET: 1:153/978 or 1:153/878      SBCNET: 13:900/3

AOL: Adv Gravis                      Genie: Page M805 Topic 6

Gravis UltraSound MIDIFIER (MIDFIR76.ZIP)

Copyright (C) 1993 by Advanced Gravis Computer Technology Ltd.

All Rights Reserved

Midifier is a program written as a lab project to prove a concept. It

is not inteneded to be a Gravis product but rather a utility. We have

used it in house for a few months and decided to release it to you

(the public) as an XMas '93 gift. Enjoy.

----------------------------Midifier documentation------------------------------

Midifier is a program which will turn your Advanced Gravis UltraSound card

into a fun-to-use general midi sound module.  Midifier will let you

play music using an external MIDI keyboard hooked up to a UltraSound

MIDI adapter.

Once midifier is started, the display should look like the following:

ɵ Configuration: Untitled ���������������������������������������������������ͻ

�  Program  # Edit  # Channel  #                                               �

�                                                                              �

� Now displaying channel 1                                                     �

�                                      �                                       �

� Volume: #�������������۱���# 100     � Pitch Bend: #���������۱�������# 08192�

�                                      �                                       �

� Expression: #������������������# 127 � PB Sensitivity: #��۱���������#02     �

�                                      �                                       �

� Pan: #���������۱�������# 064        � [ ] Sustain                           �

�                                      �                                       �

� Mod Wheel: #۱����������������#  000 � Program:  None          #             �

������������������������������������������������������������������������������ĺ

� Patches Loaded:   Memory Available: 1024K        �������������������Ŀ       �

� ���������������Ŀ ������������������������       �   All Notes Off   �       �

� �               �                                ���������������������       �

� �               � [ ] Use 8-bit patches                                      �

� �               � �������������������Ŀ                                      �

� �               � � Unload Percussion �                                      �

� �               � ���������������������                                      �

� �               � �������������������Ŀ                                      �

� �               � �   Unload Patch    �                                      �

� ����������������� ���������������������                                      �

������������������������������������������������������������������������������ͼ

The top half of the screen is showing the current parameters for MIDI channel

1.  When you use the MIDI keyboard to adjust parameters such as volume,

expression, pan, mod wheel, or pitch bend, the screen will get updated to

show you the new values.  You can also adjust the MIDI parameters by using

the mouse or keyboard.

Midifier won't actually make any sounds until you load instruments.  You

can load instruments using one of several methods:

1) Click on the drop-down-menu next to the label "Program:" and a menu

will appear with a list of instruments.  The screen should look like

the following:

ɵ Configuration: Untitled ���������������������������������������������������ͻ

�  Program  # Edit  # Channel  #                                               �

�                                                                              �

� Now displaying channel 1                                                     �

�                                      �                                       �

� Volume: #�������������۱���# 100     � Pitch Bend: #���������۱�������# 08192�

�                                      �                                       �

� Expression: #������������������# 127 � PB Sensitivity: #��۱���������#02     �

�                                      �                                       �

� Pan: #���������۱�������# 064        � [ ] Sustain                           �

�                                      �                                       �

� Mod Wheel: #۱����������������#  000 � Program: �None         �#             �

�����������������������������������������������������������������Ŀ�����������ĺ

� Patches Loaded:   Memory Available: 1024K       ��acpiano  000�#����Ŀ       �

� ���������������Ŀ ������������������������      � britepno 001 ۳f   �       �

� �               �                               � synpiano 002 �������       �

� �               � [ ] Use 8-bit patches         � honky    003 ��            �

� �               � �������������������Ŀ         � epiano1  004 ��            �

� �               � � Unload Percussion �         � epiano2  005 ��            �

� �               � ���������������������         � hrpschrd 006 ��            �

� �               � �������������������Ŀ         � clavinet 007 #�            �

� �               � �   Unload Patch    �         �����������������            �

� ����������������� ���������������������                                      �

������������������������������������������������������������������������������ͼ

You can either type the name of the instrument you want, or

use the arrow keys or page up and page down keys to find

an instrument.  Or, you can hold down the first mouse button

and slide the mouse upwards or downwards beyond the top or

bottom of the listbox to scroll.  When the instrument you want

to load is highlighted, press return, or click with the mouse.

The screen should looke something like the following:

ɵ Configuration: Untitled ���������������������������������������������������ͻ

�  Program  # Edit  # Channel  #                                               �

�                                                                              �

� Now displaying channel 1                                                     �

�                                      �                                       �

� Volume: #�������������۱���# 100     � Pitch Bend: #���������۱�������# 08192�

�                                      �                                       �

� Expression: #������������������# 127 � PB Sensitivity: #��۱���������#02     �

�                                      �                                       �

� Pan: #���������۱�������# 064        � [ ] Sustain                           �

�                                      �                                       �

� Mod Wheel: #۱����������������#  000 � Program: �honky    003 �#             �

������������������������������������������������������������������������������ĺ

� Patches Loaded:   Memory Available: 0896K        �������������������Ŀ       �

� ���������������Ŀ ��۱��������������������       �   All Notes Off   �       �

� � honky    003  �                                ���������������������       �

� �               � [ ] Use 8-bit patches                                      �

� �               � �������������������Ŀ                                      �

� �               � � Unload Percussion �                                      �

� �               � ���������������������                                      �

� �               � �������������������Ŀ                                      �

� �               � �   Unload Patch    �                                      �

� ����������������� ���������������������                                      �

������������������������������������������������������������������������������ͼ

The Honkytonk piano is loaded.

2) If you have a General MIDI keyboard, you can send choose the

instrument from your keyboard.

3) The Edit Patch Selection menu can be used to load group

patch selections.  This will be shown later.

The bottom half of the screen lets you manage patches that are loaded.

You can select one instrument out of the list of loaded instruments by

double clicking on the insturment name in the listbox.  A selected

instrument can be unloaded by pressing the "Unload Patch" button.

If you get any "Hung notes," you can turn all playing notes off by

pressing the "All Notes Off" button.

If you have a MIDI keyboard with aftertouch, you can add a vibrato

effect to a playing note by pressing the keys a little harder than normal

after starting the note.

ɵ Configuration: Untitled ���������������������������������������������������ͻ

� �Program �# Edit  # Channel  #                                               �

� ������������������Ŀ                                                         �

� ��New Config     � �el 1                                                     �

� � Open Config...   �                 �                                       �

� � Save Config      ���۱���# 100     � Pitch Bend: #���������۱�������# 08192�

� � Save Config as...�                 �                                       �

� � ---------------  ������������# 127 � PB Sensitivity: #��۱���������#02     �

� � About Midifier   �                 �                                       �

� � ---------------  �����# 064        � [ ] Sustain                           �

� � Exit             �                 �                                       �

� �������������������ٱ���������#  000 � Program:  None          #             �

������������������������������������������������������������������������������ĺ

� Patches Loaded:   Memory Available: 1024K        �������������������Ŀ       �

� ���������������Ŀ ������������������������       �   All Notes Off   �       �

� �               �                                ���������������������       �

� �               � [ ] Use 8-bit patches                                      �

� �               � �������������������Ŀ                                      �

� �               � � Unload Percussion �                                      �

� �               � ���������������������                                      �

� �               � �������������������Ŀ                                      �

� �               � �   Unload Patch    �                                      �

� ����������������� ���������������������                                      �

������������������������������������������������������������������������������ͼ

The File menu can be used to save or load configurations.  Anything that

you may have modified with the "Edit Patch Selections" menu can be

saved and restored with the configuration files.  You can also use the

File menu to exit the program.  Esc works too.

ɵ Configuration: Untitled ���������������������������������������������������ͻ

�  Program  #�Edit �# Channel  #                                               �

�            ����������������������Ŀ                                          �

� Now display��Program Selection..� �                                          �

�            � Mod Wheel Action...  �  �                                       �

� Volume: #��� Patch...             �  � Pitch Bend: #���������۱�������# 08192�

�            ������������������������  �                                       �

� Expression: #������������������# 127 � PB Sensitivity: #��۱���������#02     �

�                                      �                                       �

� Pan: #���������۱�������# 064        � [ ] Sustain                           �

�                                      �                                       �

� Mod Wheel: #۱����������������#  000 � Program:  None          #             �

������������������������������������������������������������������������������ĺ

� Patches Loaded:   Memory Available: 1024K        �������������������Ŀ       �

� ���������������Ŀ ������������������������       �   All Notes Off   �       �

� �               �                                ���������������������       �

� �               � [ ] Use 8-bit patches                                      �

� �               � �������������������Ŀ                                      �

� �               � � Unload Percussion �                                      �

� �               � ���������������������                                      �

� �               � �������������������Ŀ                                      �

� �               � �   Unload Patch    �                                      �

� ����������������� ���������������������                                      �

������������������������������������������������������������������������������ͼ

The Edit Program selection menu lets you do the following:

* Split the keyboard among multiple instruments

* Play multiple instruments simultaneous

* Play different instruments depending on how hard you hit the keys

* adjust instrument tuning

* add quick-and-dirty chorusing effect

ڴ Configuration: Untitled ���������������������������������������������������Ŀ

�����������������������������������������������������������������������������ͻ�

�� [ ] Respond to MIDI program change                                         ��

�� ����������������������������������������������������������������������     ��

�� Split patch selection by: (*) Note ( ) Velocity                            ��

��                                                                            ��

��       Note     To       Use Patch         Offset     Tune    Use           ��

��     � 0      � 127     acpiano  000  #   � 0       � 1018    [X]           ��

��     ������   ������                      ������    ��������                ��

��     � 0      � 127     britepno 001  #   � 0       � 1020    [X]           ��

��     ������   ������                      ������    ��������                ��

��     � 0      � 127     synpiano 002  #   � 0       � 1024    [X]           ��

��     ������   ������                      ������    ��������                ��

��     � 0      � 127     honky    003  #   � 0       � 1030    [X]           ��

��     ������   ������                      ������    ��������                ��

��     � 0      � 127     None          #   � 0       � 1024    [ ]           ��

��     ������   ������                      ������    ��������                ��

�� �������������������������������������������������������������������������� ��

��                                                                ��������Ŀ  ��

��                                                                ��  Ok  ��  ��

��                                                                ����������  ��

�����������������������������������������������������������������������������ͼ�

� ����������������� ���������������������                                      �

��������������������������������������������������������������������������������

The Respond to MIDI program change button toggles between using the normal

instrument selection, or the cool-neato instrument selection shown above.

The keyboard can be split either by note (pitch) or velocity (volume).

Each row can be used to select an instrument.  When a midi note is hit

that falls in the range of the first column and second column, the note

will be heard.  The offset can be used to adjust the pitch of a note

by semitones.  There are 12 semitiones in an octave.  The tune column

can be used to detune a patch.  If 1024 is used, no detune will occur.

This number will be multiplied by the frequency, and then divided by

1024.  So, 1085 will be one semitone, 1149 will be two, 1217 will be

three, etc.  A simple chorusing effect can be creating by choosing numbers

close to 1024, such as 1020, 1024, 1028, 1032, etc...

A row can be turned off by clicking on the Use column until the X

disappears.

Other input methods:  If you are trying to set a range where a Piano

should be heard (between c2 and b3), click in row one in the

"Use Patch" column, and select acpiano.  Now, click in the first column

to set the lower range.  Press the C2 note on your MIDI keyboard, and

a 36 should appear in the first column.  Now press the tab key to advance

to the next column, or click on the "To" column with the mouse.  Press

the B3 note on your MIDI keyboard, and a 59 will appear in the "To"

column.  Now, Midifier is set up to listen to only the notes between

C2 and B3.  However, the offset column is still zero, and a MIDI note

of zero is below you hearing range.  You should set up the offset so

that it matches the low range.  Click your mouse in the offset field

and press the C2 key, or type in 36.  To test out the setup, click on

a field that won't be affected by a "note-on" MIDI event.  Tab to

the "Use" column, it is a safe place to be while playing around.

Now try playing your MIDI keyboard, and you'll notice that the piano

only plays in the range from C2 to B3.  Using the same method, you

can assign other rows to different instruments.  If the numbers in

the first and second columns overlap between rows, you can play multiple

instruments simultaneously.

When you find an instrument selection that pleases you, click on the

OK button.

ڴ Configuration: Untitled ���������������������������������������������������Ŀ

�  Program  #�Edit �# Channel  #                                               �

�                                                                              �

� Now displaying��������������������������������������������ͻ                 �

�               �                                            �                 �

� Volume: #������ Use mod wheel on your keyboard to change:  ��۱�������# 08192�

�               �                                            �                 �

� Expression: #��   ( ) Vibrato                              ����������#02     �

�               �                                            �                 �

� Pan: #���������   ( ) Expression                           �                 �

�               �                                            �                 �

� Mod Wheel: #۱�   (*) Pan                                  �   #             �

���������������ĺ                                            �����������������ĳ

� Patches Loaded� ������������������������������������������ ���������Ŀ       �

� �������������ĺ                                ��������Ŀ  �es Off   �       �

� � acpiano  000�                                �   Ok   �  �����������       �

� � britepno 001�                                ����������  �                 �

� � synpiano 002��������������������������������������������ͼ                 �

� � honky    003  � � Unload Percussion �                                      �

� �               � ���������������������                                      �

� �               � �������������������Ŀ                                      �

� �               � �   Unload Patch    �                                      �

� ����������������� ���������������������                                      �

��������������������������������������������������������������������������������

The "Edit Mod Wheel Action..." menu can be used to change the effect of

the mod wheel on your MIDI keyboard.  Normally the mod wheel will activate

the frequency modulation for an instrument (vibrato).  Expression can

be used to change the current volume for an instrument.  Pan can be used to

move the instrument between the left and right speakers.

ڴ Configuration: Untitled ���������������������������������������������������Ŀ

�  Program��������������������������������������������������������ͻ           �

�         �                                                        �           �

� Now disp� UltraSound memory allocated for                        �           �

�         � custom patch: 256K                                     �           �

� Volume: �                                                        �����# 08192�

�         � ��������Ŀ                                             �           �

� Expressi� ��Record��       [X] Loop                              ����#02     �

�         � ����������                                             �           �

� Pan: #���                                                        �           �

�         �                                                        �           �

� Mod Whee�                                                        �           �

���������ĺ Start loop: #۱��������������������������#             �����������ĳ

� Patches �                                                        ���Ŀ       �

� �������ĺ End loop:   #����������������������������#             �   �       �

� �       �                                                        �����       �

� �       � ������������������������������������������������������ �           �

� �       �                                            ��������Ŀ  �           �

� �       �                                            �   Ok   �  �           �

� �       �                                            ����������  �           �

� �       ��������������������������������������������������������ͼ           �

� �               � �   Unload Patch    �                                      �

� ����������������� ���������������������                                      �

��������������������������������������������������������������������������������

Another toy-feature of Midifier is the custom patch feature.  If you add

a microphone to the second input jack from the bottom of your UltraSound

card, you can create your own toy patch.  Just click on the record button

while making noise into the microphone.  When you are done, press

enter, space, or click the mouse button on the stop button.  Now start

playing on your MIDI keyboard.  If you want to turn the looping off, press

the loop button.  If you want to edit the loop points, move the start loop

or end loop sliders.  You will not be able to create real fancy patches

with this feature.  If you would like to create real patches, look for one

of the many patch editors available from Advanced Gravis, or public networks

such as Internet, Compuserve, Genie, America Online, etc.

More features will be added to midifier at various times in the next 30 or

40 years, so keep looking.

--Mike Leibow

Forte Technologies