Note: Words and double-words are stored Intel-style.

Offset: (decimal)    Contents:

-------              ---------

0-7         ASCII text:  'GF1PATCH'

8-10        ASCII text:  '100'  (pre 2.01 disks)

'110'  (2.01 disks)

11          byte '00'

12-20         ASCII text   'ID#000002'

21          byte '00'

22          ASCII text:  copyright text

82          byte  ;I call this Voices, always set to 1

83          word  ;I call this Channels, always set to 14 decimal (0E hex)

85          word  ;number of of samples, or waves

87          word  ;master volume, usually set to 127 decimal (7F hex)

89          dword ;unknown, not used with pre 2.01 patches, value is close

;to the total length of the patch file.  May be used for

;memory management purposes.

131         ASCII text:  name of patch.

;first letter is missing in pre 2.01 patches

;apparently not used in 2.01 patches

147         dword ;unknown, related to value at 89

;set equal to 1 in pre 2.01 patches

151         byte  ;unknown, but is always 1

194         dword ;another value related to values at 89 and 147

;not used in pre 2.01 patches

198         word  ;number of samples, or waves

239         Sample block, or wave block

;there is a sample block for every sample in a patch

;the following offsets are into the sample block

0-5         ASCII text:  sample name

;this is usually 'NoName'

6          byte '00'

7          byte  ;I call this the mode byte, use is unknown

8          dword ;length of sample in bytes

12         dword ;loop start in bytes offset from beginning of sample

16         dword ;loop end in bytes offset from beginning of sample

20         word  ;sampling frequency

22         dword ;high frequency multiplied by 1000

26         dword ;low frequency multiplied by 1000

30         dword ;base frequency multiplied by 1000

34         byte  ;detune, usually equal to zero

35         byte  ;octave, usually equal to 2

36         byte  ;balance, usually 7, meaning center of stereo field

37-48        12 bytes  ;envelope data, format unknown

;I have observed the following:

;first byte: higher = slower attack

;second byte: higher = longer attack

;fourth byte: level of release

;seventh byte: lower=slower attack

;eighth byte: lower = longer delay until attack

;ninth byte: level of release

;tenth byte: length of release

49         byte  ;tremolo sweep

50         byte  ;tremolo rate

51         byte  ;termolo depth

52         byte  ;vibrato sweep

53         byte  ;vibrato rate

54         byte  ;vibrato depth

55         byte  ;flags  7 6 5 4 3 2 1 0

;I have observed the following:

;7 6:  envelope scaling (I think)

;5: 1=melodic, 0=percussion

;4: 1=read sample backwards, 0=read sample forwards

;2: 1=sample is looped, 0=sample plays through once

;0: 1=sample is 16-bit, 0=sample is 8-bit

56         byte  ;unknown, not used in pre 2.01 patches, usually '3C' hex

59         byte  ;unknown, not used in pre 2.01 patches, usually '04'

96         Sample

;sample data is stored unsigned

;16-bit samples are stored Intel style with the

;least significant byte stored first.

;If there is more than one sample in the patch, the next

;sample block starts right after the end of the

;previous sample.