rev 1. 2, Dec 1996

SAM9407

PROGRAMMER’S REFERENCE

Overview

This document details all the programming information for SAM9407 Windows applications or drivers development.

It is shared in 4 chapters

- WINDOWS API

- FIRMWARE API

- FIRMWARE Structures

- MIDI Implementation

The SAM9407 and Its associated firmware provides the following devices:

## MIDI DEVICE:

- Wave table synthesis:
34 to 64 voices of polyphony
16/8 bits samples , any sampling rate, linear interpolation, forward/reverse loop 
12/24 dB resonant filter
reverb & chorus send
ROM and/or RAM download sound bank (up to 8 simultaneous sound bank)

- MPU401 compliant

- Complete MIDI implementation described in « SAM9407 MIDI IMPLEMENTATION » chapter

## STREAMING AUDIO BUFFER DEVICE:

- 8 play + 1 record simultaneous tracks
format: mono/stereo, 8/16 bits, any sampling rate

- Play tracks: 
12 dB filter, real time sampling rate controller  
Mode: 2 outputs with reverb & chorus send / 4 outputs

## STATIC AUDIO BUFFER DEVICE:

- 34 to 62 tracks
format: mono (stereo requires 2 tracks), 8/16 bits, any sampling rate
Mode: 2 outputs with reverb & chorus send / 4 outputs

- looped wave: 
16/8 bits samples , any sampling rate, linear interpolation, forward/reverse loop 
24 dB resonant filter, real time sampling rate controller
256 K sample maximum size

- 1 shot wave: 
16/8 bits samples , any sampling rate, linear interpolation, forward loop 
12 dB resonant filter, real time sampling rate controller
no sample size limitation

## AUDIO IN DEVICE:

The audio In device provides stereo input sampled at 39.25 KHz.
The audio in signal can be send to the reverb or to the embedded audio in echo.

This Audio In can be used to connect a microphone (karaoke application) or an external audio source like a CD audio (94PC32 add on card) or a CODEC (multi-media combo board).
In the last example this input gives the ability to add effects (reverb, equalizer & surround) to the CODEC inputs (Line In, Microphone, Audio CD)..

## REVERB & CHORUS DEVICE

Provides 8 reverb + 8 chorus stereo programs compatible with GS standard.

## EQUALIZER DEVICE:

Stereo four band parametric equalizer

Default setting 450Hz , 900Hz, 4kHz, 9kHz with +/-12dB band level.

## SURROUND DEVICE

Surround processing enable to expand stereo image of a stereo signal or to create a pseudo stereo image from a monophonic source.

Surround signal can be routed either to the main left/right output or to the Auxiliary left/right output.

## PITCH SHIFTER DEVICE

This device may not be implemented in some versions of firmware.

The pitch shifter allows to change the pitch of selected sources (Audio in, Midi, Streaming Audio Buffer, Static Audio Buffer) without affecting the tempo.

# Signal Processing Synoptic

Chapter 1

WINDOWS API

DREAM94.DRV driver provides 16bits WINDOWS 3.1 API

DREAM95.DRV driver provides 16bits & 32bits WINDOWS 95 API
Actually 32bits API are 16bits API converted VIA a thunk.

DREAM95.VXD driver provides 32bits WINDOWS 95 API

# Windows Multimedia Low-Level Audio Services

Dream drivers provides entry points functions to support the Windows audio services:

- wodMessages

- widMessages

- modMessages

- midMessages

- auxMessages (not currently supported)

Those functions support enable application to use the Windows multimedia services:

Auxiliary Audio Device

auxiliary audio device services are not supported in the current version

Midi Input Device

midiInGetNumDevs		supported (1 device)

midiInGetDevCaps		supported

midiInOpen			supported

midiInClose			supported

midiInPrepareHeader		supported

midiInUnPrepareHeader		supported

midiInAddBuffer			supported

midiInReset			supported

midiInStart				supported

midiInStop				supported

midiInGetErrorText		supported

## Midi Output Device

midiOutGetNumDevs		supported (2 devices)

midiOutGetDevCaps		supported

midiOutOpen			supported

midiOutClose			supported

midiOutPrepareHeader		supported

midiOutUnPrepareHeader		supported

midiOutShortMsg			supported

midiOutLongMsg			supported

midiOutGetVolume		supported

midiOutSetVolume		supported

midiOutGetErrorText		supported

Wave Input Device

waveInGetNumDevs		supported (1 device)

waveInGetDevCaps		supported

waveInOpen			supported

waveInClose			supported

waveInPrepareHeader		supported

waveInUnPrepareHeader		supported

waveInGetErrorText		supported

waveInAddBuffer			supported

waveInReset			supported

waveInStart			supported

waveInStop			supported

Wave Output Device

waveOutGetNumDevs		supported (8 devices)

waveOutGetDevCaps		supported

waveOutOpen			supported

waveOutClose			supported

waveOutPrepareHeader		supported

waveOutUnPrepareHeader		supported

waveOutGetErrorText		supported

waveOutWrite			supported

waveOutReset			supported

waveOutStart			supported

waveOutRestart			supported

waveOutGetVolume		supported

waveOutSetVolume		supported

waveOutGetPitch			see below

waveOutSetPitch			see below

waveOutGetPlaybackRate		not supported

waveOutSetPlaybackRate		not supported

The Pitch API are supported but instead of doing Pitch transpose as required by Windows Mmedia specification, It actually change the sampling frequency.

- Pitch transpose 		modify playback spectrum 	keep playback duration

- Playback Rate variation	keep playback spectrum 	modify playback duration

- Sampling Freq variation	modify playback spectrum 	modify playback duration

The Pitch & PlaybackRate API required specific processing that actually is supported by SAM9407 with the Pitch shifter Device (see FIRMWARE API chapter).
These API can be correctly implemented if necessary.

# SAM9407 WINDOWS API LIST

All the API can be used by 16bits & 32bits applications.

mpuMessage			Send MPU command 
mpuMessageAc		Send MPU command with acknowledge

genControlRead		Read parameter value from Control Table.

surroundSetVolume		Set 3D effect level
equalizerSetBand		Set equalizer band parameters

memAlloc *			Allocate a memory block on the Dream card
memFree *			Free a memory block on the Dream card
memRead 			Read a memory block from the Dream card  
memWrite			Write a memory block into the Dream card
memGetMapAddress 	Get Address of the Dream card Memory Mapping Table
memSetMapAddress 	Set Address of the Dream card Memory Mapping Table

memLoadFirmware *	Download a firmware into the Dream card memory

waveSetBufferSize		Set the size of the wave buffers for further wave open
waveSetMainVolume	Set Main volumes
waveSetAuxVolume		Set AUX volume
waveSetFilter			Set filter parameters
waveSetPitch			Set sample rate

modOpen			Open the static audio device
modGetVoiceNumber	Get the number of available voices
modOpenVoice		Open a static wave voice
modChangeParam		Change a parameter (volume, pitch, filter) for a static wave voice
modDefineVoiceMemory	Set the sample parameters (start, loop, end) for a static wave voice
modGetPos			Get the current reading position.
modSetNewPos		Set a new reading position
modCommand		Send command (Start, Pause, Stop) to play and stop a voice
modClose			Close the static audio device

bankLoad *			Load Sound bank from .94b file to SAM9407 memory
bankUnload	**		Unload Sound bank from .SAM9407 memory
bankGetCaps **		Change the priority of a sound bank
bankSetPriority **		Fill a structure with SAM9407 memory bank information

*	Available for WIN95 API only
 **	Not Available in current 1.4 version

# mpuMessage

Syntax		WORD	mpuMessage(wCommand, lpParam, wCount)
		Send MPU command via the driver

Parameters	WORD	wCommand
			Dream expanded MPU401 command number
		LPWORD lpParam
			Pointer to a byte table holding the command parameter
		WORD wCount
			Parameter byte count

Return Value	Returns zero if the function was successful. Otherwise, returns DREAMERR_UNREF.

Comments	This API enable windows programmer to send MPU401 command which are not supported by Microsoft or Dream API.
Parameter table is byte type, word parameter are stored LSB first.
See FIRMWARE API chapter for details on MPU401 command.

## mpuMessageAc

Syntax		WORD	mpuMessageAc(wCommand, lpParam, wCount,Ack)

Send MPU command with acknowledge via the driver

Parameters	WORD	wCommand

Dream expanded MPU401 command number

LPWORD lpParam

Pointer to a byte table holding the command parameter

WORD wCount

Parameter byte count

BYTE Ack

Acknowledge that should be waited by the driver

Return Value	Returns 0 if the function was successful. Otherwise, returns DREAMERR_UNREF.

Comments	This API enable windows programmer to send MPU401 command which are not supported by Microsoft or Dream API.

Parameter table is byte type, word parameter are stored LSB first.

See FIRMWARE API chapter for details on MPU401 command.

## genControlRead

Syntax		WORD	genControlRead(lpValue,bCommand)

Read parameter value from Control Table

Parameters	LPBYTE	lpValue

Pointer on a location filled by the returned control value

one BYTE if bCommand<7f

128 BYTE if bCommand=0xff

BYTE	bCommand

0-7f: control nb for which the value is returned

0xff, Value is assumed to be a 128 BYTE table	and is filled with 128 control values.

Value[n] is the value for the control nb n.

Return Value	Returns zero if the function was successful. Otherwise, returns DREAMERR_UNREF.

Comments	Control table is described in FIRMWARE Structure Chapter

## surroundSetVolume

Syntax		WORD	surroundSetVolume(wVolume)

Set surround effect intensity.

Parameters	WORD	wVolume

Surround volume: 0 no surround, 0x7f maximum surround effect

Return Value	Returns zero if the function was successful. Otherwise, returns DREAMERR_UNREF.

Comments	 Surround effect provides additional parameters: delay, input format, output format. Those parameters could be set using mpuMessage function.

## equalizerSetBand

Syntax		WORD	equalizerSetBand(wBand,wChannel,wVolume)

Set equalizer Band Volume

Parameters	WORD	wBand

Band selected: EQ_BASS, EQ_MID1, EQ_MID2, EQ_TREBLE

WORD	wChannel

Channels select: EQ_STEREO, EQ_LEFT, EQ_RIGHT

WORD	wVolume

New Volume dB scale, 0 -12dB ,0x40 0dB, 0x7f +12dB

Return Value	Returns zero if the function was successful. Otherwise, returns DREAMERR_UNREF.

Comments	Equalizer is a serial effect. It process mixed audio signal, this includes Midi, Wave, Mod and Audio In.

Bass band is lowpass type, Medium bands are band pass type, High band is high pass type

Band filter frequency are 450Hz, 900Hz, 4khz, 9KHz.

Equalizer is parametric type, band frequency could  be adjust using mpuMessage function.

memAlloc

Syntax		WORD	memAlloc(dwSize, wType)

Allocate a memory block on the Dream card

Parameters	DWORD dwSize

size of the block to allocate (see comment)
WORD wType

type of the block to allocate (see comment)

Return Value	return zero if the function was successful. Otherwise, it returns an error number, possible returns are:

DREAMERR_INVADD		block not in 64K page

DREAMERR_OUTOFMEM	block out of SAM9407 memory

DREAMERR_UNREF		any other error

Comments	The Type of the memory block is actually written in the MMT (see « Firmware Structure » chapter).
dwSize should be less than 256K for xx11H type
dwSize has not size limit for xx30H type
dwSize should be less than 64K for any other type
Available for Windows 95 applications only

## memFree

Syntax		WORD	memFree(dwAddress)

Free a memory block on the Dream card

Parameters	DWORD dwAddress

Address of the block to free

Return Value	return zero if the function was successful. Otherwise, it returns an error number, possible returns are:

DREAMERR_OUTOFMEM	block out of SAM9407 memory

DREAMERR_UNREF		any other error

Comments	Available for Windows 95 applications only

## memLoadFirmware

Syntax		WORD	memFree(lpszFirmName)

Download a firmware into the Dream card

Parameters	LPCSTR lpszFirmName
Points to a null-terminated string that specifies the name (with extension) of the firmware file

Return Value	return zero if the function was successful. Otherwise, it returns an error number, possible returns are:

DREAMERR_ABORT		Dream card memory unit busy, read impossible, retry later.

DREAMERR_OUTOFMEM	block out of SAM9407 memory

DREAMERR_UNREF		any other error

Comments	Available for Windows 95 applications only

## memRead

Syntax		WORD	memRead(lpDst,dwSrcAddress,wCount)

Read a memory bloc from Dream card

Parameters	LPVOID lpDst

Pointer to memory bloc to receive data read from Dream card.

DWORD dwSrcAddress

32bit Dream card memory Address

WORD wCount

Count of word to read

Return Value	return zero if the function was successful. Otherwise, it returns an error number, possible returns are:

DREAMERR_ABORT		Dream card memory unit busy, read impossible, retry later.

DREAMERR_INVCOUNT	wCount > 32K word

DREAMERR_INVADD		block not in 64K page

DREAMERR_OUTOFMEM	block out of SAM9407 memory

DREAMERR_UNREF		any other error

Comments	The memory block to read should fit in 64K word page and its size can't exceed 32K word.

## memWrite

Syntax		WORD	memWrite(lpSrc,dwDestAddress,wCount)

Write a memory bloc into Dream card

Parameters	LPVOID lpSrc

Pointer to memory bloc to write to Dream card.

DWORD dwDestAddress

32bit Dream card memory Address

WORD wCount

Number of word to write

Return Value	return zero if the function was successful. Otherwise, it returns an error number, possible returns are:

DREAMERR_ABORT		Dream card memory unit busy, write impossible, retry later.

DREAMERR_INVCOUNT	wCount > 32K word

DREAMERR_INVADD		block not in 64K page

DREAMERR_OUTOFMEM	block out of SAM9407 mem

DREAMERR_UNREF		any other error

Comments	The memory block to write should fit in 64K word page and its size can't exceed 32K word.

## memGetMapAddress

Syntax		DWORD memGetMapAddress()

Get Address of the Dream card Memory Mapping Table

Parameters	none

Return Value	return the 32bit Address of Memory Mapping Table. Otherwise, it returns DREAMERR_UNREF.

Comments	The Memory Mapping Table is described in chapter OVERVIEW - SAM9407 Memory Management

## memSetMapAddress

Syntax		WORD memSetMapAddress(dwAddress)

Set Address of the Dream card Memory Mapping Table

Parameters	DWORD dwAddress

32bit Address of Memory Mapping Table in Dream card memory.

Return Value	return the 32bit Address of Memory Mapping Table. Otherwise, it returns DREAMERR_UNREF.

Comments	The Memory Mapping Table is described in chapter OVERVIEW - SAM9407 Memory Management

## waveSetBufferSize

Syntax		WORD	waveSetBufferSize(wSize,bForPlay)

Set the size of the wave buffers for further opens

if bForPlay=TRUE, the play buffers size is set, else the record buffer size is set.

Parameters	WORD	wSize

Size in word. Maximum size enabled by memWrite and memRead is 32K.

BOOL bForPlay

TRUE if modification is for play buffers, FALSE if it’s for record buffer.

Return Value	return zero if the function was successful. Otherwise, it returns DREAMERR_UNREF.

Comments	The waveSetBufferSize doesn't affect current wave but only future open waves.

## waveSetMainVolume

Syntax		WORD	waveSetMainVolume(Id, wVolLeft, wVolRight)

Set wave main output volume

Parameters	WORD	wVoice

Voice Number

WORD wVolLeft

Left Volume

WORD wVolRight

Right Volume

Return Value	return zero if the function was successful. Otherwise, it returns DREAMERR_UNREF.

Comments	This command is mainly used to control volume in multi-wave mode.

## waveSetAuxVolume

Syntax		WORD	waveSetAuxVolume(Id, wVolLeft, wVolRight)

Set wave auxiliary output volume

Parameters	WORD	wVoice

Voice Number

WORD wVolLeft

Left Volume

WORD wVolRight

Right Volume

Return Value	return zero if the function was successful. Otherwise, it returns DREAMERR_UNREF.

Comments	Aux output can be affected to rear speakers or reverb/chorus effect send. see MPU401 EXPANDED PROTOCOL for more details.

## waveSetFilter

Syntax		WORD	waveSetFilter(Id, wControl, wValue)

Set filter parameters

Parameters	WORD	wControl

Control type: WAVE_FC , WAVE_Q

WORD wValue

Value 0x0-0x7f: filter open (0x7f), filter close (0x0), no resonance (0x7f), maximum resonance (0x0)

Return Value	return zero if the function was successful. Otherwise, it returns DREAMERR_UNREF.

Comments	The filter used for wave are 12dB low pass filters. The WAVE_Q parameter is active for mono wave only.

## waveSetPitch

Syntax		WORD	waveSetPitch(Id, wAbsolutePitch)

Set the sample rate

Parameters	WORD wAbsolutePitch

Absolute value of the new sample rate

Return Value	return zero if the function was successful. Otherwise, it returns DREAMERR_UNREF.

## modOpen

Syntax		WORD	modOpen()

Open the static audio device

Parameters	none

Return Value	return zero if the function was successful. Otherwise, it returns MMSYSERR_ALLOCATED.

Comments	This API must be called before any other static wave API (mod...).

## modGetVoiceNumber

Syntax		WORD	modGetVoiceNumber()

Get the number of available voices.

Parameters	none

Return Value	return the number of available voices.

Comments	This API should be called before openning a static wave voice (modOpenVoice) to check if  the voice can be played.

## modOpenVoice

Syntax		WORD	modOpenVoice (wVoice, bVol, wVolMain, wVolAux, wPitch, wFilt)

Open a static wave voice and set volumes, pitch, filter parameters.

Parameters	WORD	wVoice

Voice number. Should be smaller than the number of available voice returned by 						modGetVoiceNumber

If  the bit 7 is set, the voice is assumed to be « cross bank ». This allow to play samples which arenot necessary inside 256kW bank. However,  such a voice actually reserves two internal voices. (See §1, Static Audio Device).

BYTE bVol

Value 0x0-0xff: General volume for the static wave

WORD wVolMain

LSB-->Right main volume (0-ff)

MSB-->Left main volume (0-ff)

WORD wVolAux

LSB-->Right aux volume (0-ff)

MSB-->Left aux volume (0-ff)

WORD wPitch

400h=nominal frequency (depending on quartz frequency) and linearly scalled (200h = 1/2 nom. freq.)

WORD wFilt

MSB-> Fc 	0(close)-ff(open)

LSB->Q      0(resonnance max.)-ff(no resonnance )

Return Value	return zero if the function was successful.

## modCommand

Syntax		DWORD modCommand(wVoice, bCommand)

Send a command to static wave (start, stop, pause)

Parameters

WORD wVoice : voice number

BYTE  bCommand	=	MOD_START

or	MOD_STOP

or	MOD_CLOSE

Return Value	0 if succed, error messages if failed.

## modChangeParam

Syntax		WORD	modChangeParam(wVoice, wType, wValue)

Change a parameter (volume, pitch, filter) for a static wave voice.

Parameters	WORD wVoice : voice number

According to wType the wValue is :

if wType=VOI_VOL		General volume for the static wave (0-ff )

if wType=VOI_MAIN		LSB-->Right main volume (0-ff)

MSB-->Left main volume (0-ff)

if wType=VOI_AUX		LSB-->Right aux volume (0-ff)

MSB-->Left aux volume (0-ff)

if wType=PITCH		Pitch : 400h=nominal frequency and linearly

scalled (200h = 1/2 nom. freq.)

if wType=VOI_FILT		Filt	MSB-> Fc 	0(close)-ff(open)

LSB->Q      0(resonnance max.)-ff(no resonnance )

Return Value	return zero if the function was successful.

Comments	This API could be called when the voice is playing.

## modDefineVoiceMemory

Syntax		WORD	modDefineVoiceMemory(wVoice, wFormat, dwStart, dwLoop, dwEnd)

Define start,loop, and end sample address and define the sample format

Parameters

WORD wVoice : voice number

WORD wFormat 	
bit 7 	: 0->16 bits ; 1->8 bits

bit 6	: used if bit 7=1.

0-> 8 bits samples are in LSB of word memory

1->Sample in MSB of word memory

bit 0-5 : loop type

0 : forward loop

1 : reverse loop

2 : reverse loop with sign inversion

Only loop type 0 is available if a crossbank sample has been defined in modOpenVoice. ( See §1, Static Audio Device)

DWORD dwStart, dwLoop, dwEnd, 	: absolute address (32 bits)

The sample is first read from dwStart to dwEnd, then it’s looping between dwLoop an dwEnd.

If the voice hasn’t been openned for « cross bank » sample, the sample must be inside a 256 kWords bank.

To define a one shot sample, the dwLoop and dwEnd should be set on 8 zero samples added after the sample .

Return Value	return zero.

## modGetPos

Syntax		DWORD	modGetPos(wVoice)

Return the current reading position for the voice

Parameters

WORD wVoice : voice number

Return Value	return absolute address of the current reading position.

Comments	This API could be called to check if  the static wave is looping.

## modSetNewPos

Syntax		DWORD	modSetNewPos(wVoice, dwOffset)

Add an offset to current reading position

Parameters

WORD wVoice : voice number

DWORD dwOffset : 32 bits offset to add.

Return Value	0 if succed, error messages if failed.

## modClose

Syntax		void	modClose()

Close the static audio device.

Parameters	none

Comments	This function must be called after having used static audio device.

# bankLoad

Syntax	WORD	bankLoad(lpszFileName, lpszBankName)
		Load Sound bank from .94b file to SAM9407 memory

Parameters	LPCSTR lpszFileName
Points to a null-terminated string that specifies the name (with extension) of the .94b sound bank file.

LPCSTR lpszBankName
Points to a null-terminated string that specifies the name of the sound bank.
A null string assumes that the BankName is the same as the FileName.

Return Value		return bank number if the function was successful. Otherwise, it returns 				0FFFFH.

Comments	Available for Win95 applications only.

# (The following API are not available in current 1.40 version)

# bankUnload

Syntax	WORD	bankUnload(wBankNumber)
		Unload Sound bank from .SAM9407 memory

Parameters	WORD wBankNumber
0-7 number of the bank to be deleted
0FFFFH Unload all banks

Return Value	returns zero if the function was successful. Otherwise, returns DREAMERR_UNREF.

Comments

bankGetCaps

Syntax	WORD	bankGetCaps(lpStructure)

Fill a structure with SAM9407 memory bank information

Parameters	LPVOID lpStructure
Points to the structure to be filled

Return Value	returns zero if the function was successful. Otherwise, returns DREAMERR_UNREF.

Comments	The structure has the following format:

WORD 	total available memory for sound bank

WORD	free memory for sound bank

8 times the following record, one per sound bank:

8 * CHAR	sound bank name

WORD	priority level of the sound bank

WORD	Size of the Sound bank

# bankSetPriority

Syntax	WORD	bankSetPriority(wBankNumber, wBankPriority)
		Change the priority of a sound bank

Parameters	WORD wBankNumber
0-7 number of the bank to be deleted
WORD wBankPriority
Bank priority level (0 to 0FFFEH)
0FFFFH priority assumes that the bank is disable but still in memory

Return Value	returns zero if the function was successful. Otherwise, returns DREAMERR_UNREF.

Comments	The bank set priority affects the midi mapping table (see FIRMWARE Structure chapter)

Chapter 2

FIRMWARE API

# I/O Interface

The I/O Interface is composed of two byte registers,  one word register and one IRQ:

| I/O address | Write from PC (OUT) | Read to PC (IN) |
| --- | --- | --- |
| MPU_base + 0 | DATA8 | DATA8 |
| MPU_base +1 | CONTROL | STATUS |
| MPU_base +2/3 | DATA16 | DATA16 |

The byte registers provides backwards compatibility with the standard MPU401 UART mode.
The control message is sent on CONTROL register with one or several data on DATA8 register (word data are send as two bytes). The read back values (if any) are available on DATA8 register.

The word register provides high rate data transfer in burst mode.
It is used to download / upload SAM9407 memory Data.
It should always be used with 16 bits I/Os instruction (OUT DX,AX  IN  AX,DX  OUTSW INSW). Using 8 bit I/Os at these addresses will give unpredictable results.

The IRQ (PC compatible rising edge) is backward compatible with MPU401 interrupt.
It is floated until the MPU401 interface is switched to UART mode, to minimize potential IRQ conflicts.

## IO Status Register:

| TE | RF | ID1 | ID0 | X | X | X | X |
| --- | --- | --- | --- | --- | --- | --- | --- |

TE : Transmit empty. 
If 0, data from SAM9407 to PC is pending and IRQ is high
Reading the data at MPU_BASE+0 will set TE to 1 and clear IRQ.

RF : Receiver full. 
If 0 then SAM9407 is ready to accept CONTROL or DATA from the PC.

TE and RF are MPU401 compliant. Two additional bits ID1 ID0 are provided. They allow to identify the logical SAM9407 device read DATA8 as follows :

| ID1 | ID0 | Device |
| --- | --- | --- |
| 0 | 0 | MIDI |
| 0 | 1 | Streaming audio |
| 1 | 0 | MOD player |
| 1 | 1 | General |

## Stand alone & UART modes

Stand alone mode:

After power-up, hardware reset or MPU reset control, the board is in stand-alone mode:

Stand alone mode enables only 2 controls:

- 3FH to switch to UART mode
 3FH control is acknowledged by receiving 0FEH as DATA8 with ID(1,0)=00 (Midi device).

- BEH to sent any control 
BEH enable to send only one control, this means that each control sent in stand alone mode should start with BEH control.

UART mode:

UART mode accepts any control.
Control 0FFH (MPU reset) switch back to stand alone mode.

# CONTROL MESSAGES OVERVIEW

A device control message consists of one CONTROL byte followed by one or several DATA8 bytes (parameters). A parameter can be byte or multiple bytes. The least significant byte is always sent first (little endian). « word » means two bytes, « double word » or « Dword » means four bytes.
The number of DATA8 bytes is fixed for a given CONTROL. After receiving the correct number of DATA8 bytes, operation resumes to the MIDI device.

| Ctrl # |  | CONTROL NAME |  |  | Action |
| --- | --- | --- | --- | --- | --- |
| 1h |  | WRT_MEM |  |  | Initialize PC to SAM9407 transfer |
| 2h |  | RD_MEM |  |  | Initialize SAM9407 to PC transfer |
| 3h |  | GET_MMT |  |  | Get Memory Mapping Table address |
| 4h |  | SET_MMT |  |  | Set Memory Mapping Table address |
| 7h |  | MASTER_VOL |  |  | Master volume |
| 8h |  | REC_MODE |  |  | Select record mode |
| 0Bh |  | TRANS_ONOFF |  |  | -Enable/disable pitch shifter function(1) |
| 0Ch |  | TRANS_GMCH |  |  | -Enable/disable pitch shifter on General Midi channels |
| 0Dh |  | TRANS_VAL |  |  | - Pitch shifter value |
| 0Eh |  | TRANS_REVSEND |  |  | - Pitch shifter reverb send |
| 0Fh |  | TRANS_CHRSEND |  |  | - Pitch shifter chorus send |
| 10H |  | EQ_LBL |  |  | Equalizer low band left |
| 11H |  | EQ_MLBL |  |  | Equalizer med low band left |
| 12H |  | EQ_MHBL |  |  | Equalizer med high band left |
| 13H |  | EQ_HBL |  |  | Equalizer high band left |
| 14H |  | EQ_LBR |  |  | Equalizer low band right |
| 15H |  | EQ_MLBR |  |  | Equalizer med low band right |
| 16H |  | EQ_MHBR |  |  | Equalizer med high band right |
| 17H |  | EQ_HBR |  |  | Equalizer high band right |
| 18H |  | EQF_LB |  |  | Equalizer low band frequency |
| 19H |  | EQF_MLB |  |  | Equalizer med low band frequency |
| 1AH |  | EQF_MHB |  |  | Equalizer med high band frequency |
| 1BH |  | EQF_HB |  |  | Equalizer high band frequency |
| 20H |  | AUD_SEL |  |  | Audio1/2 input select (1) |
| 21H |  | AUD_GAINL |  |  | Audio Left input gain (1) |
| 22H |  | AUD_GAINR |  |  | Audio Right input gain (1) |
| 25H |  | GMREV_SEND |  |  | General Midi Reverb Send |
| 26H |  | GMCHR_SEND |  |  | General Midi Chorus Send |
| 27H |  | AUDREV_SEND |  |  | Audio Reverb Send |
| 28H |  | ECH_LEV |  |  | Echo level applied on audio in (2) |
| 29H |  | ECH_TIM |  |  | Echo time applied on audio in (2) |
| 2AH |  | ECH_FEED |  |  | Echo feedback applied on audio in (2) |
| 30H |  | SUR_VOL |  |  | Surround effect volume |
| 31H |  | SUR_DEL |  |  | Surround effect delay |
| 32H |  | SUR_INP |  |  | Input mono/stereo select for surround |
| 33H |  | SUR_24 |  |  | 2 or 4 speakers output select for surround |
| 34H |  | AUDL_VOL |  |  | Left Channel Audio in volume (2) |
| 35H |  | AUDR_VOL |  |  | Right Channel Audio in volume (2) |
| 36H |  | AUDL_PAN |  |  | Left Channel Audio in pan (2) |
| 37H |  | AUDR_PAN |  |  | Right Channel Audio in pan (2) |
| 38H |  | GM_VOL |  |  | General Midi volume |
| 39H |  | GM_PAN |  |  | General Midi pan |
| 3AH |  | REV_VOL |  |  | Reverb general volume |
| 3BH |  | CHR_VOL |  |  | Chorus general volume |
| 3DH |  | EN_MIDOUT |  |  | Enable midi out |
| 3FH |  | UART_MOD |  |  | Switch to UART mode |
| 40h | W_OPEN |  | Open channel with specified format and sampling rate. |  |  |
| 41h | W_CLOSE |  | Close channel. (3) |  |  |
| 42h | W_START |  | Request SAM9407 to init its streaming audio data Emitter / Receiver |  |  |
| 43h | END_XFER |  | Sent to SAM9407 after end of XFER. |  |  |
| 44h | W_PITCH |  | Change the pitch of the wave during playback. (1) |  |  |
| 45h | W_VOLLEFT |  | Change the current left volume (1) |  |  |
| 46h | W_VOLRIGHT |  | Change the current right volume (1) |  |  |
| 47h | W_VOLAUXLEFT |  | Change the left auxiliary level (1) |  |  |
| 48h |  | GEN_INT |  | Generate an interrupt (used to check if 9407 board installed) |  |
| 49h | W_VOLAUXRIGHT |  | Change the right auxiliary level. (1) |  |  |
| 4AH | W_FILT_FC |  | Change filter cutoff frequency |  |  |
| 4Bh | W_FILT_Q |  | Change filter resonance (2) |  |  |
| 51h |  | GET_VOI |  | Get number of voices available |  |
| 52h |  | VOI_OPEN |  | Open one voice |  |
| 53h |  | VOI_CLOSE |  | Close one voice |  |
| 54h |  | VOI_START |  | Start play voice |  |
| 55h |  | VOI_STOP |  | Stop play voice |  |
| 56h |  | VOI_VOL |  | Voice output volumes |  |
| 57h |  | VOI_MAIN |  | Main channel send volumes |  |
| 58h |  | VOI_PITCH |  | Voice pitch |  |
| 59h |  | VOI_AUX |  | Aux channel send volumes |  |
| 5Ah |  | VOI_FILT |  | Voice filter |  |
| 5Bh |  | VOI_MEM |  | Defines memory used by voice |  |
| 5Ch |  | GET_POS |  | Return position of current reading |  |
| 5Dh |  | ADD_POS |  | Add an offset to current reading |  |
| 60h |  | WAVE_ASS |  |  | Wave audio output assignment (3) |
| 61h |  | MOD_ASS |  |  | MOD player audio output assignment (3) |
| 62h |  | GM_POST |  |  | Post effects applied on general midi (4) |
| 63h |  | WAVE_POST |  |  | Post effects applied on wave (4) |
| 64h |  | MOD_POST |  |  | Post effects applied on MOD player (4) |
| 65h |  | AUDECH_POST |  |  | Post effects applied on Audio in and echo(4) |
| 66h |  | EFF_POST |  |  | Post effects applied on Reverb-chorus (4) |
| 68H |  | ECH_ONOFF |  |  | Echo On/Off (3) |
| 69H |  | REV_TYPE |  |  | Reverb program select |
| 6AH |  | CHR_TYPE |  |  | Chorus program select |
| 6BH |  | EQU_TYPE |  |  | Equalizer type (3) |
| 6CH |  | REV_ONOFF |  |  | Reverb On/Off (3) |
| 6DH |  | CHR_ONOFF |  |  | Chorus On/Off (3) |
| 6EH |  | SUR_ONOFF |  |  | Surround On/Off (3) |
| 6FH |  | AUD_ONOFF |  |  | Audio On/Off (3) |
| 70H |  | HOT_RES |  |  | Hot reset |
| 72H |  | POLY_64 |  |  | Enable 64 voice polyphony (3) |
| 74H |  | CHR_DEL |  |  | Chorus delay |
| 75H |  | CHR_FEED |  |  | Chorus feedback |
| 76H |  | CHR_RATE |  |  | Chorus rate |
| 77H |  | CHR_DEPTH |  |  | Chorus depth |
| 78H |  | REV_TIME |  |  | Reverb time |
| 79H |  | REV_FEED |  |  | Reverb feedback |
| BEH |  | EN_CONTROL |  |  | Enable dream control in stand alone mode |
| FFh |  | RESET |  |  | Reset UART mode |

# GENERAL DEVICE

## System messages

| Ctrl # | CONTROL NAME | Parameters (Data) | Action | Answer |
| --- | --- | --- | --- | --- |
| 07h | MASTER_VOL | -Data (byte 0-FFh,FFh) | Master volume |  |
| 48h | GEN_INT | -Data=0 (byte) | Generate an interrupt | Id=11 Data=088h |
| 70H | HOT_RES | -Data=011H | Hot reset | Id=10 Data=0 |
| BEH | EN_CONTROL | None | Enable dream control in stand alone mode |  |
| FFh | RESET | None | Reset UART mode |  |
| 3FH | UART_MOD | None | Switch to UART mode | Id=00 Data= 0FEh |

- MASTER_VOL :
Master volume.
Data range : 0-FFh. Default=0FFh.

- GEN_INT :
This command is used to detect SAM9407 board.
It sends back 088H data and generates interrupt

- HOT_RES :
Equivalent to hardware reset. Firmware should be reloaded..

- EN_CONTROL:
This control has been implemented to enable to send any control even in Stand alone mode.
It enable to send only one control, this means that each control sent in stand alone mode should start with EN_CONTROL control.

- RESET:
Switch SAM9407 in stand alone mode

- UART_MODE:
Switch SAM9407 in UART mode

## Memory messages

| Ctrl # | CONTROL NAME | Parameters (Data) | Action | Answer |
| --- | --- | --- | --- | --- |
| 1h | WRT_MEM | -Start (Dword) -Count (word) | Initialize PC to SAM9407 transfer | Id=11 Data = ACh / ABh |
| 2h | RD_MEM | -Start (Dword) -Count (word) | Initialize SAM9407 to PC transfer | Id=11 Data = ACh / ABh |
| 3h | GET_MMT | -Data=0(byte) | Get Memory Mapping Table address | Id=1Id=00 Data=Address (Dword) |
| 4h | SET_MMT | -Address (Dword) | Set Memory Mapping Table address |  |

- GET_MMT, SET_MMT:
The Memory Mapping Table is a fixed length table stored in SAM9407 memory.
This table is described 7in « Firmware Stucture » chapter..

- Wrt_mem, Rd_mem :
Initializes the basic transfer in burst mode between PC memory and SAM9407 memory.
All transfers must be 16 bit data transfers

Start address is given as a double word (4 bytes) :
 Ad1,Ad2 offset inside a 64K page, 
 Ad3,Ad4 64K page number.

Count is number of words to transfer on 2 bytes (Lsb first) : 
 Cnt1,Cnt2.
 Count is limited to 32K words maximum (cnt2|cnt1=4000h max.)

Warning !! : A transfer cannot cross a 64K words page boundary : 
	(ad2|ad1)+(cnt2|cnt1) < 10000H

When start address and count parameters have been sent, the PC must wait for the acknowledgment byte ACh before starting transfer. Under normal traffic conditions, this acknowledgment byte is received within less than 100µs.

If transfer in burst mode is already used by streaming audio (wave play/record), SAM9407 returns byte ABh and abort rd_mem or wrt_mem command. In that case, command rd_mem and wrt_mem must be resent after having ended transfer requested by streaming audio.

ABh byte can also be returned if one transfer is done with wrong count word (case of PC software mistake). In that case, next transfers cannot be done. The only solution is to correct PC software.

The actual transfer is done through port DATA16 using REP OUTSW (if WRT_MEM) or REP INSW (if RD_MEM) assembler instructions.

Example :

; segments and SI (resp.DI) already initialized to point to user buffer

mov	dx,MPU_base+2

mov	cx,count

rep	outsw             or	rep	insw

## Config messages

All the configuration controls reinitialize the SAM9407 firmware.
Therefore control 3Fh should be sent if UART mode is required.
All streaming and static wave channels are closed. General midi reset occurs.

Following commands are reset to their default values :
 -UART_MODE
 -AUD_SEL, AUD_GAIN, -ECH_LEV, ECH_TIM, ECH_FEED
 -SUR_VOL, SUR_DEL, SUR_INP, SUR_24
 -GM_VOL, GM_PAN, REV_VOL, CHR_VOL
 -REV_TYPE, CHR_TYPE
 -EQ_xxx, EQF_xxx

Only following commands are not reset :
 -WAVE_ASS, MOD_ASS
 -all xxx_POST commands
 - ECH_TYPE, EQU_TYPE

Memory tables and sound banks are not modified.

| Ctrl # | CONTROL NAME | Parameters (Data) | Action | Answer |
| --- | --- | --- | --- | --- |
| 6FH | AUD_ONOFF | -Data (byte 0/7Fh,7Fh) | Audio On/Off | Id=11 Data=0 |
| 68H | ECH_ONOFF | -Data(byte 0/7Fh,7Fh) | Echo On/Off | Id=11 Data=0 |
| 6CH | REV_ONOFF | -Data (byte 0/7Fh,7Fh) | Reverb On/Off | Id=11 Data=0 |
| 6DH | CHR_ONOFF | -Data (byte 0/7Fh,7Fh) | Chorus On/Off | Id=11 Data=0 |
| 6BH | EQU_TYPE | -Data(byte 0-2,0) | Equalizer type | Id=11 Data=0 |
| 6EH | SUR_ONOFF | -Data (byte 0/7Fh,7Fh) | Surround On/Off | Id=11 Data=0 |
| 72H | POLY_64 | -Data(byte 0/7Fh,0) | Enable 64 voice polyphony | ID=11 Data=0 |
| 60h | WAVE_ASS | -Data(byte 0/7Fh,0) | Wave audio output assignment | Id=11 Data=0 |
| 61h | MOD_ASS | -Data(byte 0/7Fh,0) | MOD player audio output assignment | Id=11 Data=0 |

- AUD_ONOFF :
	0			Audio in off
	7FH (Default)	Audio in  on, requires 1 extra slot

- ECH_ONOFF :
	0			echo applied on Audio in off
	07FH (Default)	echo applied on Audio in on, requires 2 extra slots

- REV_ONOFF :
	0  			reverb off
	7FH  (Default)	reverb on, requires 13 extra slots

- CHR_ONOFF :
	0			chorus off
	7FH (Default)		chorus on, requires 3 extra slots

- EQU_TYPE :	Select type of equalizer.
	0(Default)		4 band equalizer Requires 8 extra slots.
	1 :		 	2 band equalizer (low, high). Requires 4 extra slots.
	2 : 			no equalizer.

- SUR_ONOFF :
	0 : 			surround effect off
	7FH (Default)	surround effect on, requires 1 extra slot

POLY_64 : (Optional)
	0 (Default)		off 
	7FH			on, 64 voices of polyphony

Available only if firmware is downloaded into SRAM instead of DRAM (because refresh is disabled when using this command).
Set polyphony to 64 voices for midi, wave and mod player modules
When POLY_64 is on, equalizer, surround, reverb, chorus, audio in/echo, record functions are off. All commands concerning these functions are not active. The only way to active them again is to send POLY_64 off.

When POLY_64 is on, only midi, wave play and mod player are available  Midi is output only on Main speaker output. Wave and mod player are always configured in 4 speaker output (wave_ass=7Fh, mod_ass=7Fh).

- WAVE_ASS :
Data = 0 : Wave module is configured for 2 speakers output, reverb and chorus can be applied to each wave :
    - Command 45h, W_VOLLEFT : main left volume
    - Command 46h, W_VOLRIGHT : main right volume
    - Command 47h, W_VOLAUXLEFT : reverb send volume
    - Command 48h, W_VOLAUXRIGHT : chorus send volume
Data = 7Fh : Wave module is configured for 4 speakers output. Reverb and chorus are not applied on waves :
    - Command 45h, W_VOLLEFT : main left volume
    - Command 46h, W_VOLRIGHT : main right  volume
    - Command 47h, W_VOLAUXLEFT : auxiliary left volume
    - Command 48h, W_VOLAUXRIGHT : auxiliary right volume
Default =0

- MOD_ASS :
Data=0 : MOD player module is configured for 2 speakers output, reverb and chorus are available :
    - Command 57h, VOI_MAIN : main send levels
    - Command 59h, VOI_AUX : effect send (left=reverb send, right=chorus send)
Data=7Fh : MOD player module is configured for 4 speakers output, reverb and chorus are disabled :
    - Command 57h, VOI_MAIN : main send levels
    - Command 59h, VOI_AUX : auxiliary send levels.
Default=0

## Routing messages

Post effects are surround + equalizer

| Ctrl # | CONTROL NAME | Parameters (Data) | Action | Answer |
| --- | --- | --- | --- | --- |
| 62h | GM_POST | -Data(byte 0/7Fh,7Fh) | Post effects applied on general midi |  |
| 63h | WAVE_POST | -Data(byte 0/7Fh,7Fh) | Post effects applied on wave |  |
| 64h | MOD_POST | -Data(byte 0/7Fh,7Fh) | Post effects applied on MOD player |  |
| 65h | AUDECH_POST | -Data(byte 0/7Fh,7Fh) | Post effects applied on Audio in and echo |  |
| 66h | EFF_POST | -Data(byte 0/7Fh,7Fh) | Post effects applied on Reverb-chorus |  |

- xxx_POST :

Post effects are surround and equalizer.

Post effects can be separately applied on each module. However general settings of post effects (EQ_xxx, EQF_xxx, EQU_TYPE, SUR_VOL, SUR_DEL, SUR_INP and SUR_24) are common for all modules.

Data=0 : post effects not applied on module.

Data=7Fh : post effects applied on module.

Default value = 07Fh for all modules.

# MIDI DEVICE

The current implementation specifies 32 MIDI channels for internal Wave table synthesis Device and a single 16 channels MIDI device for external MIDI device.

MIDI device is enable only in UART mode.
MIDI device is MPU401 UART mode compliant

This mode involved a specific protocol for MIDI device: 
	MIDI messages don’t required any CONTROL but only DATA.
	Writing DATA8 will send MIDI data to MIDI OUT and the wavetable synthesis.
	MIDI data received from MIDI IN can be read as DATA8 with ID0-1=00

The detailed MIDI implementation is detailed in chapter 3

GM_VOL, GM_PAN & EN_MIDOUT are not MPU401 compliant messages, unlike most of midi messages they required CONTROL.

In stand-alone mode, the SAM9407  plays whatever is received on the MIDI IN serial line, MIDI OUT is disabled.

MIDI OUT (external cable) is enabled after 3FH (UART mode) and 3DH (enable midi out)

MIDI OUT is disabled after any other control than 3FH & 3DH . This means that any dream control should be followed by 3DH control to restore midi out. This has been done to avoid to send the data of control messages into MIDI OUT.

Only data from channels 1 to 16 are output on external MIDI OUT.

The channel selector is controled with B0H (channel  0-15) and B1H (channel 16-31).

## Midi message path:

## MIDI messages

| Ctrl # | CONTROL NAME | Parameters (Data) | Action | Answer |
| --- | --- | --- | --- | --- |
| 38H | GM_VOL | -Data(byte 0-FFh,FFh) | General Midi volume |  |
| 39H | GM_PAN | -Data(byte 0-7Fh,40h) | General Midi pan |  |
| 3DH | EN_MIDOUT | None | Enable midi out |  |
| B0H | MID_PORT0 | None | Select chan 0 to 15 |  |
| B1H | MID_PORT1 | None | Select chan 16 to 31 |  |

- EN_MIDOUT :
Each general device control (except 3FH=UART_MOD) disables MIDI out. To enable again MIDI out, EN_MIDOUT must be sent before sending MIDI data to port MPU_base.

- GM_VOL
Range 0-FFh, linear scale.
Default value : 	GM_VOL=0FFh

- GM_PAN
0=hard left, 40h=center, 7Fh=hard right.
Pseudo logarithmic scale.
Same as GM system exclusive message « 40h 00h 06h »
Default value :	GM_PAN=040h

- MID_PORT0
Select midi port 0 (channel 0 to 15)
All the next midi messages are sent to midi port 0

- MID_PORT1
Select midi port 1 (channel 16 to 31)
All the next midi messages are sent to midi port 1

# STREAMING AUDIO BUFFER DEVICE

I/O MESSAGES from PC -> SAM9407

| Ctrl # | CONTROL NAME |  | Parameters (Data) |  | Action |  | Answer |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 40h | W_OPEN |  | -Channel (Byte) -Format (Byte) -Sampling rate (Word) |  | Open channel with specified format and sampling rate. |  |  |  |
| 41h | W_CLOSE |  | -Channel (Byte) |  | Close channel. |  | Id=01 (3) Byte=0Cx |  |
| 42h | W_START |  | -Channel (Byte) |  | Request SAM9407 to initialize its streaming audio data Emitter / Receiver |  |  |  |
| 43h | END_XFER |  | -Channel (Byte) |  | Sent to SAM9407 after end of XFER. |  |  |  |
| 44h | W_PITCH |  | -Channel (Byte) -Sampling rate (Word) |  | Change the pitch of the wave during playback. (1) |  |  |  |
| 45h | W_VOLLEFT |  | -Channel (Byte) -Volume (Word) |  | Change the current left volume (1) |  |  |  |
| 46h | W_VOLRIGHT |  | -Channel (Byte) -Volume (Word) |  | Change the current right volume (1) |  |  |  |
| 47h | W_VOLAUXLEFT |  | -Channel (Byte) -Value (Word) |  | Change the left auxiliary level (1) |  |  |  |
| 49h | W_VOLAUXRIGHT |  | -Channel (Byte) -Value (Word) |  | Change the right auxiliary level. (1) |  |  |  |
| 4AH | W_FILT_FC |  | -Channel (Byte) -Value (Word) |  | Change filter cutoff frequency |  |  |  |
| 4Bh | W_FILT_Q |  | -Channel (Byte) -Value (Word) |  | Change filter resonance (2) |  |  |  |
| 8h |  | REC_MODE |  | -Data (byte 0/7Eh/7Fh,0) |  | Select record mode |  |  |

Notes

(1) : Play mode only

(2) : Mono mode.only

(3) :Play mode.only  Byte returned is 0C0h + channel number (0-7). Acknowledge byte is sent when final half buffer read is completed.

I/O MESSAGES from SAM9407 -> PC

IRQ is used by SAM9407 to start streaming audio data block transfers between the PC and the SAM9407.

STATUS [5,4] = [ID1,ID0] with ID1=0 and ID0=1

DATA8[7..0]= Channel number (0 to 8)

The SAM9407 streaming audio buffer size is allocated by the PC using the memory mapping table (MMT) resources, (see memory table).

DETAILED PARAMETERS

For each command Channel is coded as follows:

From 0 to 7 for PLAY, 8 for RECORD

W_OPEN

Format 		byte : 			0000 00 ST 8B

8B : 	0 if 16 bits, 1 if 8 bits.

ST : 	0 if mono, 1 if stereo.

Sampling Rate 	word:			unsigned value from 0 to 48000 Hz.

W_VOLLEFT, W_VOLRIGHT, W_VOLAUXLEFT, W_VOLAUXRIGHT

Volume 		word:			linear unsigned value from 0 to 65535

W_PITCH

Sampling Rate	word			unsigned value from 0 to 48000 Hz.

W_FILT_FC

FC			 word			0 = filter closed, 0FFFFH filter open

W_FILT_Q

Q		 	word			0 = maximum resonance, 0FFFFH no resonance

- REC_MODE :

Data=0 : record only Audio In

Data=07Fh : record all signals going into Equalizer-Surround module (see Signal Processing synoptic in « overview » chapter)

Default = Data=0

Note : When in mode 07Fh, all commands xxx_POST have a double function : they are selecting if module is going into Equalizer-Surround module but also if module has to be record or not.

Data=07Eh : record all signals from surround bus (see Signal Processing synoptic)

# STATIC AUDIO BUFFER DEVICE

The Static audio  device can be used to provide hardware accelerator for Windows 95 Direct Sound.

It can also be used to implement specific MOD player Windows drivers.

| Ctrl # | CONTROL NAME | Parameters (Data) | Action | Answer |
| --- | --- | --- | --- | --- |
| 51h | GET_VOI | -Data=0 (byte) | Get number of voices available | Id=10 Voice count (1 byte) |
| 52h | VOI_OPEN | -Voice (byte) | Open one voice | No ack |
| 53h | VOI_CLOSE | -Voice (byte) | Close one voice |  |
| 54h | VOI_START | -Voice (byte) | Start play voice |  |
| 55h | VOI_STOP | -Voice (byte) | Stop play voice |  |
| 56h | VOI_VOL | -Voice (byte) - Volume (byte) | Voice output volumes |  |
| 57h | VOI_MAIN | -Voice (byte) - Main right (byte) - Main left (byte) | Main channel send volumes |  |
| 58h | VOI_PITCH | - Voice (byte) - Pitch (word) | Voice pitch |  |
| 59h | VOI_AUX | - Voice (byte) - Aux right (byte) - Aux left (byte) | Aux channel send volumes |  |
| 5Ah | VOI_FILT | - Voice (byte) - Q (byte) - Fc (byte) | Voice filter |  |
| 5Bh | VOI_MEM | - Voice (byte) - Format (byte) - Bank (byte) - Start (3 bytes) - Bank loop (byte) - Loop (3 bytes) - Bank end (byte) - End (3 bytes) | Defines memory used by voice |  |
| 5Ch | GET_POS | - Voice (byte) | Return position of current reading | Id=10 bank (byte) offset in bank (3 bytes) |
| 5Dh | ADD_POS | - Voice (byte) - Bank (byte) - Offset (3 bytes) | Add an offset to current reading |  |

## DETAILS

- GET_VOI:

The returned byte Voice_count is the maximum number of voices that can be opened at the same time in the application. It is the total number of voices available for MOD player, Wave player and MIDI.

MOD player and Wave player have higher priority than MIDI. However MOD player has same priority as Wave player.

For example, suppose Voice_count = 32. If no voices are already opened for Wave player, MOD player can used 32 voices (but in that case no voices will be available for MIDI playing). If n voices are used for Wave player, MOD player has only 32-n voices available. Because Wave player and MOD player is usually included in the same driver it is the driver programmer responsibility to memorize how many voices are opened for Wave and for MOD.

MOD voices always start from 0 until maximum allowed (Voice_count-n-1), even if Wave player is active. Voices are dynamically allocated and do not correspond to physical slots of the SAM9407.

Note that crossbank type waves request two voices for each note played.

- VOI_OPEN :

Open one voice.

Voice byte :

| D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| C | 0 | 0 | v4 | v3 | v2 | v1 | v0 |

v4|v0 : voice to open

C : 0- « in bank » voice, 1- « cross bank » voice

Cross bank voice actually reserves two internal voices.

Note : if pitch shifter device is implemented in firmware, this cross bank mode is not available.

- VOI_CLOSE

VOI_CLOSE is immediately closing the voice.

Stops the voice softly in less than 5ms.

VOI_OPEN must be sent to use the voice again.

- VOI_STOP

VOI_STOP stops the voice softly in less than 5 ms.

This voice can be used again immediately without clicking for another wave (VOI_START without sending again VOI_OPEN).

- VOI_START

Sending VOI_START supposes that VOI_OPEN control has been sent before but also that VOI_MEM, VOI_VOL, VOI_PAN ... has been sent to initialize voice parameters.

VOI_CLOSE and VOI_STOP reset all parameters to undefined values.

Only VOI_FILT is automatically initialized to : Q=0FFh, Fc=0FFh (filter open)

- VOI_VOL :

Volume for main audio output.

Slide the volume to new value in less than 5ms.

Linear volume. Range 0-FFh

- VOI_MAIN :

Send for main audio output. Linear scale.

Main right : range 0-FFH, main right send level.

Main left : range 0-FFH, main left send level.

- VOI_PITCH :

Pitch given in 2 bytes, low byte first.

400h (low byte=0, high byte=4) correspond to nominal frequency (Current version of program runs with a sampling rate of 37.5 KHz, due to current SIMM DRAM cycle times).

Pitch is linearly scaled (200h = half nominal frequency).

- VOI_AUX :

Send level for auxiliary audio output or effect.

Depending on command MOD_ASS (61h), auxiliary output can be also effect send (see 5. General device command)

Linear scale.

Aux right : range 0-FFh, auxiliary right send level or chorus send.

Aux left : range 0-FFh, auxiliary left send level or effect send.

- Voi_Filt :

First byte : Q (resonance) : range 0-0ffh (0ffh=no resonance, 0=maximum of resonance)

2nd byte : Fc (cut frequency) : range 0-0ffh (0ffh=filter open, 0=filter closed)

- VOI_MEM :

Defines addresses of wave used by voice.

SAM9407 access to external memory is made through banks of 256K samples. Waves can be « in bank » or « cross bank « . Cross bank waves take two polyphony partials to be played. Therefore it is advisable to use « in bank » waves for optimum polyphony performance.

Waves can be stored in dram into 16 bit or 8 bit format. Both formats can be used at same time (for example, one 16 bit wave can be stored from address M till N-1 and two 8 bit waves can be stored from address N till P-1, etc...)

- Format byte :

| D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| f7 | f6 | f5 | f4 | f3 | f2 | f1 | f0 |

f7 	0 :16 bit sample, 1 : 8 bit sample

f6 	used only if f7=1 (8 bit sample)

0 : sample in low byte of word, 1: sample in high byte of word

f5-f0 loop type	0 : forward loop (Play to loop end and jump to loop start)

1 : reverse loop (Play to loop end, back to loop start, forward to loop end ...)

2 : reverse loop with sign inversion (Play to loop end, invert sign and back to loop start, invert sign and forward to loop end ...)

Only loop type 0 is available if a crossbank sample has been defined for VOI_OPEN.

- Bank(byte)|Start(3bytes) : defines the sample start point

- Bank loop(byte)|Loop(3bytes) : defines the loop point of the sample

- Bank end(byte)|End(3 bytes) : defines the end point of the sample

3 byte offset inside a bank is coded LSB first :

| Byte # | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | a7 | a6 | a5 | a4 | a3 | a2 | a1 | a0 |
| 2 | a15 | a14 | a13 | a12 | a11 | a10 | a9 | a8 |
| 3 | 0 | 0 | 0 | 0 | 0 | 0 | a17 | a16 |

« In bank » samples have Bank = Bank loop = Bank end

Looping feature is necessary for all waves. « One-shot » waves must be done by adding n zero samples to the wave and setting loop point to end-n (n=8 recommended so that to allow drastic pitch change controls).

For cross bank samples loop and/or end points in the last eight locations of a bank should be avoided.

See also appendix A « MEMORY MAPPING TABLE » to know which part of memory can be used for MOD player.

- GET_POS

Return current reading position of the voice.

Useful to know if wave has reached final loop or not.

Return position value on 4 bytes :

byt1 = bank

bytes 2 to 4 = offset inside bank

byt2 = bit7-0

byt3 = bit15-8

byt4 = bit17-16

- ADD_POS

Add to current reading position an offset.

Given on 4 bytes :

byt1 = bank offset

bytes 2 to 4 = offset

byt2 = bit7-0

byt3 = bit15-8

byt4 = bit17-16

If current position + offset is going behind end point, position is set to end point.

- Writing and reading dram

Writing and reading into dram is done using general device controls 1 (WRT_MEM) and 2 (RD_MEM).

These controls  are using a start address defined on 4 bytes ad1, ad2, ad3, ad4 :

ad1 = | a7  a6  a5  a4  a3  a2  a1  a0  |

ad2 = | a15 a14 a13 a12 a11 a10 a9  a8  |

ad3 = | a23 a22 a21 a20 a19 a18 a17 a16 |

ad4 = | a31 a30 a29 a28 a27 a26 a25 a24 |

ad4|ad3 = 64K page number.

ad2|ad1= offset inside a 64K page

The correlation with 256K bank structure used for MOD driver is :

bank number = bits a25-a18

offset inside bank = bits a17-a0

bit a31-a26 = 0.

# AUDIO IN & ECHO DEVICE

| Ctrl # | CONTROL NAME | Parameters (Data) | Action | Answer |
| --- | --- | --- | --- | --- |
| 20H | AUD_SEL | -Data(byte 0/7Fh,7Fh) | Audio1/2 input select (1) |  |
| 21H | AUD_GAINL | -Data(byte 0-7Fh,0) | Audio Left input gain (1) |  |
| 22H | AUD_GAINR | -Data(byte 0-7Fh,0) | Audio Right input gain (1) |  |
| 27H | AUDREV_SEND | -Data(byte 0-7Fh,0) | Audio Reverb Send |  |
| 34H | AUDL_VOL | -Data(byte 0-FFh,FFh) | Left Channel Audio in volume (2) |  |
| 35H | AUDR_VOL | -Data(byte 0-FFh,FFh) | Right Channel Audio in volume (2) |  |
| 36H | AUDL_PAN | -Data(byte 0-7Fh,0) | Left Channel Audio in pan (2) |  |
| 37H | AUDR_PAN | -Data(byte 0-7Fh,7Fh) | Right Channel Audio in pan (2) |  |
| 28H | ECH_LEV | -Data(byte 0-7Fh,0) | Echo level applied on audio in (2) |  |
| 29H | ECH_TIM | -Data(byte 0-7Fh,2Bh) | Echo time applied on audio in (2) |  |
| 2AH | ECH_FEED | -Data(byte 0-7Fh,40h) | Echo feedback applied on audio in (2) |  |

(1) Only if CS4216/CS4218 type Codec used (2 stereo audio inputs Codec)

(2) Assumes ADC used (at least 1 stereo audio input Codec)

- AUD_xxx :

These 3 commands can be used only if a Codec with 2 audio stereo inputs and adjustable gain on audio input is used (typically CS4216 or CS4218)

AUD_SEL : 0= select audio stereo input 1, 07Fh=select audio stereo input 2 (default 7Fh)

AUD_GAINL : 0=+0dB, 07Fh=+22.5dB on audio left input (default 0)

AUD_GAINR : 0=+0dB, 07Fh=+22.5dB on audio right input (default 0)

On 94PC32 board, audio input 2 is J4 connector and audio input 1 is J6 connector

- ECH_xxx :

Controls for echo applied on audio input.

Available only if echo set on with command ECH_ONOFF

ECH_LEV: 0 to 07Fh (Default 0)

ECH_TIM (if ech_type=07Fh only) : 0 =shortest to 7Fh=longest (default 2Bh)

ECH_FEED (if ech_type=07Fh only) : 0=no feedback, 7Fh=maximum feedback (default 40h)

- AUDREV_LEV :

Reverb send level for audio in

Data=0 to 07Fh (Default=0)

Reverb used is same as reverb used for General Midi

# REVERB DEVICE

| Ctrl # | CONTROL NAME | Parameters (Data) | Action | Answer |
| --- | --- | --- | --- | --- |
| 69H | REV_TYPE | -Data(byte 0-7,4) | Reverb program select |  |
| 3AH | REV_VOL | -Data(byte 0-FFh) | Reverb general volume |  |
| 78H | REV_TIME | -Data(byte 0-7Fh) | Reverb time |  |
| 79H | REV_FEED | -Data(byte 0-7Fh) | Reverb feedback |  |
| 25H | GMREV_SEND | -Data(byte 0-FFh,80h) | General Midi Reverb Send |  |

- REV_TYPE :	Reverb program.

Same as GM system exclusive message « 40h 01h 30h » or GM control 80.

| room1 | room2 | room3 | hall1 | hall2 | plate | delay | pan delay |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0H | 1H | 2H | 3H | 4H | 5H | 6H | 7H |

Default=4 (hall2)

REV_VOL:		Reverb volume

Same as GM system exclusive message « 40h 01h 33h »

Default values:

| room1 | room2 | room3 | hall1 | hall2 | plate | delay | pan delay |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 90H | 90H | 90H | C0H | 90H | 90H | FFH | FFH |

- REV_TIME :	Reverb time.

Same as GM system exclusive message « 40h 01h 34h »

Default values:

| room1 | room2 | room3 | hall1 | hall2 | plate | delay | pan delay |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 7FH | 7FH | 7FH | 7FH | 7FH | 7FH | 18H | 7FH |

- REV_FEED :	Reverb delay feedback.

Only if reverb number=6 or 7 (delays)

This command is same as GM system exclusive message « 40h 01h 35h »

Default values:

| delay | pan delay |
| --- | --- |
| 22H | 26H |

-GMREV_SEND:	Modify reverb send level for General Midi.

80H: 		original reverb send levels of midi sequence not modified

0 to 7FH : 	original reverb send levels decreased

81h to FFH :	original reverb send levels increased

Default=80h

# CHORUS DEVICE

| Ctrl # | CONTROL NAME | Parameters (Data) | Action | Answer |
| --- | --- | --- | --- | --- |
| 6AH | CHR_TYPE | -Data(byte 0-7,2) | Chorus program select |  |
| 3BH | CHR_VOL | -Data(byte 0-FFh) | Chorus general volume |  |
| 74H | CHR_DEL | -Data(byte 0-7Fh) | Chorus delay |  |
| 75H | CHR_FEED | -Data(byte 0-7Fh) | Chorus feedback |  |
| 76H | CHR_RATE | -Data(byte 0-7Fh) | Chorus rate |  |
| 77H | CHR_DEPTH | -Data(byte 0-7Fh) | Chorus depth |  |
| 26H | GMCHR_SEND | -Data(byte 0-FFh,80h) | General Midi Chorus Send |  |

- CHR_TYPE :	Chorus program.

Same as GM system exclusive message « 40h 01h 38h » or GM control 81.

| chorus1 | chorus2 | chorus3 | chorus4 | FB chorus | flanger | short del | FB delay |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 00H | 01H | 02H | 03H | 04H | 05H | 06H | 07H |

Default= 2 (chorus3)

- CHR_VOL :	Chorus Volume

Same as GM system exclusive message « 40h 01h 3Ah »

- CHR_DEL : 	Chorus delay

Same as GM system exclusive message « 40h 01h 3Ch »

- CHR_VOL :	Chorus Volume

Same as GM system exclusive message « 40h 01h 3Ah »

- CHR_FEED :	Chorus feedback

Same as GM system exclusive message « 40h 01h 3Bh »

- CHR_RATE :	Chorus rate

Same as GM system exclusive message « 40h 01h 3Dh »

- CHR_DEPTH :	Chorus depth

Same as GM system exclusive message « 40h 01h 3Eh »

- CHR_VOL :	Chorus Volume

Same as GM system exclusive message « 40h 01h 3Ah »

GMCHR_SEND :	Modify chorus send level for General Midi.

Data=080h : original chorus send levels of midi sequence not modified

Data=0 to 07Fh : original chorus send levels decreased

Data=081h to 0ffh : original chorus send levels increased

Default=80h

Default values:

|  | chorus1 | chorus2 | chorus3 | chorus4 | FB chorus | flanger | short del | FB delay |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHR_VOL | 90H | 90H | 90H | 90H | 90H | 90H | FFH | FFH |
| CHR_DEL | 4BH | 40H | 40H | 2BH | 7FH | 56H | 7FH | 7FH |
| CHR_FEED | 00H | 07H | 09H | 0CH | 48H | 7FH | 00H | 50H |
| CHR_RATE | 03H | 09H | 03H | 09H | 02H | 01H | 00H | 00H |
| CHR_DEPTH | 05H | 13H | 13H | 10H | 0CH | 03H | 00H | 00H |

# EQUALIZER DEVICE

| Ctrl # | CONTROL NAME | Parameters (Data) | Action | Answer |
| --- | --- | --- | --- | --- |
| 10H | EQ_LBL | -Level (byte 0-7Fh,60h) | Equalizer low band left |  |
| 11H | EQ_MLBL | -Level (byte 0-7Fh,40h) | Equalizer med low band left |  |
| 12H | EQ_MHBL | -Level (byte 0-7Fh,40h) | Equalizer med high band left |  |
| 13H | EQ_HBL | -Level (byte 0-7Fh,60h) | Equalizer high band left |  |
| 14H | EQ_LBR | -Level (byte 0-7Fh,60h) | Equalizer low band right |  |
| 15H | EQ_MLBR | -Level (byte 0-7Fh,40h) | Equalizer med low band right |  |
| 16H | EQ_MHBR | -Level (byte 0-7Fh,40h) | Equalizer med high band right |  |
| 17H | EQ_HBR | -Level (byte 0-7Fh,60h) | Equalizer high band right |  |
| 18H | EQF_LB | -Data (byte 0-7Fh,0Ch) | Equalizer low band frequency |  |
| 19H | EQF_MLB | -Data (byte 0-7Fh,1Bh) | Equalizer med low band frequency |  |
| 1AH | EQF_MHB | -Data (byte 0-7Fh,72h) | Equalizer med high band frequency |  |
| 1BH | EQF_HB | -Data (byte 0-7Fh,40h) | Equalizer high band frequency |  |

EQ_xxx		Band level

| 00H | 20H | 40H | 60H | 7FH |
| --- | --- | --- | --- | --- |
| -12dB | -6dB | 0dB | +6dB | +12dB |

Default =060h (+6dB) for LB-HB, =040h(0dB) for MLB-MHB

, EQF_xxx :		Band frequency (0-7Fh), linear scale

| Band | Range | Default |
| --- | --- | --- |
| LB | 0-4.7Khz | 0CH |
| MLB | 0-4.2Khz | 1BH |
| MHB | 0-4.2Khz | 72H |
| HB | 0-18.75Khz | 40H |

# SURROUND DEVICE

| Ctrl # | CONTROL NAME | Parameters (Data) | Action | Answer |
| --- | --- | --- | --- | --- |
| 30H | SUR_VOL | -Data(byte 0-FFh,0) | Surround effect volume |  |
| 31H | SUR_DEL | -Data(byte 0-7Fh,2) | Surround effect delay |  |
| 32H | SUR_INP | -Data(byte 0/7Fh,0) | Input mono/stereo select for surround |  |
| 33H | SUR_24 | -Data(byte 0/7Fh,0) | 2 or 4 speakers output select for surround |  |

- SUR_VOL :	 Surround effect volume.
	Default=0

- SUR_DEL :	 Delay line length
	Default=2

- SUR_INP :	 	Input type select
	0	Stereo (default),	Stereo wide,	 	Input to delay line is left - right.
	7FH	Mono,			Pseudo stereo		Input to delay line is left + right.

- SUR_24 :	 	Output type select
	0	2 speakers(default)	Surround output on main outputs.
	7FH	4 speakers 		Surround output on auxiliary output.

# PITCH SHIFTER DEVICE

This device is optional. May require some extended SIMM module.

| Ctrl # | CONTROL NAME | Parameters (Data) | Action | Answer |
| --- | --- | --- | --- | --- |
| 0Bh | TRANS_ONOFF | -Data (byte) (0-0Fh) | -Enable/disable pitch shifter function(1) | Id=11 Data = 0 |
| 0Ch | TRANS_GMCH | -Data (word) | -Enable/disable pitch shifter on General Midi channels |  |
| 0Dh | TRANS_VAL | -Data (word) | - Pitch shifter value |  |
| 0Eh | TRANS_REVSEND | -Data (byte) (0-FFh) | - Pitch shifter reverb send |  |
| 0Fh | TRANS_CHRSEND | -Data (byte) (0-FFh) | - Pitch shifter chorus send |  |

note : (1) This control reinitializes the sam9407 firmware

8-2 - DETAILS

-TRANS_ONOFF :

Enable/disable pitch shifter function. When enable, requires 6 extra slots. This function can be independently selected on 4 different modules (Audio IN/Echo, Mod Player, Streaming Audio, Midi).

Some limitations of pitch shifting mode :

- Value of pitch shifting is the same for all modules.

- When setting a module in pitch shifting mode, effect (or auxiliary) send function of corresponding module is disabled. User can only access to a global reverb and chorus send value which will be the same for all modules being in pitch shifting mode (see command trans_revsend, trans_chrsend).

- Setting a module in pitch shifting mode will automatically set xxx_POST command ON for this module. Sending xxx_POST command OFF will cancel pitch shifting mode.

- If Streaming audio module or Mod Player module is set in pitch shifting mode, all channels inside module are pitch shifted.

Only for Midi module, user can select which channels are or are not pitch shifted (by using command TRANS_GMCHAN).

- If Streaming audio module or Mod Player module is set in pitch shifting mode, command xxx_ASS (60h, 61h) is disabled (always 0).

Data (1 byte), bit D7-D0 with :

* D7-D4 : Don’t care

* D3 : Audio In/Echo pitch shifter On/Off. D3=1 ON , D3=0 OFF

If D3=1 :

command AUDECH_POST (65h)= 7Fh

command AUDREV_SEND (27h)= 0

* D2 : Mod Player pitch shifter On/Off. D2=1 ON , D2=0 OFF

If D2=1 :

command MOD_POST (64h)= 7Fh

command VOI_AUX (59h)= 0

command MOD_ASS (61h)=0

* D1 : Streaming Audio pitch shifter On/Off. D1=1 ON , D1=0 OFF

If D1=1 :

command WAVE_POST (63h)= 7Fh

command W_VOLAUX (47h,49h)= 0

command WAVE_ASS (60h)=0

* D0 : General Midi pitch shifter On/Off. D1=0 ON , D0=0 OFF

If D0=1 :

command GM_POST (62h)= 7Fh

Midi controls 91 (reverb send), 93 (chorus send)=0 for midi channels assigned to pitch shifting mode only. For other channels, controls 91,93 are working normally.

Note :

For general midi, pitch shifter is working only if corresponding instrument is made with a single slot algorithm. All pcm algorithms are single slot, only FM4Y4 algorithm (fm synthesis) is double slot algorithm.

- TRANS_GMCH :

Select which channels are pitch shifted.

Data on 1 word : D15-D0.

Di=1, GM channel i pitch shifted

Di=0, GM channel i not pitch shifted

Note :

Di=1 will be active only if command TRANS_ONOFF has been sent before with bit D0=1.

Sending TRANS_ONOFF with bit D0=0, reset TRANS_GMCH value.

Note :

When a midi channel is set in pitch shifting mode, « Note Sounding Priority » is set to maximum for this channel. Note being played will never be killed by other non pitch shifted channels.

- TRANS_VAL :

Value of pitch shifting.

Data on 1 word.

Default = 04000h (no shifting).

Linearly scaled. If F0 is original frequency, new frequency F is given by formula :

F=F0 * (TRANS_VAL) / 4000h.

Recommended range 3000h-5000h (higher values are distorting signal).

Note :

Pitch shifter can be used in time stretching application.

For general midi, this can be done in simultaneously sending a pitch bend information with the pitch shifting value. Dream can provide an utility program (pitshift.exe) giving value of pitch bend and pitch shifting to apply when changing tempo of midi sequence.

All GM instrument requiring time stretching must be defined on 94WINST editor with frequency in normal mode and not in fix mode (because pitch bend is not applied if frequency is in fix frequency mode).

- TRANS_REVSEND, TRANS_CHRSEND :

Reverb and chorus send levels applied on all pitch shifted signals.

Data range : 0 to 0ffh

Default=0

Chapter 3

FIRMWARE Structures

The SAM9407 data Structures in its memory. Those structures includes data  shared by firmware and client applications.

The SAM9407 memory stores 3 structures:

- Memory Mapping Table:	defines the memory mapping (block type, start address, length) & sound bank list (name, priority level)

- Midi Mapping Table:	defines the midi program & variation + sound bank source for each instrument

- Control table:		store the current values of all the device parameters that can be read back by client applications

# Memory Mapping Table (MMT)

Memory mapping table stays resident in 9407 board memory.
This table is split in two parts, memory block definition and MIDI sound bank definition.

## Memory mapping table:

| Nb | Type | Content |
| --- | --- | --- |
| 64 | 3*word | memory block definition |
| 8 | 5*word | MIDI sound bank definition |

## Memory Block definition:

| Nb | Type | Content |
| --- | --- | --- |
| 1 | word | Block type |
| 1 | long | 32 bits memory block start address |

## Block Type:

| data [15..8] | data [7] | data [6..0] | content |
| --- | --- | --- | --- |
| 0000 0000 | 0 | 000 0000 | memory size |
| 0000 0000 | 0 | 000 0001 | free block |
| 0000 0000 | r | 000 0010 | system reserved block |
| 0000 0000 | r | 000 0011 | Memory Mapping table |
| 0000 0000 | r | 000 0100 | MIDI Mapping Table |
| 0000 0000 | r | 000 0101 | MIDI Mapping Table extension |
| 0000 0000 | r | 000 0110 | CONTROL Table |
| 0000 i i i i | r | 001 0000 | MIDI sound parameter |
| dddd i i i i | r | 001 0001 | MIDI sound PCM |
| xxxx xxxx | 0 | 010 0000 | STREAMING WAVE buffer |
| xxxx xxxx | 0 | 011 0000 | STATIC WAVE buffer |
| 1111 1111 | 1 | 111 1111 | end of memory |

Memory size block should always be the first block of the Memory Block definition.
All others memory block definition are sorted by ascending order; the start address is given by the block itself the end address is given by the next block.
End of memory block should always be the last block.
Memory Mapping Table block, MIDI Mapping Table block and MIDI Mapping Table Extension block should always be consecutive.

Data[7]:
This bit is the RAM(0) / ROM(1) flag. It defined the block memory type. ROM blocks could not be modified by user.

Memory Size 0000H 
Total memory size including system reserved block.
This value is set by firmware and should not be modified by user.

Free Block 0001H
Free block is available for user. It could be filled or split by user.
Free block is always RAM type

System Reserved Block 0082H / 0002H 
System reserved block can be program or data firmware block.
It can also be used for an address area without effective memory.
This block even RAM type can’t be modified by user.

Memory Mapping Table 0083H / 0003H
This block is the current Memory Mapping Table location.
The MMT start address can be modified by the user.
User should also modified MIDI mapping table start address (to be consecutive). 
Thus MMT and MIDI mapping table should be relocate on memory by the user.
The user should also inform the firmware of the new MMT location using the SET_MMT MPU command (see FIRMWARE API)
MMT start address is initialize by the firmware.

MIDI Mapping Table 0084H / 0004H
The MIDI Mapping Table describes the program change and variation mapping according to the MIDI specification
This block can be initialized by the firmware if the card embedded a ROM MIDI sound bank. 
To modify a MIDI Mapping Table in a ROM block, the user should relocate the MIDI mapping table in a free block and release the old block as a system reserved block.
To relocate the MIDI mapping table see MMT block relocation.

MIDI Mapping Table Extension 0085H / 0005H
This block provides an extension for MIDI Mapping table. It avoid to relocated the MMT and the MIDI Mapping table when inserting a new instrument in the MIDI mapping table.
The MIDI Mapping table Extension block is created and update by the user (512 word is recommended for initialization size)

Control Table 0006H
This block is Ram only. It is initialize and update by firmware and read only by user.
It can’t be modified or relocate by user.

MIDI Sound Parameter 0000 iiix - 90H / 10H
This is the sound bank Parameter block.
« iii » is the sound bank ID (0 to 7), « iii » points to the sound bank definition 
«  x » is parameter block NB, each sound bank parameter can be mapped at best in two 64K word blocks.

MIDI Sound PCM dddd iii0 - 91H / 11H
This is the sound bank PCM block.
« iii » is the sound bank ID (0 to 7), « iii » points to the sound bank definition 
«  dddd » is PCM block NB, each sound bank PCM can be split in blocks.

Streaming Wave Buffer xx20H
This block is RAM only.
xx is the streaming wave NB 0-7 for play, 8 for record.

Static Wave Buffer xx30H
This block is RAM only.
xx is the static wave NB (play only).

## Sound bank definition:

| Nb | Type | Content |
| --- | --- | --- |
| 8 | Char | MIDI sound bank name |
| 1 | word | MIDI sound bank priority level |

Sound bank definition can be initialized by firmware in case of ROM MIDI sound bank. Sound bank definition can be modified by user.

MIDI Sound bank name
8 ASCII character corresponding to the MIDI sound bank file name *.94b 
Unused MIDI sound bank are filled with 8*00H.

MIDI Sound priority level
Range 0 to 127

# Midi Mapping Table

Although each bank has its own MIDI mapping a single MIDI mapping table is used by the 9407 board.
This single MIDI mapping table is created/updated  by SBE and used by 9407 board software.
Sound bank exchanges with 9407 board always begin with downloading of MIDI mapping from 9407 board memory, then SBE updates MIDI mapping table and uploads the table back to 9407 board memory.

MIDI mapping table describes current MIDI mapping but also includes additional information to recover instruments that were replaced with an higher priority sound bank.

Those two kinds of sounds are referred in the instrument mapping table as

Current instrument/Drumset

Backup instrument/Drumset

MIDI mapping table:

| Nb | Type | Content |
| --- | --- | --- |
| 1 | word | total table length |
| 16 | word | sound bank page |
| 128 | word | 16bits pointer to instrument program table |
|  | variable | Drumset table |
|  | word | FFFF end of block |

Sound bank page:

Sound bank parameter maximum size is 128Kword. If block size is superior to 64K, it should be split into two 64K pages.

SBE can manage up to 8 sound banks.

The 16 word sound bank page table actually holds 8 sound bank with 2 page parameter each. Even sound bank page refer to the first parameter block and odd to the second parameter block.

Instrument program table:

| Nb | Type | Content |
| --- | --- | --- |
| n | long | current instrument definition |
|  | word | FFFF end of block |
| p | long | backup instrument definition |
|  | word | FFFF end of block |

Instrument definition:

| Nb | Type | Content |
| --- | --- | --- |
|  | byte | variation number |
|  | byte | sound bank number, actually sound bank nb *2 + 0 (for 1st param block) / 1 (for 2nb param block) |
|  | word | 16bit pointer to instrument parameter |

Drumset table:

| Nb | Type | Content |
| --- | --- | --- |
| n | 3*word | current Drumset definition |
|  | word | FFFF end of block |
| p | 3*word | backup Drumset definition |
|  | word | FFFF end of block |

Drumset definition:

| Nb | Type | Content |
| --- | --- | --- |
|  | word | program number |
|  | byte | variation number |
|  | byte | sound bank number (up to 8 bank) |
|  | word | 16bit pointer to Drumset parameter |

# CONTROL TABLE

The control table stores the current values of all the device parameters that can be read back by client applications.

It is a 128 bytes long table, the address corresponds to the device parameter control number (see Firmware API), Only a few control number among the 128 possible values are defined and only a few of these defined parameters can be read back.

| Ctrl # | CONTROL NAME | Action |
| --- | --- | --- |
| 07H | MASTER_VOL | Master volume |
| 08H | REC_MODE | Select record mode |
| 10H | EQ_LBL | Equalizer low band left |
| 11H | EQ_MLBL | Equalizer med low band left |
| 12H | EQ_MHBL | Equalizer med high band left |
| 13H | EQ_HBL | Equalizer high band left |
| 14H | EQ_LBR | Equalizer low band right |
| 15H | EQ_MLBR | Equalizer med low band right |
| 16H | EQ_MHBR | Equalizer med high band right |
| 17H | EQ_HBR | Equalizer high band right |
| 18H | EQF_LB | Equalizer low band frequency |
| 19H | EQF_MLB | Equalizer med low band frequency |
| 1AH | EQF_MHB | Equalizer med high band frequency |
| 1BH | EQF_HB | Equalizer high band frequency |
| 20H | AUD_SEL | Audio1/2 input select (1) |
| 21H | AUD_GAINL | Audio Left input gain (1) |
| 22H | AUD_GAINR | Audio Right input gain (1) |
| 25H | GMREV_SEND | General Midi Reverb Send |
| 26H | GMCHR_SEND | General Midi Chorus Send |
| 27H | AUDREV_SEND | Audio Reverb Send |
| 28H | ECH_LEV | Echo level applied on audio in (2) |
| 29H | ECH_TIM | Echo time applied on audio in (2) |
| 2AH | ECH_FEED | Echo feedback applied on audio in (2) |
| 30H | SUR_VOL | Surround effect volume |
| 31H | SUR_DEL | Surround effect delay |
| 32H | SUR_INP | Input mono/stereo select for surround |
| 33H | SUR_24 | 2 or 4 speakers output select for surround |
| 34H | AUDL_VOL | Left Channel Audio in volume (2) |
| 35H | AUDR_VOL | Right Channel Audio in volume (2) |
| 36H | AUDL_PAN | Left Channel Audio in pan (2) |
| 37H | AUDR_PAN | Right Channel Audio in pan (2) |
| 38H | GM_VOL | General Midi volume |
| 39H | GM_PAN | General Midi pan |
| 3AH | REV_VOL | Reverb general volume |
| 3BH | CHR_VOL | Chorus general volume |
| 3FH | UART_MOD | Switch to UART mode |
| 61h | MOD_ASS | MOD player audio output assignment (3) |
| 62h | GM_POST | Post effects applied on general midi (4) |
| 63h | WAVE_POST | Post effects applied on wave (4) |
| 64h | MOD_POST | Post effects applied on MOD player (4) |
| 65h | AUDECH_POST | Post effects applied on Audio in and echo(4) |
| 66h | EFF_POST | Post effects applied on Reverb-chorus (4) |
| 68H | ECH_ONOFF | Echo On/Off (3) |
| 69H | REV_TYPE | Reverb program select |
| 6AH | CHR_TYPE | Chorus program select |
| 6BH | EQU_TYPE | Equalizer type (3) |
| 6CH | REV_ONOFF | Reverb On/Off (3) |
| 6DH | CHR_ONOFF | Chorus On/Off (3) |
| 6EH | SUR_ONOFF | Surround On/Off (3) |
| 6FH | AUD_ONOFF | Audio On/Off (3) |
| 72H | POLY_64 | Enable 64 voice polyphony (3) |
| 74H | CHR_DEL | Chorus delay |
| 75H | CHR_FEED | Chorus feedback |
| 76H | CHR_RATE | Chorus rate |
| 77H | CHR_DEPTH | Chorus depth |
| 78H | REV_TIME | Reverb time |
| 79H | REV_FEED | Reverb feedback |

Chapter 4

MIDI Implementation

| MIDI MESSAGE | HEX CODE | DESCRIPTION | COMPATIBI-LITY |
| --- | --- | --- | --- |
| NOTE ON | 9nH kk vv | Midi channel n(0-15) note ON #kk(1-127), velocity vv(1-127). vv=0 means NOTE OFF | MIDI |
| NOTE OFF | 8nH kk vv | Midi channel n(0-15) note OFF #kk(1-127), vv is don’t care. | MIDI |
| PITCH BEND | EnH bl bh | Pitch bend as specified by bh¦bl (14 bits) Maximum swing is +/- 1 tone (power-up). Can be changed using « pitch bend sensitivity ». Center position is 00H 40H. | GM |
| PROGRAM CHANGE | CnH pp | Program (patch) change. Specific action on channel 10 (n=9) : select drumset. Refer to sounds / drumset list. Drumsets can be assigned to other channels (see SYSEX MIDI channel to part assign and part to rhythm allocation) | GM/GS |
| CHANNEL AFTERTOUCH | DnH vv | vv pressure value. Effect set using Sys. Ex. 40H 2nH 20H-26H | MIDI |
| MIDI RESET | FFH | Reset to power-up condition |  |
| CTRL 00 | BnH 00H cc | Bank select : Refer to sounds list. No action on drumset. cc=64 reserved for dream sound editor | GS/ DREAM |
| CTRL 01 | BnH 01H cc | Modulation wheel. Rate and maximum depth can be set using SYSEX | MIDI |
| CTRL 05 | BnH 05H cc | Portamento time. | MIDI |
| CTRL 06 | BnH 06H cc | Data entry : provides data to RPN and NRPN | MIDI |
| CTRL 07 | BnH 07H cc | Volume (default=100) | MIDI |
| CTRL 10 | BnH 0AH cc | Pan (default=64 center) | MIDI |
| CTRL 11 | BnH 0BH cc | Expression (default=127) | MIDI/GM |
| CTRL 64 | BnH 40H cc | Sustain (damper) pedal | MIDI |
| CTRL 65 | BnH 41H cc | Portamento ON/OFF | MIDI |
| CTRL 66 | BnH 42H cc | Sostenuto pedal | MIDI |
| CTRL 67 | BnH 43H cc | Soft pedal | MIDI |
| CTRL 80 | BnH 50H vv | Reverb program vv=00H to 07H (default 04H) 00H : Room1 01H : Room2 02H : Room3 03H : Hall1 04H : Hall2 05H : Plate 06H : Delay 07H : Pan delay | DREAM |
| CTRL 81 | BnH 51H vv | Chorus program vv=00H to 07H (default 02H) 00H : Chorus1 01H : Chorus2 02H : Chorus3 03H : Chorus4 04H : Feedback 05H : Flanger 06H : Short delay 07H : FB delay | DREAM |
| CTRL 91 | BnH 5BH vv | Reverb send level vv=00H to 7FH | GS |
| CTRL 93 | BnH 5DH vv | Chorus send level vv=00H to 7FH | GS |
| CTRL 120 | BnH 78H 00H | All sound off (abrupt stop of sound on channel n) | MIDI |
| CTRL 121 | BnH 79H 00H | Reset all controllers | MIDI |
| CTRL 123 | BnH 7BH 00H | All notes off | MIDI |
| CTRL 126 | BnH 7EH 00H | Mono on | MIDI |
| CTRL 127 | BnH 7FH 00H | Poly on (default power-up) | MIDI |
| CTRL CC1 | BnH ccH vvH | Assignable Controller 1. cc=Controller number (0-5Fh), vv=Control value (0-7Fh). Control number (ccH) can be set on CC1 CONTROLLER NUMBER (Sys. Ex 40 1x 1F). The resulting effect is determined by CC1 controller function (Sys.Ex. 40 2x 40-4A) | GS |
| CTRL CC2 | BnH ccH vvH | Assignable Controller 2. cc=Controller number (00h-5Fh), vv=control value (0-7Fh). Control number can be set on CC2 CONTROLLER NUMBER (Sys.Ex. 40 1x 20). The resulting effect is determined by CC2 controller function (Sys.Ex.40 2x 50-5A). |  |
| RPN 0000H | BnH 65H 00H 64H 00H 06H vv | Pitch bend sensitivity in semitones (default=2) | MIDI/GM |
| RPN 0001H | BnH 65H 00H 64H 01H 06H vv | Fine tuning in cents (vv=00 -100, vv=40H 0, vv=7FH +100 | MIDI |
| RPN 0002H | BnH 65H 00H 64H 02H 06H vv | Coarse tuning in half-tones (vv=00 -64, vv=40H 0, vv=7FH +64 | MIDI |
| NRPN 0108H | BnH 63H 01H 62H 08H 06H vv | Vibrate rate modify (vv=40H -> no modif) | GS |
| NRPN 0109H | BnH 63H 01H 62H 09H 06H vv | Vibrate depth modify (vv=40H -> no modif) | GS |
| NRPN 010AH | BnN 63H 01H 62H 0AH 06H vv | Vibrate delay modify (vv=40H -> no modif) | GS |
| NRPN 0120H | Bnh 63H 01H 62H 20H 06H vv | TVF cutoff freq modify(vv=40H -> no modif) | GS |
| NRPN 0121H | BnH 63H 01H 62H 21H 06H vv | TVF resonance modify (vv=40H -> no modif) | GS |
| NRPN 0163H | Bnh 63H 01H 62H 63H 06H vv | Env. attack time modify(vv=40H ->no modif) | GS |
| NRPN 0164H | BnH 63H 01H 62H 64H 06H vv | Env. decay time modify(vv=40H -> no modif) | GS |
| NRPN 0166H | BnH 63H 01H 62H 66H 06H vv | Env. release time modif(vv=40H ->no modif) | GS |
| NRPN 18rrH | BnH 63H 18H 62H rr 06H vv | Pitch coarse of drum instr. note rr in semitones (vv=40H -> no modif) | GS |
| NRPN 1ArrH | BnH 63H 1AH 62H rr 06H vv | Level of drum instrument note rr (vv=00 to 7FH) | GS |
| NRPN 1CrrH | BnH 63H 1CH 62H rr 06H vv | Pan of drum instrument note rr (40H = middle) | GS |
| NRPN 1DrrH | BnH 63H 1DH 62H rr 06H vv | Reverb send level of drum instrument note rr (vv=00 to 7FH) | GS |
| NRPN 1ErrH | BnH 63H 1EH 62H rr 06H vv | Chorus send level of drum instrument note rr (vv=00 to 7FH) | GS |
| NRPN 37xxH | BnH 63H 37H 62H xx 06H vv | Special SAM9407 features controls (see below) | DREAM |
| Standard Sysex | F0H 7EH 7FH 09H 01H F7H | General MIDI reset | GM |
| Standard Sysex | F0H 7FH 7FH 04H 01H 00H ll F7H | Master volume (ll=0 to 127, default 127) | GM |
| ROLAND SYSEX | F0H 41H 00H 42H 12H 40H 00H 00H dd dd dd dd xx F7H | Master tune (default dd= 00H 04H 00H 00H) -100.0 to +100.0 cents. Nibblized data should be used (always four bytes). For example, to tune to +100.0 cents, sent data should be 00H 07H 0EH 08H | GS |
| ROLAND SYSEX | F0H 41H 00H 42H 12H 40H 00H 04H vv xx F7H | Master volume (default vv=7FH) | GS |
| ROLAND SYSEX | F0H 41H 00H 42H 12H 40H 00H 05H vv xx F7H | Master key-shift (default vv=40H, no transpose) | GS |
| ROLAND SYSEX | F0H 41H 00H 42H 12H 40H 00H 06H vv xx F7H | Master pan (default vv=40H, center) |  |
| ROLAND SYSEX | F0H 41H 00H 42H 12H 40H 00H 7FH 00H xx F7H | GS reset | GS |
| ROLAND SYSEX | F0H 41H 00H 42H 12H 40H 01H 30H vv xx F7H | Reverb type (vv=0 to 7), default = 04H 00H : Room1 01H : Room2 02H : Room3 03H : Hall1 04H : Hall2 05H : Plate 06H : Delay 07H : Pan delay | GS |
| ROLAND SYSEX | F0H 41H 00H 42H 12H 40H 01H 31H vv xx F7H | Reverb character, default 04H | GS |
| ROLAND SYSEX | F0H 41H 00H 42H 12H 40H 01H 33H vv xx F7H | Reverb master level, default = 64 | GS |
| ROLAND SYSEX | F0H 41H 00H 42H 12H 40H 01H 34H vv xx F7H | Reverb time | GS |
| ROLAND SYSEX | F0H 41H 00H 42H 12H 40H 01H 35H vv xx F7H | Reverb delay feedback. Only if reverb number=6 or 7 (delays) | GS |
| ROLAND SYSEX | F0H 41H 00H 42H 12H 40H 01H 38H vv xx F7H | Chorus type (vv=0 to 7), default = 02H 00H : Chorus1 01H : Chorus2 02H : Chorus3 03H : Chorus4 04H : Feedback 05H : Flanger 06H : Short delay 07H : FB delay | GS |
| ROLAND SYSEX | F0H 41H 00H 42H 12H 40H 01H 3AH vv xx F7H | Chorus master level, default = 64 | GS |
| ROLAND SYSEX | F0H 41H 00H 42H 12H 40H 01H 3BH vv xx F7H | Chorus feedback | GS |
| ROLAND SYSEX | F0H 41H 00H 42H 12H 40H 01H 3CH vv xx F7H | Chorus delay | GS |
| ROLAND SYSEX | F0H 41H 00H 42H 12H 40H 01H 3DH vv xx F7H | Chorus rate | GS |
| ROLAND SYSEX | F0H 41H 00H 42H 12H 40H 01H 3EH vv xx F7H | Chorus depth | GS |
| ROLAND SYSEX | F0H 41H 00H 42H 12H 40H 1pH 02H nn xx F7H | MIDI channel to part assign, p is part (0 to 15), nn is MIDI channel (0 to 15, 16=OFF). This SYSEX allows to assign several parts to a single MIDI channel or to mute a part. Default assignment : part MIDI channel 0 9 (DRUMS) 1-9 0-8 10-15 10-15 | GS |
| ROLAND SYSEX | F0H 41H 00H 42H 12H 40H 1pH 15H vv xx F7H | Part to rhythm allocation, p is part (0 to 15), vv is 00 (sound part) or 01 (rhythm part). This SYSEX allows a part to play sound or drumset. There is no limitation of the number of parts playing drumset. Default assignment : part 0 plays drums (default MIDI channel 9) all other parts play sound. | GS |
| ROLAND SYSEX | F0H 41H 00H 42H 12H 40H 1nH 40H v1 v2 ... v12 xx F7H | Scale tuning, n is MIDI channel (0 to 15), v1 to v12 are 12 semi-tones tuning values (C, C#, D, ... A#, B), in the range -64 (00H) 0 (40H) +63(7FH) cents. This SYSEX allows non chromatic tuning of the musical scale on a given MIDI channel. Default v1, v2, ... ,v12 = 40H, 40H,...,40H (chromatic tuning). Scale tuning has no effect if the part is assigned to a rhythm channel or if the sound played is not of chromatic type. | GS |
| ROLAND SYSEX | F0H 41H 00H 42H 12H 40H 1nH 1AH vv xx F7H | Velocity slope from 00H to 7FH (default = 40H) | GS |
| ROLAND SYSEX | F0H 41H 00H 42H 12H 40H 1nH 1BH vv xx F7H | Velocity offset from 00H to 7FH (default = 40H) | GS |
| ROLAND SYSEX | F0H 41H 00H 42H 12H 40H 1nH 1FH vv xx F7H | CC1 Controller number (00-5FH) (default = 10H) | GS |
| ROLAND SYSEX | F0H 41H 00H 42H 12H 40H 1nH 20H vv xx F7H | CC2 Controller number (00-5FH) (default = 11H) | GS |
| ROLAND SYSEX | F0H 41H 00H 42H 12H 40H 2nH 00H vv xx F7H | Mod pitch control (-24,+24 semitone) (default = 40H) | GS |
| ROLAND SYSEX | F0H 41H 00H 42H 12H 40H 2nH 01H vv xx F7H | Mod tvf cutoff control (default = 40H) | GS |
| ROLAND SYSEX | F0H 41H 00H 42H 12H 40H 2nH 02H vv xx F7H | Mod Amplitude control (-100%-+100%) (default=40H) | GS |
| ROLAND SYSEX | F0H 41H 00H 42H 12H 40H 2nH 03H vv xx F7H | Mod lfo1 rate control (default = 40H). n is don’t care. Rate is common on all channels | GS |
| ROLAND SYSEX | F0H 41H 00H 42H 12H 40H 2nH 04H vv xx F7H | Mod lfo1 pitch depth (0-600 cents) (default=0AH) | GS |
| ROLAND SYSEX | F0H 41H 00H 42H 12H 40H 2nH 05H vv xx F7H | Mod lfo1 tvf depth (default = 0H) | GS |
| ROLAND SYSEX | F0H 41H 00H 42H 12H 40H 2nH 06H vv xx F7H | Mod lfo1 tva depth (0-100%) (default = 0H) | GS |
| ROLAND SYSEX | F0H 41H 00H 42H 12H 40H 2nH 10H vv xx F7H | Bend pitch control (-24,+24 semitone) (default = 42H) | GS |
| ROLAND SYSEX | F0H 41H 00H 42H 12H 40H 2nH 11H vv xx F7H | Bend tvf cutoff control (default = 40H) | GS |
| ROLAND SYSEX | F0H 41H 00H 42H 12H 40H 2nH 12H vv xx F7H | Bend Amplitude control (-100%-+100%) (default=40H) | GS |
| ROLAND SYSEX | F0H 41H 00H 42H 12H 40H 2nH 14H vv xx F7H | Bend lfo1 pitch depth (0-600 cents) (default=0AH) | GS |
| ROLAND SYSEX | F0H 41H 00H 42H 12H 40H 2nH 15H vv xx F7H | Bend lfo1 tvf depth (default = 0H) | GS |
| ROLAND SYSEX | F0H 41H 00H 42H 12H 40H 2nH 16H vv xx F7H | Bend lfo1 tva depth (0-100%) (default = 0H) | GS |
| ROLAND SYSEX | F0H 41H 00H 42H 12H 40H 2nH 20H vv xx F7H | CAF pitch control (-24,+24 semitone) (default = 40H) | GS |
| ROLAND SYSEX | F0H 41H 00H 42H 12H 40H 2nH 21H vv xx F7H | CAF tvf cutoff control (default = 40H) | GS |
| ROLAND SYSEX | F0H 41H 00H 42H 12H 40H 2nH 22H vv xx F7H | CAF Amplitude control (-100%-+100%) (default=40H) | GS |
| ROLAND SYSEX | F0H 41H 00H 42H 12H 40H 2nH 24H vv xx F7H | CAF lfo1 pitch depth (0-600 cents) (default=0AH) | GS |
| ROLAND SYSEX | F0H 41H 00H 42H 12H 40H 2nH 25H vv xx F7H | CAF lfo1 tvf depth (default = 0H) | GS |
| ROLAND SYSEX | F0H 41H 00H 42H 12H 40H 2nH 26H vv xx F7H | CAF lfo1 tva depth (0-100%) (default = 0H) | GS |
| ROLAND SYSEX | F0H 41H 00H 42H 12H 40H 2nH 40H vv xx F7H | CC1 pitch control (-24,+24 semitone) (default = 40H) | GS |
| ROLAND SYSEX | F0H 41H 00H 42H 12H 40H 2nH 41H vv xx F7H | CC1 tvf cutoff control (default = 40H) | GS |
| ROLAND SYSEX | F0H 41H 00H 42H 12H 40H 2nH 42H vv xx F7H | CC1 Amplitude control (-100%-+100%) (default=40H) | GS |
| ROLAND SYSEX | F0H 41H 00H 42H 12H 40H 2nH 44H vv xx F7H | CC1 lfo1 pitch depth (0-600 cents) (default=0AH) | GS |
| ROLAND SYSEX | F0H 41H 00H 42H 12H 40H 2nH 45H vv xx F7H | CC1 lfo1 tvf depth (default = 0H) | GS |
| ROLAND SYSEX | F0H 41H 00H 42H 12H 40H 2nH 46H vv xx F7H | CC1 lfo1 tva depth (0-100%) (default = 0H) | GS |
| ROLAND SYSEX | F0H 41H 00H 42H 12H 40H 2nH 50H vv xx F7H | CC2 pitch control (-24,+24 semitone) (default = 40H) | GS |
| ROLAND SYSEX | F0H 41H 00H 42H 12H 40H 2nH 51H vv xx F7H | CC2 tvf cutoff control (default = 40H) | GS |
| ROLAND SYSEX | F0H 41H 00H 42H 12H 40H 2nH 52H vv xx F7H | CC2 Amplitude control (-100%-+100%) (default=40H) | GS |
| ROLAND SYSEX | F0H 41H 00H 42H 12H 40H 2nH 54H vv xx F7H | CC2 lfo1 pitch depth (0-600 cents) (default=0AH) | GS |
| ROLAND SYSEX | F0H 41H 00H 42H 12H 40H 2nH 55H vv xx F7H | CC2 lfo1 tvf depth (default = 0H) | GS |
| ROLAND SYSEX | F0H 41H 00H 42H 12H 40H 2nH 56H vv xx F7H | CC2 lfo1 tva depth (0-100%) (default = 0H) | GS |

notes :

NRPN sending method : CTRL#99=high byte, CTRL#98=low byte, CTRL#6=vv

Example : NRPN 0108H = 40H -> CTRL#99=1, CTRL#98=8, CTRL#6=64

x or xx means « don’t care »

DREAM SPECIAL NRPN CONTROLS

The various features of the SAM9407 are controlled by NRPN MIDI messages as follows :

| NRPN # (High¦Low) | Description | Power-up default |
| --- | --- | --- |
| 3700H | Equalizer Low band (bass) 0=-12dB, 40H=0dB, 7FH=+12dB | 60H (+6dB) |
| 3701H | Equalizer Med Low band 0=-12dB, 40H=0dB, 7FH=+12dB | 40H (0dB) |
| 3702H | Equalizer Med High band 0=-12dB, 40H=0dB, 7FH=+12dB | 40H (0dB) |
| 3703H | Equalizer High band (treble) 0=-12dB, 40H=0dB, 7FH=+12dB | 60H (+6dB) |
| 3708H | Equalizer Low cutoff freq 0=0Hz, 7FH=4.7 kHz | 0CH |
| 3709H | Equalizer Med Low cutoff freq 0=0Hz, 7FH=4.2 kHz | 1BH |
| 370AH | Equalizer Med High cutoff freq 0=0Hz, 7FH=4.2 kHz | 72H |
| 370BH | Equalizer High cutoff freq 0=0Hz, 7FH=18.75 kHz | 40H |
| 3710H | Input select 0=select microphones 7FH=select AUXL/ AUXR (note 1) | 00H (mikes) |
| 3711H | Mike1/AUXL input gain 0=0dB to 7FH=+22.5dB (note 1) | 00H (0dB) |
| 3712H | Mike2/AUXR input gain 0=0dB to 7FH=+22.5dB (note 1) | 00H (0dB) |
| 3720H | Surround effect volume 0= no effect, 7FH= maximum effect | 00H |
| 3724H | Mike1/AUXL volume 0 to 7FH | 40H |
| 3725H | Mike2/AUXR volume 0 to 7FH | 40H |
| 3726H | Mike 1/AUXL pan 0=hard left, 40H=center, 7FH=hard right | 00H (left) |
| 3727H | Mike 2/AUXR pan 0=hard left, 40H=center, 7FH=hard right | 7FH (right) |
| 3728H | Mike/AUX echo level 0 to 7FH | 40H |
| 3729H | Mike/AUX echo time 0=shortest to 7FH=longest | 40H |
| 372AH | Mike/AUX echo feed-back 0=no feed back to 7FH=maximum feedback | 40H |
| 372CH | Surround effect delay 0=shortest to 7Fh=longest | 2H |
| 372DH | Surround effect input 0=stereo, 7Fh=mono | 0H |
| 372EH | Surround effect output mode 0=2 speaker mode, 7Fh=4 speaker mode | 0H |

note 1 : Aux inputs are available only with CS4216/CS4218 type DAC/ADC.