LxVox V1.0

(c)1998 Stefan Peichl, Heidelberg, Germany

http://hplx.pgdn.de

LxVox is a very small (1700 bytes) WAV file player running in

64KB of memory on any x86 machine. It plays 8bit mono WAV files

of any size and supports any sampling rate. LxVox may also be

used as a 'plug-in' with D&A's Internet browner WWW/LX.

WHAT'S IN YOUR LXVOX.ZIP FILE

LXVOX.COM      WAV player

LXVOX.DOC      This file

WAV2COM.COM    Converts WAV files into executable COM files

DEMO.WAV       2000Hz sample file

HOW TO USE LXVOX

If called without parameters, LxVox searches the current

directory for all files with the extension WAV. If files are

found, they are played. If no files are found, LxVox exits

and displays a small help screen.

To play back a specific WAV file, use its full or relative

pathname as parameter for LxVox. The wildcards * and ? are

allowed in the filename to play back a group of sounds with

similar names. Available command line switches are:

/H    Help screen display

/L    Loop endless through files

/N    No file info output

/S    Scan all subdirectories too

/Vx   Volume set to x=1-3 (HP200LX only)

While LxVox is running, three keys are available:

ESC   Exit LxVox

SPACE Suspend replay, press any key to continue

CR    Skip full replay (useful in a series of files)

NOTE: During replay of a WAV file, the interrupts are turned

off most of the time, which makes it hard to recognize

a keystroke. Just hold down the key until it reacts.

Use the WAV2COM utility to create a COM file from a WAV file.

WAV2COM takes as parameter the complete name of a WAV file and

creates a COM file of the same name. The wildcards ? and *

are allowed in the filename to convert a group of files with

a single command. The advantage of COM files is, that they

can be compressed using a standard file compression tool like

DIET or PKLITE. Compression of converted WAV files may be up

to 50%, because WAV itself is an uncompressed format.

NOTE: WAV2COM only converts up to 63KB of a WAV file. If your

WAV file is bigger than that, the rest is left behind.

Indeed your WAV file remains untouched.

-EXAMPLES-

LXVOX         ;plays all WAVs of the current directory

LXVOX D:/S    ;plays all WAVs of a CDROM (drive D:)

LXVOX ?.WAV/L ;loops endless trough all single letter WAVs

WAV2COM       ;displays the WAV2COM help screen

WAV2COM *.WAV ;converts all WAVs to COMs in the current dir.

LXVOX, HV AND VOICEMAIL

You may use LxVox as a HV 'plug-in' in order to listen to WAV

files on the Internet. Let's say you keep LxVox in your C:\BIN

directory. To make LxVox available within HV, you have to add

two lines to your HV.CFG file. Put into the section [Viewers]:

WAV=C:\BIN\LXVOX %s

and put into the section [MIME]:

sound/wave=WAV

Whenever you visit an Internet site containing/pointing to

a WAV file, just click on it and HV launches LxVox to play

that file.

You may indeed also use LxVox to replay a voicemail which you

received in the WAV format.

TECHNICAL

The poor sound quality of a WAV file on the Palmtop is due to

the unshielded speaker of the Palmtop. On desktops, the high

frequency noise is better filtered through the case.

LxVox interprets sampling rates below 8000 Hz as 'delta

modulated', which means, that every bit represents one sound,

whereas at sampling rates above 8000Hz, every byte represents

one sound. I implemented this feature, because I never saw a

commercial software, recording at sample rates lower than

11000Hz. However simple input devices are often designed to

acquire delta modulated data. The famous SYNTEX project from

c't 8-1988 (a german computer magazine) described such a

hardware/software solution connected to the parallel port.

Recording quality was reasonable even at rates of 16000Hz

(resulting in 16000/8=2000Hz if looked at bytes). In other

words: A 100KB file may hold 1 minute of recorded voice.

See DEMO.WAV for a small, low quality example recorded with

SYNTEX on a desktop at 16000Hz. If you accept this quality

for internet telephone, a 10 seconds sentence could be sent

out in a 20KB file. A normal 10 seconds WAV file recorded

without delta modulation at 22000Hz would have 220KB. This

is 11 times larger, indeed at better quality.

If you compare a normal WAV file with a 256 color image,

then a delta modulated WAV file compares to a B&W image.

Same as with pictures, delta modulated WAV files are smaller

at the cost of information. There is no compression involved.

At recording time, a comparator only checks, if the actual

data is lower or higher than the last one. Lower results

in bit=0, higher in bit=1. This can be done with simple

hardware. If we ever see an input device for the palmtop,

it would be most likely connected to the serial port and

use the delta modulation method. Put a WAV header in front

of the data, and you are ready to listen to it using LxVox.

There is a palmtop voice recorder called 'MIKE' by Haruya Mori.

However I'm not good in hardware and unable to built it myself.

If you are interested in details, search for VOICE.ZIP under:

http://www.palmtop.net/super.html

WAV HEADER

This is the structure of the WAV header in assembler notation.

The header length is always 44 bytes and immediately followed

by the uncompressed data. This is an example for 8 bit mono:

;---RIFF CHUNK-------------------------

db  'RIFF'  ;identifier

dd  ?       ;overall length=file length-8 (fill in)

db  'WAVE'  ;RIFF chunk type

;---FMT CHUNK--------------------------

db  'fmt '  ;identifier

dd  16      ;fmt chunk length

dw  1       ;type

;0=stereo

;1=mono

dw  1       ;number of channels

dd  ?       ;sample rate in hertz         (fill in)

dd  ?       ;bytes per second=sample rate (fill in)

dw  1       ;bytes per sample

;1= 8 bit mono

;2= 8 bit stereo or 16 bit mono

;4=16 bit stereo

dw  8       ;bits per sample (8,12,16)

;---DATA CHUNK-------------------------

db  'data'  ;identifier

dd  ?       ;data length=file length-44   (fill in)

;---DATA-------------------------------

NOTE: All length entries as well as the file length itself

must be divisible by 4!

This information should enable you to write converters from

other sound formats into WAV, or to create your own WAV files

from a recording device.

DISCLAIMER AGREEMENT

LxVox is copyrighted freeware. Users of LxVox must accept this

disclaimer of warranty:

"LxVox is supplied as is. The author disclaims all warranties,

expressed or implied, including, without limitation, the

warranties of merchantability and of fitness for any purpose.

The author assumes no liability for damages, direct or

consequential, which may result from the use of LxVox."