These files can be added to you config.sys file on DOS 2.1 thru DOS 3.2

to add the ability to control the number of drives and other items for

any of these DOS versions. If you choose to use these you may also use

your regular version of FORMAT.COM.

CONFIGJR.SYS is a device driver that will allow you to configure your

PCjr quickly and easily. To use this driver put the file CONFIGJR.SYS

on a bootable DOS 2.1 - 3.2 disk. On the same disk put the following

line as the FIRST line of your CONFIG.SYS file. Then reboot.

device=configjr.sys

This will:

Install your expansion memory (256K to 640K).

By filling up your first 128K, DOS

will be forced to load your programs into your expanssion memory,

which will cause your programs to run faster.

Put your screen in 40 colunm, low intensity, black & white.

Options:  ( -d  -c  -t  -v  -k  -l )

If you have extra floppy drives use the -dn option, where n is 1 to 4.

The -c option will turn on the keyboard click.

The -tn option will set the video display mode, where n is:

Alpha Modes

0 for 40 X 25 B/W  (Default)

1 for 40 X 25 Color

2 for 80 X 25 B/W

3 for 80 X 25 Color

Graphics Modes			You can get some funny looking screens

4 for 320 X 200  4 Color	  when you boot up in a Graphics Mode

5 for 320 X 200  B/W 4 Shades	  because DOS expects Alpha Modes.

6 for 640 X 200  B/W 2 Shades

7 is not a valid mode

Extended Graphics Modes

8  for 160 X 200  16 Color

9  for 320 X 200  16 Color

10 for 640 X 200  4  Color

The -vn option will reserve video ram, where n is 4 to 96.

Normally 16K is set aside for video ram, you can either decrease

or increase this amount with this option. Using	-v4  will reserve

4K for video, giving you an extra 12K of user ram, but you must not

use any graphics modes. High values are used for reserving ram to

be used for high-res graphics (32K) or extra video pages.

The -k option will set Caps-Lock (Note: If you have a keyboard with

an indicator light for Caps-Lock, the light will not be turned on.)

The -l option will look for extra RAM, starting at 640K and will stop

at 740K, and give it to DOS. (Note: 640K is where the ROM stops testing

RAM and 740K is where video ram "logically" starts.)

This option will only perform a superficial check for RAM. You will need

to use a ramtest program to check for bad chips above 640K.

Example:	    device=configjr.sys -d3 -c -t2 -v4

Tell DOS you have 3 physical drives A:	B:  C:

Turn on keyboard click

Set the video display for 80 columns, black & white

Reserve 4K for video ram (text only)

As the system boots, a message prints out the number of disk drives you

asked for, and the total ram your system has.