TANDYSND 1.2

Tandy 1000 Sound Extensions

Written By:

Joseph A. Albrecht

Table of contents:

1 - Introduction To The TANDYSND Package

2 - BASIC And PASCAL Programming Interface

3 - ASSEMBLER Programming Interface

4 - Miscellaneous Details About The TANDYSND Package

CHAPTER 1 - Introduction

TANDYSND  is a  memory resident  sound program  for your  Tandy 1000/PCjr series

personal computer. It  provides you with a complete set  of sound functions that

can be executed  from your favorite programming language  such as compiled BASIC

and PASCAL. To use TANDYSND, just install  it when you start up your computer by

including the following line in your AUTOEXEC.BAT file:

TANDYSND

That's all there is to it. Since the  program is memory resident you can use any

of the program's sound functions anytime you want.

The following list are some of the functions TANDYSND provides you with:

Using the Tandy 1000/PCjr's 3 voice sound and noise capabilities

Making sounds and noises in background while your program executes

Changing the speed of the system timer for precise sound and noise duration

These  functions are  accessed by  passing  the  TANDYSND program  a set  values

through your computer's  CPU registers and executing an  INT instruction. Sounds

pretty easy, huh!  Well, you never have to be  that concerned with the mechanics

of the TANDYSND program. I have provided  you with a set of interfacing programs

for compiled BASIC and PASCAL. These interfaces act as a translator between your

program and the TANDYSND program. For example,  if you wanted to produce a noise

in TURBO PASCAL all you have to do is include a line like this in your program:

Noise(4, 8, 10);

The interface  takes the values  passed to  it,  places the values  into the CPU

registers, and  executes the INT  instruction without you  ever having to  worry

about  those  intricate  details.  The  following  is  list  of  interfaces  for

programming languages currently provided:

Microsoft QuickBASIC 4.5 and greater

SPECTRA Publishing PowerBASIC 2.0 and 2.1

Borland TURBO PASCAL 4.0 and greater

Chapter 2 describes the BASIC and PASCAL programming interfaces telling you what

each function does and how to use it  in your program. For those of you who have

a different programming  language and would  like to design  your own interface,

read chapter 3. This describes the  ASSEMBLY language interface for TANDYSND. It

gives all  the technical information necessary  for setting up an  interface for

any other programming language such as  C. Chapter 4 gives all the miscellaneous

details of TANDYSND.

CHAPTER 2 - BASIC And PASCAL Programming Interfaces

This chapter describes the BASIC and PASCAL programming interfaces for TANDYSND.

All the  functions are listed in  alphabetical order with a  description of what

the function  does, an explanation of  each parameter, and an  example of how to

use  it in  your program  for  both  BASIC and  PASCAL. Following  the functions

listing there is a section on the necessary steps to integrate the interface for

each language product into your program.

Functions summary for BASIC and PASCAL:

********************************************************************************

AudioOff

Turns all current sounds and noises off and disables background processing.

See the 'ExtSound' and 'Noise'  functions for more information on background

processing.

Parameters:

None

Examples:

BASIC

CALL AudioOff

PASCAL

AudioOff;

********************************************************************************

ExtSound(Frequency, Duration, Volume, Channel)

Generates a sound with the  specified frequency, duration, volume, and sound

channel.

This function works  exactly like the SOUND function  in the special version

of GW-BASIC that came with you Tandy  1000. Sounds can be made in background

if you enable sound buffering. This means that your program will continue to

execute while sounds  are being made. Each sound channel  has it's own sound

buffer. Each buffer can hold up to 64 entries.

Parameters:

Frequency is a number 110 - 32767:

This is  the tone to produce  in Hertz. Certain frequencies  correspond to

musical  notes.   The  following  chart  shows   the  frequency  for  each

corresponding note:

Note  Freq     Note  Freq     Note  Freq     Note  Freq

----  ----     ----  ----     ----  ----     ----  ----

C2      65     A3     220     F#5    740     C7    2489

C#2     69     A#3    233     G5     784     C#7   2637

D2      73     B3     247     G#5    831     D7    2794

D#2     78     C4     262     A5     880     D#7   2960

E2      82     C#4    277     A#5    932     E7    3136

F2      87     D4     294     B5     988     F7    3322

F#2     93     D#4    311     C6    1047     F#7   3520

G2      98     E4     330     C#6   1109     G7    3729

G#2    104     F4     349     D6    1175     G#7   3951

A2     110     F#4    370     D#6   1245     A7    4186

A#2    116     G4     392     E6    1319     A#7   4435

B2     123     G#4    415     F6    1397     B7    4699

C3     131     A4     440     F#6   1480     C8    4978

C#3    139     A#4    466     G6    1568     C#8   5274

D3     147     B4     494     G#6   1661     D8    5587

D#3    156     C5     523     A6    1760     F#8   5919

E3     165     C#5    554     A#6   1865     G8    6271

F3     175     D5     587     B6    1976     G#8   6645

F#3    185     D#5    622     D#8   2093     A8    7040

G3     196     E5     659     E8    2217     A#8   7459

G#3    208     F5     698     F8    2349     B8    7902

The Texas Instruments SN76496 sound chip  in the Tandy 1000 cannot produce

frequencies below  110 Hertz. If you  use the above chart  please remember

you cannot make notes below A2.

Duration is a number 0 - 65535:

This is the number of clock ticks the sound will last. See the 'FastTimer'

function for more information on clock ticks.

Volume is a number 0 - 15:

0 is the quietest.

15 is the loudest.

Sound channel is a number 0 - 2:

This is  the sound channel  to use when  producing the sound.  You can set

each sound channel to a different tone to make a 3-note chord.

Examples:

BASIC

CALL ExtSound(880, 8, 7, 0)

PASCAL

ExtSound(880, 8, 7, 0);

********************************************************************************

FastTimer(Speed)

Sets the speed of the 8253 timer chip.

The 8253 timer chip is the driving force behind the 'ExtSound', 'Noise', and

'Pause' functions. These functions rely on a certain period of time to last.

This period of time  is known as a clock tick. When  you first start-up your

computer  the  timer  chip  is  set  at  18.2  clock  ticks  per second. The

'FastTimer' function allows  you to change the speed to  a rate greater than

that. The  reason for this is  to increase the performance  of the functions

previously mentioned. The 'ExtSound' function is a good example. You can use

the 'ExtSound' function to produce notes  to make music. There is a limiting

factor in  that at a  rate of 18.2  clock ticks per  second notes that  last

1/19th or less their normal time cannot be produced. GW-BASIC sets the timer

chip to  run at 4  times it's normal  rate when it  uses the PLAY statement.

This allows it to make notes up to 1/64th their normal time. The 'FastTimer'

function allows you to run the timer  chip up to 256 times it's normal rate.

This gives  you almost unlimited  flexibility in writing  programs that make

sounds or  noises to last  exactly as long  as you want.  Another area where

this function can  be useful is in the  use of the 'Pause' function.  A more

precise time than 1/18th of a second can be used if you need it. I recommend

that you use values of 8 or less  if you are going to use this function only

for sounds or  noises and set it to  rates greater than that for  only short

periods of time in the use of  the 'Pause' function. This is because running

the 8253 timer  faster than it's normal setting will  cause your computer to

run slower. The higher the rate setting the slower your computer will run.

An extremely important  thing to remember about using  this function is that

you MUST use the 'ResetTimer' function  before you end your program. Failure

to do so can result in anything from your current time of day being wrong to

a locked up computer system. Avoid using  this function if you are using the

debugging facilities  of one of the  compilers. As soon as  a break point is

reached the program  is ended for all practical purposes.  The main thing to

remember is the 'ResetTimer' timer  function must be executed before control

is passed back to the operating system or the compiler you are working with.

Parameters:

Speed is a number 2 - 256:

This is the number  of times greater you want the 8253  timer chip to run.

A setting of 4 will make it run 4 times faster than before. This will make

clock ticks occur 72.8 times a second.

Examples:

BASIC

CALL FastTimer(8)

PASCAL

FastTimer(4);

********************************************************************************

GetTimer(TimerValue)

Returns  the current  speed of  the  8253  timer chip.  See the  'FastTimer'

function for more information on the 8253 timer chip.

Parameters:

TimerValue - Variable to store the speed of the 8253 timer chip.

Examples:

BASIC

CALL GetTimer(TimerValue%)

PASCAL

GetTimer(TimerValue);

********************************************************************************

Noise(Source, Volume, Duration)

Generates a periodic or white noise  sound with the specified volume for the

specified length of time.

This function works  exactly like the NOISE function  in the special version

of  GW-BASIC  that  came  with  your  Tandy  1000.  Test  it out to hear the

difference between the periodic and white noise sound effects. Noises can be

made  in background  if you  enable noise  buffering. This  means that  your

program  will continue  to execute  while noises  are being  made. The noise

buffer can hold up to 64 entries.

Parameters:

Source is a number 0-7:

0-3 are periodic noises

4-7 are white noises

Volume is a number 0-15:

0 is the quietest

15 is the loudest

Duration is a number 0 to 65535:

This is the number of clock ticks the noise will last. See the 'FastTimer'

function for more information on clock ticks.

Examples:

BASIC

CALL Noise(4, 8, 12)

PASCAL

Noise(4, 8, 12);

********************************************************************************

NoiseBufOff

Disables background processing of noises.

See the 'Noise' function for a description of background noise processing.

Parameters:

None

Examples:

BASIC

CALL NoiseBufOff

PASCAL

NoiseBuffOff

********************************************************************************

NoiseBufOn

Enables the processing of noises in background.

See the 'Noise' function for a description of background noise processing.

Parameters:

None

Examples:

BASIC

CALL NoiseBufOn

PASCAL

NoiseBufOn;

********************************************************************************

NoiseOff

Turns  off all  current noises   off and  disables background  processing of

noises.

See the 'Noise' function for a description of background noise processing.

Parameters:

None

Examples:

BASIC

CALL NoiseOff

PASCAL

NoiseOff

********************************************************************************

Pause(Delay)

Delays a program for the specified number of clock ticks.

Clock  ticks occur  at a  default rate  of 18.2  times a  second unless  the

'FastTimer'  function  is  used.  A  number  of  36  would  delay  a program

approximately 2  seconds at the default  clock tick rate. This  function has

the  advantage being  more precise  than a  high level  language delay  loop

because  it uses  an assembly  language interface.  Use it  when you need to

delay for a precise number of clock ticks.

Parameters:

Delay - Number of clock ticks to delay program. See the 'FastTimer' function

for more information on clock ticks.

Examples:

BASIC

CALL Pause(20)

PASCAL

Pause(20);

********************************************************************************

ResetTimer

Resets the 8253  timer chip to it's default speed  of 1. See the 'FastTimer'

function for a description of the 8253 timer chip.

Parameters:

None

Examples:

BASIC

CALL ResetTimer

PASCAL

ResetTimer;

********************************************************************************

SoundBufOff

Disables background processing of sounds.

See  the   'ExtSound'  function  for  a   description  of  background  sound

processing.

Parameters:

None

Examples:

BASIC

CALL SoundBufOff

PASCAL

SoundBufOff;

********************************************************************************

SoundBufOn

Enables background processing of sounds.

See  the  'ExtSound'  function  for   a  description  of  background  sound

processing.

Parameters:

None

Examples:

BASIC

CALL SoundBufOn

PASCAL

SoundOn;

********************************************************************************

SoundOff

Turns  off all  current sounds   off and  disables background  processing of

sounds.

See the 'ExtSound' function for a description of background sound processing.

Parameters:

None

Examples:

BASIC

CALL SoundOff

PASCAL

SoundOff;

********************************************************************************

SpeakerOff

Disables the exterior speaker.

Parameters:

None

Examples:

BASIC

CALL SpeakerOff

PASCAL

SpeakerOff;

********************************************************************************

SpeakerOn

Enables the exterior speaker.

Parameters:

None

Examples:

BASIC

CALL SpeakerOn

PASCAL

SpeakerOn;

********************************************************************************

QuickBASIC programming interface:

To use TANDYSND  with the QuickBASIC programming environment  simply type in the

following line:

QB /L SOUNDQBS

This will start QuickBASIC and load in  the interface for TANDYSND which is in

the  SOUNDQBS.QLB  quick  library.  The  next  step  to  use  TANDYSND in your

QuickBASIC programs is  to make sure that every program  you write begins with

the following line:

'$INCLUDE: 'SOUNDQBS.INC'

SOUNDQBS.INC  contains all  the subprogram  definitions necessary  to use  the

QuickBASIC  interface  for  TANYSND.  The  interface  for TANDYSND expects all

variables  passed to  it to  be of  type integer  so SOUNDQBS.INC contains the

following line:

DEFINT A-Z

This  makes sure  that all  variables used  in your  program are  of the  type

integer  by default.  If you  compile a  program from  outside the  QuickBASIC

programming environment, be sure to  specify the SOUNDQBS.LIB library when you

use the LINK program to create the EXE file.

PowerBASIC programming interfaces:

The first step  to using TANDYSND with PowerBASIC is  to compile the SOUND.PBS

file to  a unit. Load  the file into  PowerBASIC and compile  it to disk. This

only has to be done once. After you have done this initial step make sure that

every program you write starts with the following line:

$INCLUDE "SOUNDPBS.INC"

That's all there is to using TANDYSND with PowerBASIC.

The interface  for PowerBASIC is almost  entirely made up of  INLINE assembler

subprograms.  Be  careful  when  making  calls  to  TANDYSND  using because no

checking is done by the compiler to  validate the number of parameters or type

of variables  used when you make  the call.

TURBO PASCAL programming interface:

The first step to using TANDYSND with TURBO PASCAL is to compile the SOUND.PAS

file to a unit.  Load the file into TURBO PASCAL and  compile it to disk. This

only has  to be done  once. After you  have done this  initial step just  list

SOUND as one of the units in your program's USES statement like this:

USES

Sound;

That's all there is to it.

CHAPTER 3  - ASSEMBLER Programming Interface

This chapter gives  the technical details of the TANDYSND  package. It will give

you the  necessary information to write  an ASSEMBLY language interface  for the

programming language you  are working with. I'm going to  assume that anyone who

reads this chapter  already knows how to program in  80x86 ASSEMBLY language. If

you don't  then I suggest that  you skip this section.  It probably will confuse

you more than help.

An ASSEMBLY language  interface for TANDYSND is the key  to writing an interface

in another programming  language. The BASIC and PASCAL  interfaces provided with

the package are written in ASSEMBLER. TANDYSND is a memory resident program, all

requests  to it  are done  through  the  video BIOS  interrupt. The  AH register

contains the  hexadecimal value EE which  indicates to TANDYSND that  a function

request is about  to be made. The AL register  contains the function number. The

remaining registers  BX, CX, DX,  SI, DI, and  DS contain the  parameters of the

requested function. For  example, if you wanted to produce  a sound the function

call would look like this:

MOV     AX,0EE81H       ;Sound function

MOV     BX,10           ;Set volume to 10

MOV     CX,1            ;Use sound channel 1

MOV     SI,400          ;Set frequency to 400 HERTZ

MOV     DI,4            ;Make it last 4 clock ticks

INT     10H             ;Call TANDYSND

The next  section of this  chapter will list  each of the  TANDYSND functions in

numerical order. The format of each function listing is as follows:

Title

Function number

Description

Register entry values

Register exit values

Example

Comments

The title  is the name  I use for  the function in  the ASSEMBLY language source

file, it is just listed here for  reference purposes. The function number is the

the value for  the AL register, it is listed  in decimal format. The description

explains  what each  function does.  The register  entry values  are the  values

required to execute  the function. The exit values are  any register values that

the function might  return to your program. The example  is an ASSEMBLY language

code fragment showing a call to TANDYSND for that function. The comments section

is optional  and is included  only where I  feel extra clarification  is needed.

Entry values for the AX register for each function are in hexadecimal.

********************************************************************************

TITLE:

System

FUNCTION NUMBER:

128

DESCRIPTION:

This is  a blanket routine to  incorporate many system level  functions of the

TANDYSND package.

ENTRY VALUES:

AX = EE80

BX = Action

0 = Return residency status in AX

4 = Disable all current sound processing

5 = Enable sound buffer processing

6 = Disable sound buffer processing

7 = Disable all current noise processing

8 = Enable noise buffer processing

9 = Disable noise buffer processing

10 = Disable all current sound and noise processing

EXIT VALUES:

BX = 00 entry value

AX = DEAD indicates this package is resident

Here is a list of each subfunction's description:

BX = 00:

This will enable you to determine  if the TANDYSND program is installed

or not. If it is resident the AX register will contain the value 0DEADH

to indicate this fact. This is useful  to make sure the program you are

going to run was loaded after the TANDYSND program.

BX = 04:

This  turns all  current sound  processing off.  All current sounds are

turned  off,  the  sound  buffer  is  cleared,  and  sound buffering is

disabled.

BX = 05:

This turns sound buffering on allowing you to make sounds in background

while your program is executing.

BX = 06:

This turns  sound buffering off. The  current sound is left  alone, but

the sound buffer is cleared of any remain sounds to make.

BX = 07:

This  turns all  current noise  processing off.  All current noises are

turned  off,  the  noise  buffer  is  cleared,  and  noise buffering is

disabled.

BX = 08:

This turns noise buffering on allowing you to make noises in background

while your program is executing.

BX = 09:

This turns  noise buffering off. The  current noise is left  alone, but

the noise buffer is cleared of any remain noises to make.

BX = 10:

This  turns all  current noise  and sound  processing off.  All current

noises  and sounds  are turned  off, the  noise and  sound buffers  are

cleared, and noise and sound buffering is disabled.

COMMENTS:

See chapter 4 for the default values for the TANDYSND program.

********************************************************************************

TITLE:

Sound

FUNCTION NUMBER:

129

DESCRIPTION:

Generates a  sound with the  specified frequency, duration,  volume, and sound

channel.

ENTRY VALUES:

AX = EE81

BX = Volume (0-15)

0 is the quietest

15 is the loudest

CX = Sound channel (0-2)

Indicates which one of the three sound channels to use

SI = Frequency (110 - 32767)

DI = Duration (0-65535)

Indicates the number of clock ticks the sound will last

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE81H       ;Sound function

MOV     BX,10           ;Set the volume

MOV     CX,0            ;Set the sound channel

MOV     SI,800          ;Set the frequency

MOV     DI,5            ;Set the duration

INT     10H             ;Call TANDYSND

COMMENTS:

See the 'ExtSound' function in chapter 2 for more information.

********************************************************************************

TITLE:

Noise

FUNCTION NUMBER:

130

DESCRIPTION:

Generates a  periodic or white noise  sound using the specified  type, volume,

and duration.

ENTRY VALUES:

AX = EE82

BX = Type (0-7)

0-3 are periodic noises

4-7 are white noises

CX = Volume (0-15)

0 is the quietest

15 is the loudest

DX = Duration (0-65535)

This is the number of clock ticks the noise will last

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE82H       ;Noise function

MOV     BX,1            ;Set the type

MOV     CX,7            ;Set the volume

MOV     DX,3            ;Set the duration

INT     10H             ;Call TANDYSND

COMMENTS:

See the 'Noise' function in chapter 2 for more information.

********************************************************************************

TITLE:

Delay

FUNCTION NUMBER:

131

DESCRIPTION:

Delays a program the specified number of clock ticks.

ENTRY VALUES:

AX = EE83

BX = Duration (0-65535)

This is the number of clock ticks the delay will last

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE83H       ;Delay function

MOV     BX,8            ;Set the duration

INT     10H             ;Call TANDYSND

COMMENTS:

See the 'Pause' function in chapter 2 for more information.

********************************************************************************

TITLE:

FastTimer

FUNCTION NUMBER:

132

DESCRIPTION:

Sets the speed of the 8253 timer chip.

ENTRY VALUES:

AX = EE84

BX = Speed (2-256)

This is the number of times greater the chip should operate.

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE84H       ;Set 8253 timer chip speed function

MOV     BX,2            ;Set the speed to twice it's normal rate

INT     10H             ;Call TANDYSND

COMMENTS:

See the  'FastTimer' function in chapter  2 for more information  on the 8253

timer chip.

********************************************************************************

TITLE:

ResetTimer

FUNCTION NUMBER:

133

DESCRIPTION:

Resets the 8253 timer chip to it's normal speed which is a setting of 1.

ENTRY VALUES:

AX = EE85

EXIT VALUES:

None

EXAMPLE:

MOV     AX,0EE85H       ;Reset 8253 timer chip speed function

INT     10H             ;Call TANDYSND

COMMENTS:

See the  'FastTimer' function in chapter  2 for more information  on the 8253

timer chip.

********************************************************************************

TITLE:

GetTimer

FUNCTION NUMBER:

134

DESCRIPTION:

Returns the current speed of the 8253 timer chip.

ENTRY VALUES:

AX = EE86

EXIT VALUES:

BX = Current speed of the 8253 timer chip.

EXAMPLE:

MOV     AX,0EE86H       ;Get 8253 timer chip speed function

INT     10H             ;Call TANDYSND

MOV     [CurSpeed],BX   ;Store current 8253 timer chip speed

COMMENTS:

See the  'FastTimer' function in chapter  2 for more information  on the 8253

timer chip.

********************************************************************************

CHAPTER  4 - Miscellaneous Details About The TANDYSND Package

TANDYSND revision history:

1.0 -  Initial release of TANDYSND.

--------------------------------------------------------------------------------

The following are the default values for TANDYSND program:

Sound buffering:  OFF

Noise buffering:  OFF

--------------------------------------------------------------------------------

Some final items:

TANDYSND is  fully software compatible  with the Tandy  1000/PCjr graphics and

sound package, GRAFIX, that I have also written. This package is intended as a

subset of functions from GRAFIX for those of you who only need Tandy 1000/PCjr

3-voice sound and  noise support. This package is entirely  FREE and is placed

in the public domain. The only thing I  ask is that my name and copyright must

be include no matter what changes are made.