Memory Sidecar System Crashes

Reported by PCjr Users Group

EDITOR'S NOTE: The Metro PCjr Users Group found

recently that many of its members have been

clobbered with memory expansion system

failures. Although we have not heard that

our readers are having these problems, we

thought it would be useful to reprint the

following article from the "Metro PCjr

Journal" to help those who might run into

the same problem:

An epidemic of system crashes has broken

out among PCjr users with expanded memory

who have had their computers for a year or

more. Technicians think the problem stems

from dirt or tarnish building up on the

connector pins on the side of the system

unit where memory sidecars attach.

The problems take several forms. Some

users report their systems will count only

128K of memory on startup, them boot. Others

find their systems count 128K, report an

"ERROR A," then lock up. Sometimes the

computers will, after repreated trying, find

expanded memory and work properly, sometimes

not.

In some instances, the computer will crash

after counting all memory and booting, in

others it will report an "ERROR A" somewhere

between 128K and the end of the memory

count.

Some users who have removed their sidecars

have found their system will work again once

they replace it. Others have no luck after

removing and replacing their memory

boards.

++++++ Tecmar, IBM, Microsoft  ++++

The problem has been found with boards

from Tecmar, IBM and Microsoft. Lou Davidson

of MSC Inc. of Baltimore, which has upgraded

memory boards for many members of our group,

says custormers have sent him a number of

apparently faulty sidecars which he has been

unable to get to fail in tests in his

shop.

That is what led him and partner Dave

Wissman to suspect a problem with the

connector pins, which are a part of the

PCjr, not the memory boards. The boards have

a set of female connectors into which the

pins plug.

Davidson found a rash of similar

complaints from PCjr usrs on Compuserve's

PCjr SIG. He recommends periodic cleaning of

the connector pins using rubbing alcohol and

an artist's brush with bristles long enought

to reach into the pin housing, but not stiff

enough to risk bending the pins.