jrHotshot Installation

May Be Hard in Some jrs

The Metro PCjr Journal recently reviewed

the jrHotshot internal memory expansion unit

and pointed out that a few PCjrs have 8088

chips that are soldered in place, making it

difficult to install the jrHotshot.

While noting that jrHotshot is probably

the least expensive way to expand a 128K jr

to 640K, the Metro PCjr Journal also

observed that if some memory expansion has

already been added to your jr, you may find

it cheaper to have that unit upgraded.

As for the soldered 8088 chips in some

jrs, most of jrs have 8088 chips which can

be easily removed since they are placed in

sockets. However, if you are thinking of

purchasing a jrHotshot, which jr Newsletter

reviewed in the November issue, you should

check your jr to see if yours is one of the

few with a soldered 8088 chip.

To do this, first review the Diskette

Drive section of your PCjr Guide to

Operations. This will make you familiar with

the positioning of the controller card and

disk drive. To begin, disconnect the star pins on

the bottom of jr. Next, pry the top off your

jr and pull the disk controller card out of

its slot. Carefully lift the disk drive up

and check the chip located under where the

disk drive was located. It should say Intel

and have the number 8088 on it. Look closely

to determine whether or not this chip is

soldered in place.

Even if it is, you can still purchase a

jrHotshot and expand your memory. The 8088

chip can either be removed by you, if you

are very handy with electrical soldering

procedures. Or, most of the dealers handling

the jrHotshot will install it for you for an

additional fee.

jrHotshots are available

from Solid Rock Electronics, Owl Software,

The Peanut Gallery, and Computer Reset.

Check ads in this issue for further

details.