PCjr Questions and AnswerS

Q. I would like to take advantage of the added

storage capacity of 3.5" drives. Am I stuck with the 5

1/4" drives on my PCjr or is there a way I can modify

the jr to use 3.5" drives ?

A. This question has been asked more and more

frequently lately, perhaps because of IBM's introduction

of its new system, which uses 3.5" drives. They are

capable of storing twice as much data as 5 1/4"

diskettes and are also supposed to be more reliable. In

order for your PCjr to use 3.5" drives, you need a

driver controller and software to take advantage of the

80 tracks and 720K format on 3.5" drives. Ordinarily

the software is DOS 3.2 , which is designed to

work with these smaller, but more capacious diskettes.

However, PC Enterprises has put together not only a 3.5"

drive expansion chassis which can go on top of your

current drive, either as a third or a second drive, it

has also come up with software to use the drives with

DOS 2.1, allowing you to store up to 813K on each

diskette. For more information call 1-800-922-PCJR.

Q. I have tried using Freelance, by Lotus, on my

PCjr, which has 640K and two drives, but although I can

get it running and I can even use the Microsoft mouse

with it, I can't get the keyboard to work. I thought the

few incompatabilities between PCjrs and PCs had to do

with the video location and graphics, but what's the

problem with the keyboard?

A. There are some differences in the way your PCjr

handles keyboard information. These differences rarely

interfere with running programs written for PCs, but it

is possible that they could. The most likely problem is

that your PCjr's keyboard does not have a buffer in it

like the PC's keyboard has. On the PC, if the computer

isn't quite ready to accept the input from the keyboard,

the keyboard can "store" the keystrokes and feed them to

the computer when it is ready to accept them. PCjr

keyboards generate a non-maskable interrupt, which means

the ROM-BIOS (the input/output system that is programmed

into your jr's ROM) has to act upon the keyboard input

immediately. Also, due to a bug in the Input/Output

routines in the PCjr BIOS (the Read Only Memory system

that handles input from the keyboard among other

things), your computer can lock up under certain

circumstances, such as when you press or release a key

while the computer is writing to or reading from a

diskette. These lock-ups are very rare, but they can

occur. However, since we do not know for certain that

this is the cause of the problem when using Freelance,

we'll go on to the good news -- we have heard that loading

Superkey or possibly other keyboard programs, enables

you to run Freelance on a PCjr. Although we could

speculate on the reasons why this solves the problem, we

don't want to waste any more space on guesses and will

just leave you with the fact that this solution

apparently works.

Q. I use PC-Write for many different kinds of word

processing tasks and I would like to be able to

customize the key strokes so that I could type one or

two keys and have 20 or 30 keystrokes executed

automatically. Is this possible to do with PC-Write?

A. Yes, it's quite simple to do and it can be done

in any of the versions of PC-Write. We use that

capability all the time. In version 2.7 you type ED

ED.DEF (or the file name of another ED file if you have

set one up yourself). Then, in order to be able to use

the control key in combination with another key to carry

out any number of keystrokes, you type in the key you

want to use in combination with the control key,

followed by a colon, and then the combination of

keystrokes you want to be executed. If you want

letters, you put them in quotation marks. If you want

keyboard operations, such as Enter or backspace, etc.,

just put in the operation code number (there's a list

in the manual). For example, putting this line in your

ED.DEF file:

A:"Sincerely Yours,"

QRwill enable PC-Write to type "Sincerely Yours,"

(without the quotation marks, of course) whenever you

press Ctrl A. There is no limit to the number of words

and operation codes you can assign to the Ctrl plus

letter keystrokes, so you can save a lot of finger work

if there are repetitive things that you do regularly

with your word processor. In version 2.5 for those with

128K, its the RULER file that you would edit rather than

the ED file. Check the PC-Write manual for further

details and other customizing possibilities.