PCjr Owner Discovers

How to Use Quad Drives

Many of us dyed-in-the-wool PCjr users want to try

everything possible with our machines to get the last

ounce of performance from a machine that, although

much maligned by many (mostly ill-informed, so-called

PC-journalists), is still quite versatile and still

remains the most economical, genuine IBM.

I have tried to use 80-track (quad density) drives of

720k to 1.2mb on my Jr with mixed success and have

searched for a good, reliable solution. At one time I

thought that the Weltec 1.2mb floppy (which sort of

emulates an AT drive but works from a PC-type FDC) was

the answer. BUT the drive is *VERY SLOW* and not

reliable on a PCjr. PLUS you must purchase special

disks which are expensive. AND the company is not

responsive to consumer problems (I'm still trying

after three months to get my defective unit repaired,

replaced or returned).

Well, here it is!

DOS 3.2 was created for the new IBM Convertible

(laptop) and includes support for every disk format

IBM has ever used.

The 80-track 720k format used on the convertible's 3

1/2" floppies sounded similar to the most common 9-

sector format used by many 80-track quad density

drives on other machines.

YOU NEED:

IBM PCDOS 3.2 on 5 1/4" bootable disk. IBM

PCjr 128k minimum 2d drive PCjr modification

(mine was the Creative Firmware

board that plugs into the modem slot - but others may

work as well)

80-Track drive (5 1/4" OR 3 1/2")

External power supply, cabinet and cable for the drive

WHAT TO DO:

Make up a bootable DOS 3.2 DSDD system disk

At the DOS prompt enter:

copy con config.sys [press enter]

device=driver.sys /d:1 [press enter]

device=driver.sys /d:1 [press enter]

^Z [Fn6 followed by enter - writes the file to disk]

Set up the 80-track drive

for Mitsubishi M4853 jumper DS0, & HM only on the

block of 8 jumpers adjacent to the 34 'finger'

edge card connector.

For CDC 1503 (full-height) jumper leave positions 'M'

and '1' jumpered on the 14 pin socket close to J1.

(You can break the straps on the jumper block or

just remove the plug-in block and make jumpers out

of a couple of 5/8" lengths of solid hook-up wire).

WHAT YOU GET

You now have a disk that will boot with DOS 3.2 and gives:

Drive A: & B: are 360k DSDD

Drive C: & D: are 720k DSQD

You can format standard SSDD (cheap) disks

as DSQD in drive C: or D: as QD.

format c:/v [enter]

You can do diskcopy from C: to D:

You can do diskcomp from C: to D:

The directory of your QD disk can be read by

the AT 1.2mb drive You can read directories

of 40-track DSDD disks in the QD drive You

get pretty high density storage at a

cheap price.  Your disk access time on the

QD drive is a little faster than

a standard DSDD drive (the step rate and

distance is shorter) I have *NEVER* had a

SSDD disk give any bad sectors when

formatted as QD so media cost/kb of storage

is very low.

This procedure may make a

reasonable backup alternative for

PCjr owners with hard disks. (I have tried

Irwin and other tape drives on my 25mb HD

PCjr without success).

WHAT YOU DON'T GET:

Your QD drive will not read or write files

to a 40-track disk Your QD disk files can

not be read or written to by the AT 1.2mb

FD drive even though that is also an 80-

track drive.  Your QD disks if formatted

with '/s' can not boot an AT (or PC or

XT without patching).

P.S.  There is no reason why the above principles

won't work on an AT, PC or XT with appropriate

modifications, but I haven't tried it myself.

Have at it!

-- P. Eskildsen

Lafayette, CA