HOW TO SILENCE THAT NOISY DISK DRIVE

ED.NOTE: This article on how to cure a noisy disk drive

is taken form the Eugene PCjr Club Newsletter, which

reprinted it from another club's newletter. The jr disk

drive is often unusually noisy and this method of

solving the problem can prolong the life of one of your

computer's most important components.

The noise is not the whirring of the disk drive motor,

but a kind of grinding squawk that could well be

compared to a boxcar coming to a halt. Aside from the

annoyance, especially if you're using a program that

goes to disk fairly often, the sound indicates a problem

that will most likely get worse as time passes.

The problem is that the drive head carriage is hanging

up on the rail it travels back and forth on to access

different tracks on the diskette. (This has been

compared to the straight-line-tracking tonearms on some

stereo turntables, where the pickup cartridge moves back

and forth in a straight path across the record instead

of the more usual slight arc found with pivoted tone

arms.) The grinding sound comes from the head mechanism

vibrating as the stepped motor pushes it down a

(possibly misaligned) rail which has insufficient, or

no, lubrication.

Easy Solution

The solution to this isn't hard to come by. It's easy

to do yourself, and hardly worth the downtime you'd have

if you took it to the shop. If you're worried about

doing something wrong, find a repair shop that will do

the job, but be prepared to part company with your

computer for a few days instead of a 15 minute job that

takes very little technical knowhow.

Also, you may not get any satisfaction at a store

anyway, depending on which one you take it to. This is

because the offical IBM line on lubricating drive rails

appears to be "Just forget it." The reason behind this,

according to one technician, is that plastic parts in

contact with the lubricant can be corroded. This may be

true with machine oil, such as 3-in-One, but ain't

necessarily so with other plastic compatible types of

lubricant.

Some repair shops see the folly in this and will

lubricate your drive anyway, even under warranty. They

evidently realize that you're faced with a tradeoff- you

can either let your drive vibrate itself to pieces or

you can take a chance on corrosion. Since the average

life of a drive is supposed to be less than five years

anyway, the choice seems pretty obvious, unless you want

to put up with the noise for a long time.

You can do the job yourself if you're careful and

don't have an inborn fear of machinery. If you haven't

popped open the hood of your jr to see what's inside,

this may be the time. All you will need is some proper

lubricant, some cotton swabs, and a good look at what you

are dealing with.

Be Careful

A little care is called for here, because you don't

want to get any lubricant (or your fingers) on the drive

heads themselves, i.e., the parts that actually touch the

surface of the diskettes. To find the right place to

lubricate, you may also need a flashlight, but everything

is in plain sight and the only thing tht has to be

removed is the top. The best way to see what to

lubricate is to slip a disk in and then turn the power

back ON, watching the head mechanism shift back and

forth.

You can push the head carriage back and forth yourself

(after pulling the plug), but do so gently and keep

your fingers on the top or sides and away from the heads

themselves. Don't worry about getting the carriage out

of position, because it's supposed to get back to home

base automatically every time you power up. You

lubricate the round shiny metal rods that the head

travels back and forth on.

When you put the lubricant on, do so just a little at

a time, and not too many times. A little bit goes a long

way. This also helps make sure that none drips or falls

down below. Move the carriage back and forth a few times

to spread the lubricant along the rail, and you're done.

What kind of lubricant? Take your pick.

Ordinary light machine oil would probably be all

right, but there's that possibility of corrosion, and

anyway it wouldn't last as long as silicon grease or

Teflon oil. Radio Shack sells a Teflon oil (Precision

Oiler) in a handy container about the size of a large

ball point pen, with a tipped applicator that is easy to

squeeze into the tight places.

Another possibility is silicon grease, which comes in

two types; the white type which is to be avoided as too

messy and "stiff," and the clear silicon grease

(actually greyish) and marketed sometimes as "Phonolube"

to lubricate stereo turntable motors and gears, and it

works well.

Lube 10,000 jrs

Also, molybdenum disulfide grease ("molly grease") is

available at some auto parts stores especially speed

shops, but is not available in small amounts, AND

greases may attract dust. The Radio Shack Teflon oil is

supposed to resist dust and last indefinitely. Also it's

only $1.48 for enough to lube 10,000 juniors.

Some people have applied their lubricants with match

sticks or toothpicks, but cotton swabs are better

because they conform to the round shape of the rail and

it's easier to get the lubricant on the bottom surface.

Don't use Q-tips or other medicinal swabs. Use the

kind made especially for cleaning tape recorder heads.

These won't leave a trail of cotton fibers in their

wake.