User Reports Problems

With Upgraded Tecmar Board

EDITOR'S NOTE: The following letter

from Hans J. Weedon recounts his experiences

upgrading a Tecmar jrCaptain. The letter

orginally appeared in PC Report, a

publication of the Boston Computer Society's

IBM SIG.

I upgraded my daughter's PCjr with a

Tecmar jr Captain. This worked find for a

while, but the Writing Assistant that came

in the bargain could not be used for spell

checking very efficiently. It was slow and

often bombed when the disk drive became

"flaky" which it often does on the jr.

We therefore decided to upgrade the Tecmar

board to 512K by removing the 64K chips and

installing 256K chips. This was done very

carefully and all was happiness. On a

RAMDISK the Writing Assistant spell checker

ran like "lightning". After about one half

year of use however the RAMDISK started to

have intermittent problems. The Tecmar

MEMTEST program showed that there were no

problems with the memory addition. I used

DEBUG to modify the MEMTEST program so that

it could write test results on the disk.

After about thirty hours of continuous

testing the problem started to show up.

Whole sections of memory had all sorts of

problems for a few seconds and then fixed

itself.

After much wrong guessing, the problem was

finally found to be the 5 volt regulator on

the heat sink fin in the jr Captain module.

The voltage regulator is plugged into a very

inexpensive socket on the main board. This

socket apparently loses its retention force

and become intermittent under the extra heat

load of the 512K of memory.

The regulator and power supply can easily

stand the extra load, but not the socket. I

removed the socket and soldered the

regulator directly into the board, and the

PCjr has worked fine for nearly a year since

then.

I strongly recommend that in addition to

replacing the 64K chips with 256K chips,

that you solder in the voltage regulator as

well. This will avoid the temptation to

throw the machine at the wall in frustration

over having lost the most important piece of

data.