Telecommunicating With Your PCjr

By Delmar Mineard

By now, most PCjr owners have become seasoned veterans

and are looking for new frontiers to conquer.

Telecommunications offers new challenges and

opportunities to put your PCjr to further use. It is

your window to the world.

For remote computer game playing, as well as for more

serious computing purposes, the telephone serves, in

effect, as a long extension cable, connecting the

mainframe "host" computer to a distant terminal- yours.

On both ends of the line, a modem- short for

modulation/demodulator- transforms digital computer data

into the audible, analog signals that the phone lines

carry, and back to digital again.

Today's personal computer users employ the telephone

not just to access remote mainframe systems, but for an

ever-increasing range of services. Electronic mail

permits the nearly instantaneous exchange of letters,

memos, contracts, articles, and other documents. On-line

information services offer rapidly updated prices for

securities and commodities, as well as huge data banks

of news, research reports, classified ads, and dozens of

other kinds of material. Teleconferencing encourages

conversations among far-flung participants- over a

considerable span of time, if need be- and keeps a

permanent record of dialogue.

Technical Limits

There are, however, technical limits to

telecommunications with computers, principally because

data moves over telephone lines more slowly than we

would like. Data speed, often referred to as the "baud

rate", is measured in bits per second (bps)). Three

hundred bps, long the standard used by information

services, corresponds to about 30 characters per second.

This is slower than the speed at which most people read-

so much slower that a 25 line screen seems to take an

eternity to fill with information.

The 1200 bps modem, which became widely available a

few years ago, made possible a four-fold increase in

speed, and the vast majority of computer

telecommunications now function at this rate.

Now with 2400 bps modems becoming less expensive,

computer hobbyists are making the switch too. 2400 baud

Bulletin Board Systems (BBS) have become the majority

since Hayes and U.S.Robotics made their 2400 baud modems

available to bulletin board System Operators (SYSOP) at

substantial discounts.

Bulletin Board Systems

A BBS is often run by an individual (the SYSOP) who

loves computers and wants to encourage the growth of

telecommunications. He or she has set up a computer with

the appropriate software and modem so you can log onto

the system. BBS's are there to exchange information,

both technical and non-technical, general message base,

and private messages for all ages and sexes. My wife and

kids are all members of a local BBS. And of course, all

BBS's have data base files of public domain software

programs for you to down load to your computer. Often

times you'll find categories of programs such as:

Communications

Data Base

Adventure Games

PCjr Programs

Humorous

Graphics

Utilities

Special files

Turbo Pascal

Photography

Lotus Worksheets

Games

PCjr Text Files

Educational

File Management

Word Processing

Printer

Programming Aids

Text files

Recent Uploads

How To Start Telecommunicating

Telecommunications with the PCjr is not as easy as

with a PC. It presents some special problems. This

article will address these items and prepare you to get

started on the right foot.

All the programs mentioned in this article were tested

with my equipment, which consists of: IBM PCjr with 640K

of RAM; Tecmar Jr. Captain and Cadet; CTS Datacomm 2424

ADH 2400 bps modem.

I strongly recommend that you buy an inexpensive new

or used external 1200 bps modem to start with. Avoid the

PCjr 300 bps internal modem made for IBM by Novation.

Although a good product, it does not use the Hayes

Command set, but uses a special command set for IBM. Not

very many people or User Groups have experience with

this special command set, making it very difficult for

the beginner to get started. The documentation that

comes with this internal modem is insufficient. If your

User Group has people who have or have used it, then you

may want to consider buying one. Locally, they sell used

for $30 to $40. Later, if you get hooked on

telecommunications, you can move up to the 2400 bps

modems.

What Do You Need ?

You need six iems to start your future in

telecommunications:

1. IBM PCjr Adapter cable for Serial devices, P/N

8600026

2. RS-232 Cable, standard connector, 6 feet long

3. A new or used 1200 bps external modem. Price varies

from $100 to $300. New or used, it's a good buy. If you

buy a 300 bps modem you will have problems selling it

later. This speed for telecommunications is dead.

4. Communication Program. There are many good public

domain programs available from local users groups or

computer stores for free or a minor charge. You need one

to communicate with your modem. Later, I will recommend

specific ones that work great on the PCjr. Don't go out

and buy a $100 or $200 commercial package, such as Cross

talk XVI or Smartcom II. The Public Domain programs are

just as good, if not better. I know, since I have used

both types.

5. SWAPCO.EXE or COMSWAP.EXE. These public domain

programs swap the communication ports in DOS so that

your software communiations program can find it. You

only need one of these programs. They are available on

local BBS's and through user groups.

6. Patience. Learning the in's and out's of

telecommunications is like it was when you first started

using your junior. So don't give up. After a month or

two, you too will be an expert.

Telecommunicating Software

There are a lot of good Public Domain communications

programs that will work with the junior and some that

won't. Out of the five that I have tried, my preferences

are as follows:

TMODEM:

If your PCjr has only 128K of memory, the best public

domain program is "TOMDEM". TMODEM is short for Turbo

Modem. It is in color and has nice pop-up windows. It is

easy to use and has a help screen that is two key

strokes away. Once you read the easy to understand

documentation, you will be up and running quickly.

TMODEM is written by Paul Meiners of P&M Software

Company, 9350 Country Creek #30, Houston, Texas, 77036.

Cost (donation) is $20.00.

Personally, I like TMODEM better than QMODEM which is

another popular program preferred by many PCjr users.

TMODEM is loaded with advanced features that make

telecommunications easy and fun.

There is not enugh memory left to use a RAM disk drive

for down/up loading with 128K RAM. Otherwise, this is a

fine program. It is far superior the the Communications

programs provided in Cartridge Basic, on the DOS

Supplemental Programs Disk, and the one provided on the

"PCjr Sampler" Demo disk. This program will work with

the IBM Internal "Infernal" modem, too.

PC Talk III:

PC Talk III was written by Andrew Fluegelman of

Headlands Press, Inc. for the IBM PC. When the PCjr was

introduced, several patches were made available so PCjr

owners could run this popular Public Domain Program.

Two years ago, PC-Talk III was upgraded for the PC and

PCjr and released as EXECJR26. You can often find it on

local BBS's as "EXECJR26.ARC". It is also available from

jr Newsletter.

This is an excellent program and is especially good

for beginners. It is easy to use and does its job well

This was my first program for my junior. The one

disadvantage is its limited number of various transfer

protocols. Only ASCII and Xmodem protocols are

supported. It is necessary to list the file name

followed by "=x" for Xmodem file transfers (don't type

the quotation marks). It is very

easy to forget the "=x" and then you get an error

message. This is a major disadvantage with this program.

EXECJR26 is available from The Headlands Press, Inc.,

PO Box 862, Tiburon, Calif. 94920. Cost is $35.00.

PROCOMM:

For those with 640K of memory and who know how to use

RAM disk drives, this is "THE" program for the jr. The

best configuration is using a RAM disk for easy up loads

and down loads. It will run on a PCjr with 128K of RAM.

It has many advanced features that make

telecommunications a joy to use on the PCjr. A RAM disk

drive makes the lack of DMA unnoticeable. Value per

dollar, there is more in this program than Crosstalk XVI

that sells for $195. As you can see, I really love this

package. It's my absolute favorite.

ProComm will also run on any PC compatible computer.

The most recent version is 2.4.2. As in most Shareware

programs there are a few "bugs" in them. ProComm 2.1,

2.2, and 2.3 are no exceptions. I prefer version 2.2

over 2.1 or 2.3. However, 2.3 will work on the PCjr.

Just don't do any ASCII up loads into any message bases.

For the last three weeks I have been using ProComm 2.4.2

and feel it is the best version to date. That is why I

am a registered user of this fine telecommunications

program.

ProComm uses windows very effectively, allowing you to

pick the colors of the various windows. I really like the

whistle you get when the windows change. The sixty-six

page documentation, version 2.2, is very good and it

does address the PCjr configuration problem. The

Documentation has been completely updated for version

4.0. There have been some minor keystroke changes from

the earlier versions, so be sure you read the new

documentation for version 2.4.2.

One thing that is not mentioned clearly in the

Documentation is the transmit pacing adjustment. This

affects the keyboard macro sending speed. Go to the set-

up menu, Alt-S; select General setup, #4; and change the

transmit pacing value, item #17, from 30 to 0. For BBS

communications this will work fine. For mainframe

applications, Datastorm recommends the 30 ms value. The

30 ms value causes a 30 ms deay between every character

transmitted to the receiving modem.

ProComm was written and released by PIL Software and

was just recently renamed Datastorm Technology, Inc.

Their address is: PO Box 1471, Columbia, MO 65205. Cost

is $25.00. It is also availale from jr Newsletter (Disk

#15).

(End of Part I)