o would rather have a device driver that does the

same thing instead of a TSR, I include INT9SYS.  To use it, put the line

DEVICE=INT9SYS.SYS

in your CONFIG.SYS file.  You should use either INT9JR or INT9SYS, but not

both (though using both won't hurt).

I've done this, and tried to test it as much as I can.  It hasn't interfered

with normal operations, SideKick, comm programs, or editors, and it certainly

makes QB3 work as advertised.  But if anybody finds any problems, let me know.

For my next trick ....  That NMI thing associated with the jr keystrokes is

the main reason why programs that use the 8087 math chip do not typically work

on jrs equipped with 8087 daughterboards, like TIAC's (the one I have).

Normal PCs have the 8087 use the NMI to signal errors in math, like divide-by-

zero.  The program can then grab the NMI to capture such errors without having

to check for them in software, making the program faster. Normally, nothing

else uses NMI, so most programs that grab NMI don't make provisions to pass it

on.  So, on the jr, if a program does that, it grabs the interrupt that

enables you to type, basically locking the system if the program wants you to

type something at it.  I developed a routine to fix this problem on

Microsoft's mainline languages (C, Fortran, Pascal), and called it JR87OEM.ARC

in the Hardware DL on the CompuServe PC Junior Forum.  For