Like most other jr owners, I was dismayed when I got my update copy of

Microsoft QuickBasic 3.0 and found it typing characters twice while I was

using the built-in editor.  I had heard that the jr "would not be supported"

by QB 3.0, but they said the same thing about 2.0, and I use that all the

time.  So, I went to see what the trouble was.

The big symptom was the character was being entered on both the make and break

of the keystroke.  Ha, bet I caught you with terminology there!  The "make" of

a keystroke is simply the key being pressed down, the "break" is when the key

is released.  The hardware treats these as two different events, telling the

computer a different thing for the make and break of the same key (the

difference is that the make scan code's high bit is low, while the break's is

high).  This to me meant that the break keystroke was losing its high bit

before it got processed by the INT 9H routine, which translates scan codes

into characters.  That was also why the shift keys got stuck in the QB3

editor; the computer never saw the break of the shift key, so it assumed you

were always holding it down.

Well, fellow jr lovers, it turns out that the fault in this matter is the

jr's, at least the jr ROM BIOS.  Normal INT 9H on a PC and compatible gets the

scan code to process from a "port".  When you read this port, it does not

change what is there (it's a "non-destructive read").  Thus, when INT 9H is

grabbed, the grabbing procedure can read from the port and know that it

won't affect anything farther down the INT 9 chain, because they should read

from the port, too, and get the same value.  It's a valid assumption, and

MicroSoft assumed it for the QB3 editor's INT 9H grabbing strategy.

But it doesn't hold for the jr.  In order to make the jr INT 9 level

compatible with the PC, the jr BIOS uses a three-tiered approach.  First, a

software interrupt translates the infrared signals into a 62-key scan code

(this happens to be the NMI interrupt, more on that later).  A second software

interrupt then translates this 62-key code into an IBM 83-key code, and then

passes it on to the third software interrupt, INT 9H.  This makes the INT 9H

code very similar between the two machines, plus gives the possibility of

adding an 83-key keyboard to the jr.  In software, you would just bypass the

first two interrupts and go to the third.  (If you don't believe me, look at

the ROM diagnostics with Ctrl-Alt-Ins.  There are *two* keyboard diagnostics,

one for 62 keys, the other for 83 keys.)

BUT, even though the second level software interrupt does a write to the port,

the jr INT 9H procedure does NOT do a read from the port.  It assumes the scan

code will be coming in the AL register.  When INT 9H directly follows the

second level interrupt, that is the case.  BUT, if something else has grabbed

INT 9H, it may not put the scan code into the AL register before passing it

on.  This is probably why the early version of SideKick, an INT 9H grabber,

didn't work on the jr.  In QB3's case, what was in the AL register before it

got to the ROM BIOS to be decoded was the scan code with the high bit masked

off, which explains everything.

The fix is so simple, it's a wonder why Microsoft didn't offer it on the QB

disk package and tell jr owners "Do this to get it to work."  All you have to

do is have a TSR program right ahead of BIOS in the INT 9H chain, have it

read the port and put the value in the AL register, and then pass control to

BIOS.  When QB3 comes along, it grabs INT 9H from the TSR routine, not BIOS.

When control eventually gets to BIOS, the TSR routine has put things in the

right place.

Well, I wrote it!  I call it INT9JR, and I've included the assembly source and

the .COM file with this ARC.  Just put it before any other INT 9H grabbing

routine, so it is always followed by BIOS.  This typically means putting the

line

INT9JR

as close to the beginning of AUTOEXEC.BAT as possible.

For those of you out there who would rather have a device driver that does the

same thing instead of a TSR, I include INT9SYS.  To use it, put the line

DEVICE=INT9SYS.SYS

in your CONFIG.SYS file.  You should use either INT9JR or INT9SYS, but not

both (though using both won't hurt).

I've done this, and tried to test it as much as I can.  It hasn't interfered

with normal operations, SideKick, comm programs, or editors, and it certainly

makes QB3 work as advertised.  But if anybody finds any problems, let me know.

For my next trick ....  That NMI thing associated with the jr keystrokes is

the main reason why programs that use the 8087 math chip do not typically work

on jrs equipped with 8087 daughterboards, like TIAC's (the one I have).

Normal PCs have the 8087 use the NMI to signal errors in math, like divide-by-

zero.  The program can then grab the NMI to capture such errors without having

to check for them in software, making the program faster. Normally, nothing

else uses NMI, so most programs that grab NMI don't make provisions to pass it

on.  So, on the jr, if a program does that, it grabs the interrupt that

enables you to type, basically locking the system if the program wants you to

type something at it.  I developed a routine to fix this problem on

Microsoft's mainline languages (C, Fortran, Pascal), and called it JR87OEM.ARC

in the Hardware DL on the CompuServe PC Junior Forum.  For QB87, the 8087

version of QB3, that fix isn't applicable, so I'm going to have to come up

with a new one.  Trust me, I know what I'm doing.

John Bongiovanni

CompuServe 70137,2401

PCjr Conference Co-Sysop

Lunacy PC Board BBS

(818) 894-1248