These files will enable PCjr users to do things never before possible:

-You can configure a "high" RAMdisk (up to 512k) *AND* load Cartridge

BASIC, use SCREEN 5 or 6, or do anything else you used to be able to

do with CBASIC back when you only had 128k!  [use RAMDRIVE.SYS file]

-You can run BASIC or BASICA *without* a cartridge (from disk) [see

FIXBASIC.TXT file].

-You can configure you system in several *new* ways with a *replacement*

for PCJRMEM.COM, JRCONFIG, or whatever memory driver you're now using.

[use FIXMEMJR.SYS /G#]

-You can speed up graphics programs (not written in BASIC).  [FIXMEMJR

again!]

The use of all these files is fully explained in all the files herein.

There are some "quirks" I'll point out here, tho, in case you're so anxious

to get started that you read them too fast!

Organized by file:

RAMDRIVE.SYS--This was *boon* for me because I have several statistics

programs (written in BASIC) which used some 300k of data files.  I had

to load two sets of files separately to use them all into a 128k RD--

the maximum heretofore possible due to the way the Cartridge BASIC checks

available memory.

NOTES:--You do NOT use a memory-recognition driver (PCJRMEM.COM,

JRCONFIG, etc.), because video memory STAYS at the high

end of 128k.

--You CANNOT "un-hook" the driver (to reconfigure with a

command line-type memory driver)--you MUST reboot.

--You CAN configure *any* size RAMdisk: 10k, 11k, 27k,

etc., up to 512k!

FIXMEMJR.SYS /G#--This driver is a replacement for PCJRMEM.COM, JRCONFIG,

etc.  There are three "switches": G1, G2 or G3 (explained

in NOTES.DOC).

NOTES:--You CAN use this driver with RAMDRIVE.SYS, but list this

one first (before RAMDRIVE).

--Contains a trap to prevent accessing Cartridge BASIC when

memory greater than 256k (since it won't work too well

anyway in graphics modes when video buffer is relocated).

--Provides an option to set up a 128k video buffer, which

"uses up" all SLOW low memory--thus enabling commercial

(non-BASIC) graphics programs to run in high, presumably

faster, memory.

These files and drivers are furnished (and written by) Jeff McKee, of

Teledyne Corporation--IBM's subcontractor for construction of the PCjr.  He

has released these files to the public domain, and says:  "ENJOY!"

Uploaded 7/30/87 by George Peters, 71330,200