IBM INTERNAL USE ONLY

*********************

POWER-ON DIAGNOSTIC ERROR REPORTING

OVERVIEW:

THE POWER-ON DIAGNOSTICS FOR PCjr HAVE SEVERAL ERROR

INTERFACES TO ACCOMMODATE THE VARIOUS SITUATIONS IN WHICH THEY

(THE DIAGNOSTICS) ARE USED.

1.  MANUFACTURING - WHEN THE 4 BITS THAT ARE USED FOR THE

JOYSTICK "FIRE" BUTTONS (NORMALLY PULLED "UP") ARE ALL PULLED

TO GROUND (LEFT NIBBLE OF INPUT FROM PORT 201H = 0 OR 1) THE

DIAGNOSTICS ASSUME THAT THEY ARE IN "MANUFACTURING MODE".  IF

A FAILURE IS FOUND, A ROUTINE ID IS SENT TO ADDRESS "10H"

FOLLOWED BY TWO BYTES OF DETAILED ERROR INFORMATION

OUTPUTTED TO I/O ADDRESS "11H & 12H".  (THIS IS FOR ERROR LATCHES

ATTACHED TO THE CHANNEL THAT WILL CAPTURE AND REPORT

THESE 3 BYTES OF DATA TO THE OUTSIDE WORLD.)  THE SAME DATA IS

SENT (IN THE SAME SEQUENCE) TO THE OUTSIDE WORLD VIA THE ON-BOARD

RS232 INTERFACE (RS232 COMMUNICATION MAY NOT WORK FOR SOME EARLY

(I.E. VERY BASIC) ERRORS). AFTER OUTPUTTING THE 3 ERROR BYTES

THE SYSTEM WILL ENTER A HARD HALT THAT ONLY A

SYSTEM RESET WILL BREAK OUT OF.

IT IS INTENDED THAT THE MANUFACTURING ERROR CODES PROVIDE A

MAXIMUM AMOUNT OF DETAILED ERROR INFORMATION AS IS POSSIBLE SO

THAT A FAILURE CAN BE ISOLATED TO A FUNCTIONAL AREA OF CIRCUITY

OR, IN SOME CASES, TO A SINGLE CHIP.

********NOTE********

IN "MANUFACTURING MODES" AN OUT IS SENT TO ADDRESS 13H AND THE

STATE OF THE AUDIO OUTPUT CONNECTOR IS FLIPPED APPROX. EVERY 55

MILLISECONDS.

2.  SERVICE - SOMEWHAT OF A COMBINATION OF MANUFACTURING AND

CUSTOMER MODES.  BEFORE THE CRT IS STARTED, ERRORS ARE REPORTED

VIA BEEPS FROM THE INTERNAL SPEAKER WITH THE GOAL OF EITHER

CALLING THE PLANAR BOARD OR 64K ATTRIBUTE CARD AS THE FRU.

AFTER THE CRT IS STARTED, MANUFACTURING ERROR CODES ARE

DISPLAYED ON THE CRT. (NOTE- THE DOCUMENTATION PROVIDED IN THE

SERVICE PACKAGE WILL POINT TO FRUS, NOT CHIPS).  WHEN THE ERROR

CODE HAS BEEN DISPLAYED, THE PROCESSOR WILL ENTER A HARD HALT

STATE.

3.  CUSTOMER - BEFORE THE CRT IS STARTED, AN ERROR WILL BE

COMMUNICATED TO THE CUSTOMER BY 2 SHORT BEEPS FROM THE INTERNAL

SPEAKER.  AFTER THE CRT IS STARTED, THE WORD "ERROR" ALONG WITH A

SINGLE LETTER THAT DESCRIBES IN GENERAL TERMS WHAT PART OF THE

SYSTEM HAS FAILED WILL BE SHOWN ON THE CRT.  WITH THE EXCEPTION

OF MEMORY ERRORS (WHICH HALT THE SYSTEM) THE CUSTOMER IS GIVEN

THE OPTION OF CONTINUING WITH SYSTEM START-UP WITH THE DEPRESSION

OF THE "ENTER" KEY AT THE END OF THE DIAGNOSTIC SEQUENCE.

THE LOGIC BEHIND LETTING THE CUSTOMER CONTINUE AFTER AN

ERROR IS THAT THE PORTION OF THE SYSTEM THAT HAS FAILED MAY BE OF

NO IMPORTANCE TO THE CUSTOMER (OR AT LEAST NOT CRITICAL AT THE

TIME OF THE FAILURE.)

EXAMPLES OF THIS MIGHT BE:  1.  A FAILURE OF THE SERIAL

PRINTER PORT AND THE SYSTEM DOES NOT HAVE A SERIAL PRINTER

ATTACHED.

2. IR RECEIVER CARD FAILURE AND CUSTOMER HAS HIS KEYBOARD CABLE

ATTACHED.

------------JOY STICK FIRE BUTTON USAGE----------------

---PORT 201 VALUE-----                                   ----MEANING------

0000XXXXB (GROUND PINS J2- AB04,B03 & BB04,B03)         BURN-IN TEST (MFG)

0001XXXXB (GROUND PINS J2- AB03 & BB04,B03 + RESISTORS) RUN-IN TEST (MFG)

0010XXXXB (GROUND PINS J2- AB04 & BB04,B03)             LOOP POST (SERVICE)

0100XXXXB (GROUND PINS J2-AB04,B03 & BB03)              RUN DIAGNOSTICS IN

"SERVICE MODE"

(XXXX=DON'T CARE)

MANUFACTURING ERROR CODE        SERVICE CODE            CUSTOMER LEVEL CODE

------------------------        ------------            -------------------

000001 (FVT/PFT TESTER)    SILENCE                     SILENCE

NOTHING (BURN-IN/RUN-IN)

------                     -------                     -------

ERROR IN CPU REGISTERS, FLAGS, OR CONDITIONAL JUMP

INSTRUCTIONS.

FE0002 (FVT/PFT TESTER)    2 BEEPS                      2 BEEPS

??0002 (BURN-IN/RUN-IN)    -------                     -------

------

8255 PORT A OR B DATA ERROR. - THIS IS THE FIRST I/O OPERATION

THAT HAS BEEN CHECKED SO THE I/O DECODE ROM CHIP MAY ALSO BE

CAUSING THE PROBLEM.  8255 OUTPUTS ARE NOT VALIDATED BY THIS

TEST.

FC0003 (FVT/PFT TESTER)   2 BEEPS                      2 BEEPS

??0003 (BURN-IN/RUN-IN)   -------                      -------

------                    -------                      -------

ROM CHIP AT ADDRESS F000:0000 FAILED CHECKSUM.

FC0004 (FVT/PFT TESTER)   2 BEEPS                      2 BEEPS

??0004 (BURN-IN/RUN-IN)   -------                      -------

------

ROM CHIP AT ADDRESS F800:0000 FAILED CHECKSUM.

FB04XX (FVT/PFT TESTER)   2 BEEPS                      2 BEEPS

??04XX (BURN-IN/RUN-IN)   -------                      -------

------

MEMORY ERROR ON PLANAR BOARD MEMORY. XX=BITS IN ERROR (ANY BIT

"ON" IN "XX" IS A FAILING BIT (I.E., CHIP)

FA05XX (FVT/PFT TESTER)   3 BEEPS                      2 BEEPS

??05XX (BURN-IN/RUN-IN)   -------                      -------

------

MEMORY ERROR ON 64K ATTRIBUTE CARD. XX=BITS IN ERROR (ANY BIT

"ON" IN "XX" IS A FAILING BIT (I.E., CHIP)

F906XX (FVT/PFT TESTER)   2 BEEPS                      2 BEEPS

??06XX (BURN-IN/RUN-IN)   -------                      -------

------

UNDEFINABLE MEMORY ERROR ON 128K SYSTEM.  THIS OCCURS WHEN

ERRORS ARE FOUND IN BOTH THE ODD AND EVEN BYTES FOR THIS

CONFIGURATION (THAT IS TO SAY, BOTH THE PLANAR BOARD MEMORY AND

THE 64K ATTRIBUTE CARD ARE FAILING.) XX BITS IN ERROR ARE

PROVIDED FOR WHATEVER HELP THEY MAY GIVE.

F707XX (FVT/PFT TESTER)   2 BEEPS                      2 BEEPS

F607XX (BURN-IN/RUN-IN)   -------                      -------

------

8259 INTERRUPT CONTROLLER ERROR. IF XX=00 ERROR WAS CAUSED BY

EITHER A INTERNAL REGISTER FAILURE IN THE 8259 OR BY A DATA PATH

FAILURE.  FOR XX=NON-ZERO, IT IS ALMOST CERTAIN THAT THE 8259

ITSELF IS AT FAULT. (IT LET AN INTERRUPT THROUGH WHEN ALL

INTERRUPTS SHOULD HAVE BEEN MASKED OFF INTERNAL TO THE CHIP)

MANUFACTURING ERROR CODE        SERVICE CODE            CUSTOMER LEVEL CODE

------------------------        ------------            -------------------

F608XX (FVT/PFT TESTER)   2 BEEPS                      2 BEEPS

F508XX (BURN-IN/RUN-IN)   -------                      -------

------

8253 TIMER ERROR.

XX=00 STUCK BITS IN TIMER 0

XX=01 TIMER 1 CANNOT BE CLOCKED BY TIMER 0

XX=02 TIMER 0 INTERRUPT (LEVEL 0) FAILED

XX=03 STUCK BITS IN TIMER 1

XX=04 TIMER 2 OUTPUT HIGH WHEN IT SHOULD BE LOW

XX=05 STUCK BITS IN TIMER 2

XX=06 TIMER 2 OUTPUT DOES NOT GO HIGH WITH TERMINAL COUNT

F509XX (FVT/PFT TESTER)   2 BEEPS                      2 BEEPS

F409XX (BURN-IN/RUN-IN)   -------                      -------

------

FAILURE IN CRT SUPPORT LOGIC

XX=00   NO VERTICAL SIGNAL

XX=01   VERTICAL STUCK ON SOLID

XX=02   NO VIDEO ENABLE SIGNAL (CORRESPONDS TO HOROZ. RETRACE)

XX=03   VERTICAL STUCK ON SOLID

XX=04   VIDEO ENABLE STUCK ON SOLID

XX=05   WRONG NUMBER OF ENABLES (HOROZ. RETRACES) BETWEEN

VERTICAL RETRACES

XX=06   VERTICAL RETRACES ARE COMING TOO FAR APART IN TIME

XX=07   VERTICAL RETRACES ARE TOO CLOSE TOGETHER IN TIME

XX=08   VERTICAL INTERRUPT (8259 LVL. 5) FAILED

XX=1Y   DOT NOT COMING ON. Y = 0, BLUE. 1, GREEN. 2, RED.

3, INTENSITY.

XX=2Y   DOT STUCK ON. Y = 0, BLUE. 1, GREEN. 2, RED.

3, INTENSITY.

MANUFACTURING ERROR CODE        SERVICE CODE            CUSTOMER LEVEL CODE

------------------------        ------------            -------------------

F30AXX (FVT/PFT TESTER)   0AXX                            A

F20AXX (BURN-IN/RUN-IN)   ----                           ---

------

MEMORY ERROR ON PLANAR BOARD MEMORY. XX=BITS IN ERROR (ANY BIT

"ON" IN "XX" IS A FAILING BIT (I.E., CHIP)

F30BXX (FVT/PFT TESTER)   0BXX                            A

F20BXX (BURN-IN/RUN-IN)   ----                           ---

------

MEMORY ERROR ON 64K ATTRIBUTE CARD. XX=BITS IN ERROR (ANY BIT

"ON" IN "XX" IS A FAILING BIT (I.E., CHIP)

F30CXX (FVT/PFT TESTER)   0CXX                            A

F20CXX (BURN-IN/RUN-IN)   ----                           ---

------

UNDEFINABLE MEMORY ERROR ON 128K SYSTEM.  THIS OCCURS WHEN

ERRORS ARE FOUND IN BOTH THE ODD AND EVEN BYTES FOR THIS

CONFIGURATION (THAT IS TO SAY, BOTH THE PLANAR BOARD MEMORY AND

THE 64K ATTRIBUTE CARD ARE FAILING.) XX BITS IN ERROR ARE

PROVIDED FOR WHATEVER HELP THEY MAY GIVE.

F31YXX (FVT/PFT TESTER)   1YXX                            A

F21YXX (BURN-IN/RUN-IN)   ----                           ---

------

MEMORY ERROR IN ADDRESS SPACE ABOVE 128K. Y=SEGMENT HAVING THE

ERROR, XX=ERROR BITS.

*******NOTE ON MEMORY ERRORS**********

IF AN ERROR ONLY OCCURS IN MFG/SERVICE MODE, IT PROBABLY MEANS THERE

IS A PROBLEM IN THE REFRESH (GATE ARRAY) LOGIC.  (A "DATA RETENTION"

TEST IS PERFORMED IN THESE MODES).

MANUFACTURING ERROR CODE        SERVICE CODE            CUSTOMER LEVEL CODE

------------------------        ------------            -------------------

F22000 (FVT/PFT TESTER)   2000                            B

F12000 (BURN-IN/RUN-IN)   ----                           ---

------

STRAY NMI INTERRUPT OR RECEIVE ERRORS FROM KEYBOARD.

F221XX (FVT/PFT TESTER)   21XX                            B

F121XX (BURN-IN/RUN-IN)   ----                           ---

------

IR RECEIVER CARD ERROR.

XX=01   RECEIVER CARD PRODUCING ACTIVE OUTPUT WITH NO INPUT

XX=02   RECEIVER CARD NOT PRODUCING OUTPUT WITH INPUT APPLIED

XX=03   NOT ABLE TO GENERATE NMI INTERRUPT FROM IR CARD

F222XX (FVT/PFT TESTER)   22XX                            B

F122XX (BURN-IN/RUN-IN)   ----                           ---

------

STUCK KEY ON KEYBOARD. XX= SCAN CODE.

F12300 (FVT/PFT TESTER)   2300                            C

F02300 (BURN-IN/RUN-IN)   ----                           ---

------

WRAP BACK OF CASSETTE INTERFACE FAILED

F123FF (FVT/PFT TESTER)   23FF                           N/A

F023FF (BURN-IN/RUN-IN)   ----                           ---

------

CASSETTE  RELAY FAILED TO PICK (SERVICE/MFG MODES ONLY)

MANUFACTURING ERROR CODE        SERVICE CODE            CUSTOMER LEVEL CODE

------------------------        ------------            -------------------

F023XX (FVT/PFT TESTER)   23XX                            D

EF23XX (BURN-IN/RUN-IN)   ----                           ---

------

SERIAL PORT (ON PLANAR BOARD) 8250 FAILURE

XX=02 BITS THAT SHOULD ALWAYS BE "0" IN 8250 INT. ENABLE REG. WERE "ON"

XX=03 BITS THAT SHOULD ALWAYS BE "0" IN 8250 INT. I/D REG. WERE "ON"

XX=04 BITS THAT SHOULD ALWAYS BE "0" IN 8250 DATA SET CONTROL REG. WERE "ON"

XX=05 BITS THAT SHOULD ALWAYS BE "0" IN 8250 LINE STATUS REG. WERE "ON"

XX=06 RECEIVED DATA AVAILABLE INTERRUPT STATUS NOT PRESENTED FROM 8250

XX=16 RECEIVED DATA AVAILABLE INTERRUPT STATUS COULD NOT BE CLEARED

XX=17 TRANSMITTER HOLDING REG. INTERRUPT STATUS COULD NOT BE CLEARED

XX=08-0B RECEIVER LINE STATUS INTERRUPT COULD NOT BE GENERATED IN 8250

08=OVERRUN ERROR

09=PARITY ERROR

0A=FRAMING ERROR

0B=BREAK INTERRUPT

XX=18-1B RECEIVER LINE STATUS INTERRUPT COULD NOT BE CLEARED IN 8250

XX=0C-0F MODEM STATUS INTERRUPT COULD NOT BE GENERATED

0C=DELTA CLEAR TO SEND

0D=DELTA DATA SET READY

0E=RING INDICATE

0F=DELTA RECEIVE LINE SIGNAL DETECT

XX=1C-1F COULD NOT RESET MODEM STATUS INTERRUPT IN 8250

****NOTE***** THE "INTERRUPTS" NOTED IN THE ERRORS ABOVE ARE INTERNAL STATUS

BITS IN THE 8250. THEY HAVE NOTHING TO DO WITH THE SYSTEM 8259

INTERRUPT CHIP.

XX=10 SYSTEM INTERRUPT (8259) FROM 8250 FAILED

XX=11 TRANSMIT HOLDING REGISTER EMPTY DID NOT OCCUR DURING WRAP TEST

XX=12 RECEIVED DATA AVAILABLE DID NOT OCCUR DURING WRAP TEST

XX=13 TRANSMIT DATA NOT EQUAL TO RECEIVED DATA IN WRAP TEST

XX=14 DATA SET CTL.OUTPUTS NOT EQUAL TO DATA SET CTL.INPUTS IN WRAP MODE

XX=15 TRANSMITTER NOT IDLE AFTER WRAP TEST

MANUFACTURING ERROR CODE        SERVICE CODE            CUSTOMER LEVEL CODE

------------------------        ------------            -------------------

EF24XX (FVT/PFT TESTER)   24XX                            E

EE24XX (BURN-IN/RUN-IN)   ----                           ---

------

MODEM 8250 ERROR

XX VALUES ARE IDENTICAL TO THOSE FOR THE ON-BOARD 8250 LISTED ABOVE

EE25XX (FVT/PFT TESTER)   25XX                           F/G

ED25XX (BURN-IN/RUN-IN)   ----                           ---

------

CRC ERROR IN ROM. XX=MSB OF SEGMENT IN ERROR. -NOTE- CUSTOMER

ERROR CODE "F" REPRESENTS ERROR IN I/O ROM ADDRESS SPACE, "G"

POINTS TO AN ERROR IN CARTRIDGE SPACE. HIGH ORDER BYTE OF SEGMENT IN ERROR

IS ALSO SHOWN IN CUSTOMER MODE FOLLOWING THE "F" OR "G" ERROR.

EC26XX (FVT/PFT TESTER)   26XX                            H

EB26XX (BURN-IN/RUN-IN)   ----                           ---

------

DISKETTE ERROR.

XX=01 RESET TO DISKETTE CONTROLLER FAILED

XX=02 RECALIBRATE & SEEK TO TRACK 38 FAILED

XX=03 WATCHDOG TIMER FAILURE

DISKETTE DIAGNOSTIC ERROR MESSAGES

MANUFACT. ID = 3EXYZZ

THE CODES APPEAR IN THE FORM "3EXYZZ", WHERE:

X=0, SIDE 0

X=1, SIDE 1

Y=OPERATION THAT FAILED

0=RESET

2=READ

3=WRITE

4=VERIFY

5=FORMAT

ZZ=ENDING STATUS

80=TIMEOUT

40=SEEK FAILURE

20=NEC CONTROLLER ERROR

10=CRC ERROR IN DATA FIELD

09=DMA ACROSS 64K BOUNDARY (SHOULD NEVER HAPPEN)

08=DMA OVERRUN (SHOULD NEVER OCCUR)

04=SECTOR NOT FOUND

02=MISSING ADDRESS MARK

01=INVALID COMMAND ISSUED TO NEC CONTROLLER

98=SPEED ERROR

97=DISKETTE ATTACHMENT CARD NOT PRESENT

99=READ BUFFER MISCOMPARE WITH WRITE BUFFER

CRT TEST ERROR CODES

MANUFACT. ID = 3D42XX

MOST ERRORS IN THE CRT TEST MUST BE VISUALLY DETECTED.

HOWEVER, THERE IS ONE SYSTEM DETECTABLE ERROR.

MANUFACTURING ERROR CODE        SERVICE CODE            CUSTOMER LEVEL CODE

------------------------        ------------            -------------------

42XX                    42XX                            B

----                    ----                           ---

ERROR IN CRT/CPU PAGE REGISTER LOGIC (GATE ARRAY, 6845, OR

ASSOCIATED TTL GLUE.

JOYSTICK DIAGNOSTIC

MANUFACT. ID = 3CPQRS

B) SERVICE AND MANUFACTURING MODES

These joystick tests are similar to the customer level test because

several codes corresponding to separate parts of the joystick interface

are returned. There a separate code is returned for each of the four

joystick axes. This test assumes that four fixed resistors are connected

where the joystick variable pots would be. The required values are

below.

STICK A X AXIS    |     20K Ohms (1%)  J2-B-B02

Y AXIS    |     39K Ohms (1%)  J2-B-A03

STICK B X AXIS    |     62K Ohms (1%)  J2-A-B02

Y AXIS    |     82K Ohms (1%)  J2-A-A03

The error codes returned are as follows:

PQRS    <-----<<  error codes for the four axes

P corresponds to stick A X axis

Q corresponds to stick A Y axis

R corresponds to stick B X axis

S corresponds to stick B Y axis

The codes:

0 means timeout occurred within tolerance.

8 means timeout occurred too early.

Anything else means timeout occurred too late.

NOTE because a known fixture is attached to the

joystick port, any error indicates a problem on

the planar board.

INTERNAL MODEM DIAGNOSTIC ERROR CODES

MANUFACT. ID = 39XXYY

;------------------------------------------------------------------------------

;                                                                             :

;   MODEM DIAGNOSTIC ROUTINE                                                  :

;                                                                             :

;       TESTS IF THE MODEM IS PRESENT, TESTS THE BAUD RATE GENERATOR,         :

;       INITIALIZES THE MODEM, PERFORMS HARDWARE INTEGRITY, DTR, TRANSPARENT, :

;       ANALOG, AND DIAL TONE TESTS. (THE MODEM UART IS TESTED DURING POWER   :

;       ON DIAGNOSTICS.)                                                      :

;       SUCCESSFUL COMPLETION OF THE DIAGNOSTIC LEAVES THE MODEM IN ITS       :

;       INITIALIZED STATE.                                                    :

;                                                                             :

;                                                                             :

;   MANUFACTURING LEVEL ERROR CODES:                                          :

;   -----------                                                               :

;   | XX | YY |       DESCRIPTION:                                            :

;   -----------                                                               :

;     01   10H        MODEM CARD IS NOT PRESENT                               :

;                                                                             :

;                     FAILURES DURING:                                        :

;                                                                             :

;     02   2XH        INITIALIZATION OF MODEM                                 :

;     02   3XH        REQUEST FOR TERSE RESPONSES                             :

;     02   4XH        QUERY FOR STATUS (TO DETERMINE IF DIGITAL TEST PASSED)  :

;     02   5XH        QUERY FOR STATUS WHEN NO RESPONSE IS EXPECTED (DTR TEST):

;     02   6XH        REQUEST TO PUT MODEM IN TRANSPARENT MODE (FOR TRANS-    :

;                     PARENT TEST)                                            :

;     02   7XH        QUERY FOR STATUS WHEN NO RESPONSE IS EXPECTED (DURING   :

;                     TRANSPARENT TEST)                                       :

;     02   8XH        QUERY FOR STATUS WHEN RESPONSE IS EXPECTED (DURING      :

;                     TRANSPARENT TEST)                                       :

;     02   9XH        ANALOG LOOP BACK TEST                                   :

;     03   AXH        DIAL TONE TEST FAILURES                                 :

;                                                                             :

;                     WHERE X = 0, 1, 4 ,6, OR 7 INDICATES A PROBLEM WHILE    :

;                     SENDING A COMMAND STRING OR A SINGLE CHARACTER TO       :

;                     THE MODEM.  WHILE WAITING FOR THE TRANSMITTER HOLDING   :

;                     REGISTER INTERRUPT TO OCCUR TO DETERMINE IF A CHAR-     :

;                     ACTER WAS TRANSMITTED:                                  :

;                     0 = MODEM STATUS INTERRUPT OCCURRED                     :

;                     1 = NO INTERRUPT OCCURRED                               :

;                     4 = RECEIVED DATA AVAILABLE INTERRUPT OCCURRED          :

;                     6 = LINE STATUS INTERRUPT OCCURRED                      :

;                     ALSO,                                                   :

;                     7 = CLEAR TO SEND AND/OR DATA SET READY WERE NOT ON     :

;                         (SHOULD BE WIRED HIGH)                              :

;                                                                             :

;                     WHERE X = 9 THROUGH E INDICATES A PROBLEM WHILE         :

;                     WAITING FOR OR READING THE MODEM RESPONSE.              :

;                     9 = RECEIVED DATA AVAILABLE INTERRUPT TIMEOUT (DIDN'T   :

;                         OCCUR W/IN 15 SECONDS)                              :

;                     A = LINE STATUS INTERRUPT OCCURRED WHILE WAITING FOR    :

;                         RECEIVED DATA AVAILABLE INTERRUPT                   :

;                     B = THE MODEM TRIED TO SEND BACK MORE THAN 100          :

;                         CHARACTERS                                          :

;                     C = LINE STATUS INTERRUPT OCCURRED WHILE READING MODEM  :

;                         RESPONSE                                            :

;                     D = INTERRUPT (TRANSMITTER HOLDING REGISTER EMPTY OR    :

;                         MODEM STATUS) STUCK ON                              :

;                     E = DATA READY BIT HIGH ON EXIT, BUT NO INTERRUPTS      :

;                         WERE PENDING                                        :

;                                                                             :

;                     WHERE X = F MEANS:                                      :

;                     F = WRONG RESPONSE RECEIVED FROM THE MODEM              :

;                         (SOMETHING OTHER THAN WHAT WAS EXPECTED WAS         :

;                         RECEIVED)                                           :

;                                                                             :

;                     NOTE:                                                   :

;                         THERE ARE NO 29-2F ERRORS BECAUSE NO RESPONSE IS    :

;                         LOOKED FOR AFTER INITIALIZING THE MODEM.            :

;                         IF ERRORS 59-5F OR 79-7F OCCUR, THE MODEM TRIED     :

;                         TO OR DID RESPOND WHEN IT SHOULDN'T HAVE.           :

;                         THERE IS NO 8F ERROR BECAUSE THE RESPONSE IS NOT    :

;                         CHECKED DURING THAT TEST.                           :

;                                                                             :

;     04   15H        BAUD RATE GENERATOR TEST FAILED - EITHER THE MODEM OR   :

;                         THE PLANAR DOESN'T WORK PROPERLY.                   :

;------------------------------------------------------------------------------

PARALLEL PRINTER DIAGNOSTIC

MANUFACT. ID = 38XXXX

|     XXXX      |       CONDITION

--------------------------------------------------------------------------

|     4100      |   PRINTER CARD NOT FOUND. (SELECTION CHAR. REV. VID.)

|     4200      |   PRINTER OUT OF PAPER

|     4300      |   PRINTER CONFIGURATION PROBLEM. EG, NO POWER TO PRINTER,

|               |       PRINTER CABLE NOT CONNECTED PROPERLY, WRAP PLUG

|               |       CONNECTED, PRINTER CARD PROBLEM. USER SHOULD

|               |       CAREFULLY VERIFY CORRECT SETUP, RERUN TEST, AND

|               |       HAVE UNIT SERVICED IF ERROR CONDITION PERSISTS.

|     4400      |   PRINTER ERROR OCCURRED DURING PRINT.

|               |   PRINTER CARD PROBLEM

|     4500      |   DATA PORT ERROR

|     4600      |   STATUS PORT ERROR

|     4700      |   CONTROL PORT ERROR OR NO WRAP PLUG CONNECTED

|     4800      |   PRINTER INTERRUPT FUNCTION ERROR

--------------------------------------------------------------------------

RS232 EXTERNAL WRAP DIAGNOSTIC

MANUFACT.ID = 36XXXX

;       THE DIAGNOSTIC IS ONLY RUN FOR SERVICE AND MANUFACTURING LEVELS (THE  :

;       TEST IS EXACTLY THE SAME REGARDLESS OF WHICH LEVEL IS SELECTED).      :

;                                                                             :

;       *******  IT IS ASSUMED THAT THE RS232 WRAP IS CONNECTED PRIOR TO      :

;                EXECUTING THIS DIAGNOSTIC.                                   :

;                                                                             :

;                           XXXX = 0001  TRANSMITTER HOLDING REGISTER WAS     :

;                                        NOT EMPTY WHEN IT SHOULD HAVE BEEN   :

;                                  0002  RECEIVED DATA AVAILABLE INTR DIDN'T  :

;                                        OCCUR OR THE WRONG 8250 INTR WAS     :

;                                        DETECTED (RECEIVER LINE STATUS OR    :

;                                        MODEM STATUS INTR)                   :

;                                  0003  TRANSMISSION ERROR - THE CHARACTER   :

;                                        RECEIVED WASN'T THE SAME AS THE ONE  :

;                                        TRANSMITTED                          :

;                                  0004  THE DATA SET CONTROL OUTPUTS WERE    :

;                                        NOT THE SAME AS THE DATA SET CONTROL :

;                                        INPUTS                               :

;                                  0005  TRANSMITTER WAS NOT IDLE WHEN IT     :

;                                        SHOULD HAVE BEEN                     :

;                                       *** 1-5 RETURNED FROM WRAP_TEST       :

;                                  0006  BAUD RATE GENERATOR PROBLEM          :

;------------------------------------------------------------------------------

PROCEDURES AND TEST COVERAGE FOR EACH "MODE"

--------------------------------------------

A: BURN IN MODE.

THIS MODE IS DESIGNED AS A BARE PLANAR BOARD TEST TO BE RUN IN

PLANAR BOARD BURN-IN CHAMBER. IT IS BASICALLY A LOOP OF A SUBSET OF

THE POWER-ON DIAGNOSTICS. IT IS INVOKED BY HAVING ALL 4 FIRE BUTTON

INPUTS ON THE JOYSTICK INTERFACE PULLED TO GROUND (PINS J2- AB04,B03 & BB04,B03)

COVERAGE IS IN THE 85-90% RANGE FOR SOLID, SINGLE STUCK FAULTS (FLOATS

AND GROUNDS). NO EXTERNAL WRAP CONNECTORS (EXCEPT FOR THE JOYSTICK GROUNDS)

ARE NEEDED.

1. CHECK 8088 REGISTERS, FLAGS AND CONDITIONAL JUMPS.

2. CHECK PORT A AND B OF 8255.

3. CHECK-SUM SYSTEM ROM MODULES.

4. CHECK LOW 2K OF RAM AND 2K AT THE TOP OF THE FIRST 64K

ADDRESS SPACE USING "AA","55","FF", AND "00" DATA PATTERNS.

ALSO TEST ADDRESSING.

5. CHECK INTERNAL REGISTERS OF THE 8259 INTERRUPT CONTROLLER

AND CHECK FOR HOT INTERRUPTS.

6. CHECK OUT SYSTEM TIMER (8253-ALL 3 CHANNELS)

7. TEST RED, GREEN, BLUE & INTENSITY VIDEO OUTPUTS, CHECK TIMING OF

HORIZONTAL & VERTICAL RETRACE SIGNALS. (TEST 6845 AND CRT GATE ARRAY)

8. TEST REMAINDER OF MEMORY IN THE SAME FASHION THAT THE FIRST 4K

WAS DONE. ALSO PERFORM A REFRESH (DATA RETENTION) TEST.

9. WRAP BACK TEST OF CASSETTE INTERFACE. (INTERNAL WRAP)

10. PERFORM INTERNAL WRAP BACK OF ON-BOARD SERIAL INTERFACE (8250).

B: RUN-IN MODE.

THIS MODE PROVIDES A LOOP OF THE ALL THE DIAGNOSTICS THAT ARE POSSIBLE

WITHOUT MANUAL INTERVENTION. IT ASSUMES THAT ALL SET-UP PROCEDURES HAVE BEEN

DONE (WRAPS PLUGGED IN, DISKETTE IN THE DRIVE WITH HANDLE CLOSED, ETC.)

IT IS INVOKED BY GROUNDING PINS AB03 & BB04,B03 OF THE JOYSTICK INTERFACE.

THE RESISTORS PROVIDED BY THE "SERVICE MODE" PLUG MUST BE IN PLACE ON THE

JOYSTICK INTERFACE, ALSO. IT ASSUMES THAT THE RS232 WRAP CONNECTOR IS IN PLACE

BECAUSE THE DIAGNOSTICS ARE SELF-CONFIGURING, THIS TEST CAN BE USED IN

PLACE OF THE BURN IN MODE IF THE CORRECT WRAPS ARE PROVIDED.