JRXTPWR NOTES:                                            April 19, 1989

Modification to Jr to replace power card with external XT Power Supply

Parts List:

This modification requires you to obtain the following:

XT Power Supply (could use a PC-1 power supply with less wattage)

Edge Connector (to replace the connector on the base of the

PCjr power card)

16 or 18 gauge STRANDED wire (automotive wire is fine)

(use 16 gauge wire if you are putting the XT power supply

more than a few inches away from the back of Jr)

General Notes:

(1) The edge card connector will probably need to be cut out of

some card created for some other purpose.  A VIC20 cartridge

has the proper edge card connector.  The source I used was

Radio Shack (Part # 276-154A which is a prototype card).  In

any case you will need to saw or cut the card to have a fairly

small connector with the edge contacts properly seating in the

PCjr main board socket.

(2) Depending on where you position the XT power supply, you may not

need to extend the disk drive power connectors  -- the XT

connectors can reach the Jr disk drive(s) in some cases.  If

you have to extend, you might want to consider taking the cable

that connects the Jr power card to the internal disk and using

that cable (I cut mine in two parts giving me two extenders to

connect to the XT power supply disk power cables).

(3) All connections from the Jr edge card connector go to either the

P8 or the P9 XT connectors (clearly labeled on the power supply

I obtained).  I used the XT Tech reference plus a voltmeter to

verify which wire was which on these connectors.  Note that my

diagram below lists color codes for this wiring -- your power

supply may differ but the position within the P8 and P9 plugs

will not be different.  I also used the same color wiring for

my extensions to the edge card connector to avoid possible

confusion.

(4) On the Radio Shack edge card connector, I did not depend upon

the rather small printed circuit leads to the edge contacts.

Instead I drilled a small hole at the top of each

contact and connected the power supply wires directly

at that point.

(5) I provided a connection point on the edge card connector for

the Jr fan (+12v) and plan to add a connection for a mouse

also (mine will be +5v but some mice are +12v).  I made the

connectors for the fan plug just by wiring two solid wire

connections to Common and +12v with about 1/4 inch of solid

wire extending from the card which in turn fit the fan power

plug (a paper clip provides a piece of solid wire of the right

size).  On my Junior the black fan wire is Common and the red

wire is +12v.

(6) This modification provides -12v rather than the -6v supplied

by Junior's 7" power card.  This is a good change -- not a

a problem (the 11" Junior power card also supplies -12v).

(7) I cut the edge card connector using a coping saw -- takes some

care but should not be difficult.  Important thing is to cut the

connector so that the contacts are properly lined up with the

contacts in the socket on the main board.  Since the main board

connection takes 10 contacts and the Radio Shack card provided

22 contacts, I had enough edge connector to make a second

attempt if my first edge connector had a problem.

(8) I brought the main power wires inside the Jr by bringing them

through the opening previously occupied by the Power Switch.

The routing of the disk drive power connections may take a

little creativity -- in my case I brought them through an

opening I had previously cut to provide an entrance for the

flat cable for a second diskette drive.  You can bring them

through the same opening as the main board power cable if you

have enough length in the diskette power cable.

(9) The main board connection for the Jr power supply card has contacts

ONLY on the LEFT as you face the front of your Jr.  BE SURE THAT YOU

WIRE FOR THOSE CONTACTS.  On the prototype board that I used, the

contacts on the RIGHT side (as you face the front of Jr) are NOT

connected to the contacts on the LEFT side.  My connections actually

were made to both sides of the edge card connector but only the

contacts on the LEFT side are used by Jr.

(10) I recommend wiring the edge card connector first before you attach

the wiring to the XT Power Supply.  This will permit you to use

resistance checks to make certain that none of the connections on the

edge card connector are shorted or incorrect (you won't be able to

make this test after the connections to the power supply are made).

(11) Check the cables visually, then place the edge card connector on an

insulated surface and turn on the XT power supply.  Check all of the

voltages with a voltmeter BEFORE you connect to Jr and you should

not have any problems.   Don't forget to turn the power off before

making any connections or changes.

(12) Placement of XT Power Supply:  If you are placing the power supply

behind Junior, you might want to build a little "table" about

1 1/2 in. high to provide a space for Junior's cables to go

under the Power Supply.

EDGE CARD CONNECTOR "picture":

This diagram is oversize to show the detail of the

connector.  The actual connector is about 2 inches long by

1.5 inches high.  The portion of the connector that inserts into

the main board socket is about 1 5/8 inches long.  You can see

exactly what this has to look like by pulling the power supply card

out of your Jr and looking at the connector on the power card.

You do not need to duplicate the "key" on the Junior card (the

portion of the edge connector that is outside the main board

socket) although it is OK to do so.

When you have this fabricated, be sure and insert it into the main

board socket and double check that the contacts on the LEFT side

(as you face the front of Jr) are properly lined up.  Also it is a

good idea to draw a line on the edge card connector at this time

that shows where the top edge of the main board socket comes to on

the card -- this will help you determine where you can place your

wiring on the edge card connector.

_____________________________________________________

!                                                   !

!                                      C  +12       !

!                                      o  o         ! <== fan

!                                                   !  connection

-------!----------                                         !

wires  !                (see wiring diagram           JR   !

from   !              for connection detail)        FRONT  !

XT pwr !         ("C" below is for "Common")        =====> !

-------!----                                               !

!     -12v C   C   +5v +5v +5v +5v C   C  +12v      !

!_____                                        ______!

! O   O   O   O   O   O   O   O   O   O ! <= holes drilled

! "   "   "   "   "   "   "   "   "   " !    for connections

! "   "   "   "   "   "   "   "   "   " !

! "   "   "   "   "   "   "   "   "   " !

! "   "   "   "   "   "   "   "   "   " !

-----------------------------------------

10   9   8   7   6   5   4   3   2   1     <== contact numbers)

Note that the PCjr tech reference manual incorrectly

identifies the location of the +12v and the -12v (-6v)

contacts -- the above wiring diagram is correct.

WIRING DIAGRAM FOR EDGE CARD CONNECTOR:

JR Edge Card Connector               Cable                        XT Pwr.Supply

----------------------               -----                        -------------

contact

volts   no.  !                                                  ! conn-pin  volt

+12     1   !o-------------------Yellow-----------------------o!  P8 - 3   +12

C      2   !o-------------------Black------------------------o!  P8 - 6     C

C      3   !o-------------------Black------------------------o!  P8 - 5     C

+5     4   !o-*                                               !

+5     5   !o--*----------------Red--------------------------o!  P9 - 4    +5

+5     6   !o-------------------Red--------------------------o!  P9 - 5    +5

+5     7   !o-------------------Red--------------------------o!  P9 - 6    +5

C      8   !o-------------------Black------------------------o!  P9 - 2     C

C      9   !o-------------------Black------------------------o!  P9 - 1     C

-12    10   !o-------------------Blue-------------------------o!  P8 - 4   -12

Notes on above cable:

(1) Contacts #4 & #5 of Edge Card Connector are both connected to the

same wire from the XT power supply.  This was an arbitrary choice.

I considered wiring the three +5v power supply wires to a "bus" on

the edge card connector and then wiring from the "bus" to the four

+5v Jr contacts.  You might want to consider that possibility.

(2) The XT Power Supply connection numbers are in the form of

plug number - pin number, e.g. P9 - 4 means Plug 9, Pin 4.

(3) P9 - 3 and P8 - 1 & 2 are unused.  If you have something that needs

-5v, it is available on P9 - 3.

(4) Colors shown on wires are those that were used on my XT Power Supply.

(5) I removed the XT Power Supply plugs and made soldered connections

of the power supply wires to my extension wires.  You might be

able to locate a connector that could be wired that would mate with

the power supply plugs -- I didn't attempt to do this.

(6) After making all of the connections (soldering), insulating, and

testing, I wrapped the bundle of wires with a piece of electricians

tape about every three inches to in effect create a single "cable".

You can use common electricians black tape to insulate all of

your connections but a similar tape labeled "insulating tape"

is a better choice if you can locate it.

DISK DRIVE POWER CABLE:

Plug at disk drive                                              XT Power Supply

------------------                                              ---------------

volts   Pin                                                       conn-pin volts

+12     1   !o-----------------Yellow-------------------------o! P11 - 1   +12

C      2   !o-----------------Black--------------------------o! P11 - 2     C

C      3   !o-----------------Black--------------------------o! P11 - 3     C

+5     4   !o-----------------Red----------------------------o! P11 - 4    +5

(be sure you still have +5v on pin 4 and +12v on pin 1)

Connecting Plug Diagrams (K = key on plug)

6 5 4 3 2 1                 6 5 4 3 2 1                  4 3 2 1    (Pins)

____________K__             ______K________              _______K___

! o o o o o o !             ! o o o o o o !              ! o o o o !

!_____________!             !_____________!               \_______/

P9                            P8                      Disk Power

(the key locations are approximate - use a voltmeter to be sure you

know which connector is which and that your power supply matches the

above information)

Final Note:

If you see any errors in this documentation, please let me know as soon

as possible so that I can distribute a correction.  If you have suggestions for

improvements, those will be welcome too.           Happy Junioring!

Chuck Ramsdale      ISLA(CFR)     8-464-1389