* - PCJRSIDE FORUM appended at 00:44:02 on 88/05/23 GMT (by DELISLE at CLTVM2)

* Subject: Upgrading 128K IBM memory sidecar to 256K

* Ref:     Append at 12:07:22 on 88/05/09 GMT (by DELISLE at CLTVMF)

*

*  Just an update on the progress of my 256K sidecar upgrade package.

* So far I have 3 successful upgrades, 2 without any problems of any kind.

* I also have 3 sidecars that are vegetables. They have so many different

* kinds of errors that I am unable to figure out what is wrong with them.

* Because of these problems, I'm holding off on the package. I feel that I

* must fix these sidecars first to understand the problems before sending a

* a set of instructions to someone of unknown ability and having them turn

* their only sidecar into a paper weight.

*

*  I have no doubt that I can eventuality fix all 3 failing units, but at

* this point I don't know just what it will take to do it. The steps listed

* in the above reference are correct as to what must be changed. If anyone

* should try the upgrade based on those steps, My only suggestion would be

* to install sockets first. With chips piggy-backed on to soldered in chips

* like I did, it is extremely difficult to trouble shoot some problems.

*

*  I would be interested in anyone's experience in fixing sidecar problems.

* (Understatement of the year).                    Cheers,

*

* Wayne DeLisle Sr.             8-794-5211                    Charlotte, NC

* ========================================================================

* Date: 3 March 1989, 14:02:46 EST

* From: RPETTIT  at RALVM31

* To:   DELISLE at CLTVMF

* Sec:  IUO

*

* Subject: 64K DRAM Stacking

* Wayne:  I have read your append of 5/9/88 (really keep up with things here).

* This sounds very interesting.  What success have you (or anyone) had in this

* conversion and what precautions should I take.

* Also, I need help in clarifying your instruction #6, 'Select two consecutive

* memory switches in back to set the active segments...'.

* Thanks,

* Dick Pettit  441-6568

* ========================================================================

* Date: 3 March 1989, 4:41pm EST

* From: Wayne DeLisle Sr.         8-794-5211           DELISLE  at CLTVMF

*       Dept. 57S, B202-1FF182

*       Charlotte, N.C. 28257

* To:   RPETTIT  at RALVM31

* Re:   64K DRAM Stacking

*

* Dick,

* The chip stacking works fine. I have several that I have done. The first

* 2 sidecars that I did had a chip failure within 4 weeks. The problem was

* poor solder joints on the chip legs. It is very difficult to get a

* soldering iron between the chips and the other components on the board.

* The others were done by pulling the chips and adding sockets, then

* piggybacking the chips separately and plugging them in. It's more work

* that way, but is a lot easier to trouble shoot. What you have created by

* doing this is the equivalent of 2 sidecars, so you must set 2 memory

* option switches in order for it to work.

*

* Regards,

*

* Wayne

* =========================================================================

* Date: 6 March 1989, 07:00:46 EST

* From: RPETTIT  at RALVM31

* To:   DELISLE at CLTVMF

* Sec:  IUO

*

*

* Subject:   64K DRAM Stacking

*

* Re: Note from you, sent on 03/03 at 16:50:29

* Subject:

* Thanks Wayne, gonna give it a go this week.

*

* Dick Pettit

* ========================================================================

* Date: 10 March 1989, 07:34:52 EST

* From: RPETTIT  at RALVM31

* To:   DELISLE at CLTVMF

* Sec:  IUO

*

* Subject: 384K conversion

* Wayne:

* I installed the extra 64k drams last night, and after fixing 2 badly

* soldered pins (found visually) and realizing that the case cover was metallic

* and shorted to the bent-up pin 4's (fixed by adding a thin plastic sheet

* inside the case), IT WORKS.  POSTed to 384k, made several usable ramdisks,

* so far so good.

* Thanks again for the help.

* Dick Pettit  RTP

* =========================================================================

* Date: 10 March 1989, 7:54am EST

* From: Wayne DeLisle Sr.         8-794-5211           DELISLE  at CLTVMF

*       Dept. 57S, B202-1FF182

*       Charlotte, N.C. 28257

* To:   RPETTIT  at RALVM31

* Re:   384K conversion

*

* Nice job!

* =========================================================================