Do-It-Yourself Panic Button

By Tom Porpiglia

Sooner or later all computer users are likely to

experience

the frustration of system lock-up. You have

painstakingly entered your data, and

suddenly your keyboard doesn't respond.

CTRL-ALT-DEL doesn't even work!  What to

do? You can reach the power switch

on the back. You can turn off your

power strip and reset everything plugged

into it. But wouldn't it be nice to just reset

your jr?

IBM in their emminent wisdom and cost

containment (they probably saved all of a

$1.50) didn't put a reset switch in the

PCjr. But you can do what IBM didn't do for you.

It's a very simple, straight

forward modification that can be

accomplished with little difficulity and

investment.

All that is needed is a momentary push

button, single pole single throw (spst),

normally opened, hole mounted switch, 2 ea.

24 inch pieces of 20 gage wire, a soldering

iron and solder, needle nose pliers,

diagonal cutters, a small screwdriver, and

a drill and appropriate bit.

Start by inserting an old diskette in the

drive and close the door. Disconnect all

of your peripheral cables from the chassis,

remove the top cover, and disconnect the

ribbon cable from the disk drive, noting

the orientation of the colored stripe.

Next select a suitable location for your

switch and carefully drill a hole. I

decided to put mine on the front panel

above the infra-red reciever opening.

Mount the switch securely in the hole, and

solder a wire to each of the terminals.

Twist the wires together to form a twisted

pair cable.

Route the cable to the rear of the chassis,

and then position the chassis so that the

rear of the computer is facing you.

Refer to the drawing below, and locate the

8284 integrated circuit in the left corner

of the chassis, nearest you. Below this IC

you will see the Crystal (XTAL) and three

resistors (R1, R2, R3). R1 and R2 are the

same value, 510 ohms (green, brown, brown)

and one end of this pair (the end facing

the rear of the chassis) is connected to

each other. Solder one wire to this

connection of the resistors.

Next, locate

the capacitor (C1) to the right of the IC.

The leg of the capacitor that faces the

front of the chassis should be maked with a

stripe and is the negative (-) terminal.

The remaining leg of the capacitor should

be facing the rear of the chassis and is

the positive (+) terminal. Solder the

remaining wire to the positive terminal.

This completes the installation of the

reset switch. Re-connect the ribbon cable

to the disk drive properly orienting the

cable, and replace the cover. Re-connect

all cables.

To test your modification, simply boot up

your computer and depress the re-set button

at the DOS A> prompt. Your computer should

start a re-boot, just as if you has pushed

the CTRL - ALT - DEL sequence.

Now when you get into a lock-up situation

all you need do is press the panic switch.

-------------FRONT-----------------

!  disk                           !

!  drive                          !

!                                 !

!                                 !

!                                 !

!                                 !

!  18        10      -            !

!      8284           C1          !

!  1          9      +            !

!                                 !

! xtal R1 R2 R3                   !

!                                 !

------------------REAR-------------

Note: An alternative to installing the above is to simply

partially remove a cartridge in the PCjr cartridge slot.

This will cause the computer to reset. But if you plan

to do this often for some reason, you might be better

off with the installed reset button, since the cartridge

pins may be damaged with extensive use.