The comments and opinions expressed in this document are the

author's and do not represent, in any way, those of the

IBM Corporation.

Do you need to degauss your heads?

Rarely will a head need to be degaussed, if ever!

This is a personal matter for most, but if you suspect

that the diskette drive is misbehaving, erasing data, etc,

then read on.  I suggest you clean the read write heads first,

then and only then demag per these instructions.

As for errors due to "magnetized" heads, it is a process

of elimination, just one more thing to for you to try...

For a marginal head, demagnetizing may well be the trick.

What normally happens, is the heads get a build up of oxide

(particles of media that clings to the head disk contact surface

much like mud does to an auto). The oxide will corrupt the head

disk interface because the debris can distort the read and write

characteristics of the head.  Or worse yet, the debris may fall onto

your data and corrupt it.  So, clean the heads per the

RWCLEAN DOC procedure, and then demagnetize the heads if you still

feel it is necessary.

Many sector not found errors, disk not ready errors,

and some diskette format problems will be caused by a drive that

is disturbed by outside RFI or magnetic sources.  If this is what

you have been experiencing, then move your monitor (this also

includes any external speakers) away from the diskette drive.

6 inches is a good distance to start with.  Some drives require

more distance, and some less.  Other people have successfully

solved this problem by putting aluminum foil or other shielding

(such as a cookie sheet) between the monitor and the PCjr.

This allowed them to keep the monitor on top of the PCjr.

John L. Dickinson at Tucson, (JLDKNSON at TUCVM2) has done

several tests and he concluded: the type of metal or its thickness

doesn't seem to matter much, but does help reduce the problem.

I found an engineering report that verifies John's testing.

The material thickness that doesn't provide enough magnetic

shielding is about as thick as the foil on a gum wrapper.

Very thin.  The report also substantiated his findings on the

type of metal used.

Another item to check, is the temperature and humidity that your

PCjr is operating in.  The operating limits for most IBM Personal

Computers (except the IBM Industrial Computer) is as follows:

Hot & Wet -  85 deg F, 80% Relative Humidity

Hot & Dry - 105 deg F, 8% Relative Humidity

Cold & Wet - 50 deg F, 80% Relative Humidity

Cold & Dry - 50 deg F, 8% Relative Humidity

The above limits are for a non-air conditioned building

environment, with normal winter heating and normal ventilation.

My home is Dehumidified and air conditioned.  The relative humidity

still runs in the 50 to 60 % range!  My PCjr easily heats a well

insulated 8 foot x 12 foot room several degrees F.  If your Junior

is running outside these limits, expect to get lots of strange

inconsistent errors.  If your computer appears to operate without

problems outside of the temperature humidity  limits, you will be

stressing the product, and shortening its useful life.  On the

otherhand, If your computer goes a little berserk when operated

outside of the temperature and humidity limits...don't worry.

Condition your "computer room" to the above limits, and your

computer will once again be working properly.

If you still experience drive problems, then read on.

DEMAGNETIZE THE READ/WRITE HEADS

Get a degausser, you can use one for cassette decks, VCR's,

8 tracks, reel to reel tape machines etc.  If you really want

to do the job, I suggest you get a demagnetizer of 2500 gauss.

This unit is pretty large, so I assume you have the more readily

available, less powerful cassette tape deck type demagnetizer.

1) Move Diskettes, IBM badge, wrist watch, monitor, etc to

a magnetically safe place (several feet from the degausser).

2) Remove the cover of the PCjr.

3) Turn the demagnetizer on at least 1 foot away from the heads,

and then place the demagnetizer very close to the heads.

(A strong degausser may cause a noticeable vibration

of the heads...if the heads vibrate, skip step 4 and go to step 5)

4) Don't move the degausser for the next 15 to 20 seconds.

5) Begin to pull the degausser away from the heads very,

very slowly.  Don't wiggle, just pull evenly away until you have

the degausser 2 - 3 feet away from the heads.  Should take at

least 5 seconds to travel this distance.

6) Turn your degausser off.  If you can't turn it off,

move farther away with it, and then unplug it

7) Replace the cover to the PCjr.

If you allow the heads to come in contact with each other,

they will normally "ring" or stick together.  This is to be

expected...(don't let them ring together, it is not good for them).

It will seem that they are magnetically clinging to each other, but

they are not. Their air bearing surfaces (part that touches the

diskettes) are so smooth and clean, that the air is

forced out, which then acts like a vacuum to "ring" the heads

together, (just like gage blocks for those of you who may have

used them). This is why cardboard was put in your drive while

it was in storage and shipping.

OTHER FIXIT TIPS

These are tips that PCjr owners claim to fix a failing drive.

I have not verified these fixes, but if they work, who am I to

argue with success?

Unplug the cables that attach to the drive, and clean the contacts.

Some folks report that this was their only problem - dirty contacts.

If repeated cleaning of the rails has been helpful only for a while,

it could be that the diskette is not being "grabbed" with sufficient

force to make it spin smoothly.  Very few people have reported this

problem, but here is how they fixed it:  (The little arm coming down

when you close the lever appears to have lost some of its firmness

in clamping down.)  There is a little stub coming down from the rod

to the lever in the front that, by pushing down on a flat plate,

forces the arm down.  Simply put several pieces of masking tape

under where the stub touches the plate. This forces the center ring

down harder, grabs the diskette more firmly and the problem went

away!  After a few months the tape ITSELF got a little squished, so

more tape was added.  Not elegant, but it salvaged the drive!

Another PCjr owner used a .007 inch shim under the stub (a piece

of punched card was used as the shim).

Another person has reported that by oiling the diskette drive motor

resulted in success.  There is a small piece of tape covering a

bushing on the bottom of the motor.  Remove it, oil the motor,

let it run a while (while rusty oil runs out), clean it off and

place a write protect sticker over the hole.

Please send your comments to Slash at RCHVMP3 regarding this procedure.

GOOD LUCK...................... Mike Shirk/Heath, PE.  RCHVMP3(SLASH)

Rochester MN           tie 553-7110

Document History:

V0.8     01-12-89     Added OTHER TIPS section

V0.7      7-20-87     Added Temperature and Humidity conditions

V0.6      6-25-87     Clarified steps 3 and 5

V0.5      4-07-87     Added Engineering Report collaboration

V0.4      1-27-87     Added John Dickinson Comments

V0.3      1-14-87     Added external magnetic problems & solutions

V0.2     11-19-86     Renamed document, added when to degauss heads

V0.0     11-18-86     Released Document