From "EDN" magazine, June 5, 1979, page 84

------------------------------------------

8080 ROUTINE GENERATES CRC CHARACTER

by Fred Gutman

California Microwave, Inc., Sunnyvale, CA

Communications  and magnetic-storage controllers often use cyclic

redundancy checking (CRC) to enhance data-transmission  accuracy;

this method's popularity is due, in part, to its easy implementa-

tion  with shift registers and exclusive-OR gates (EDN,  Sept  5,

1978, pgs 119-123).

Many  software methods can emulate this hardware mechanization.

The more natural software method discussed here directly executes

the division of a message by a generating polynomial. An example,

programmed on the 8080, appears in the figure.

CRC  divides  a  message M(x) of any  length  by  a  generating

polynomial  P(x)  to form a quotient Q(x) and a  remainder  R(x):

M(x)/P(x)=Q(x)+R(x).  R(x) is appended to the message and checked

at  the receiving end of the communication channel or upon  read-

back of magnetic-storage information.

In  these operations,  addition and division are based  on  the

exclusive-OR function without carry;  only bit-by-bit differences

are  important,  not  their arithmetic  sum.   Thus,  while  this

process resembles nonrestoring binary division, its mechanization

is  somewhat  simpler.   Note  that you don't have to  store  the

quotient - the remainder is the only useful part.  This versatile

method  accepts  different generating  polynomials  (table);  you

simply insert them into the routine in the figure.  In each case,

an  R-bit remainder is left in location REM for  transmission  or

checking.  (R is the order of the generating polynomial.)

TABLE

POLYNOMIAL                MASK         HEX FORM

CR16:  x16+x15+x2+1                H=128          8005

SDLC:  x16+x12+x5+1                H=128          1021

CCITT: x16+x15+x10+x6+x5+1         H=128          8461

x16+x15+x13+x7+x4+x2+x+1    H=128          A097

HDLC:  x14+x2+x+1                  H=128          4007

x8+x7+x2+1                  L=128          0185

CR12:  x12+x11+x3+x2+x+1           H=8            180F

BCC:   x8+1                        L=128          0101

CR16 REVERSE: x15+x14+x+1          H=128          4003

CCIT REVERSE: x16+x11+x4+1         H=128          0811

x8+x7+x5+x4+1        L=128          01B1

FIGURE

011F         MESS    DS    1

0120 0000    REM     DW    0

0122         DIVP    LHLD  REM   ;REMAINDER

0125 7C              MOV   A,H

0126 E680            ANI   128   ;Q-BIT MASK SEE TABLE

0128 F5              PUSH  PSW   ;SAVE STATUS

0129 29              DAD   H     ;2 X R(X)

012A 3A1F01          LDA   MESS  ;MESSAGE BIT IN LSB

012D 85              ADD   L

012E 6F              MOV   L,A

012F F1              POP   PSW

0130 CA3B01          JZ    $+11  ;IF Q-BIT IS ZERO

0133 7C      QB      MOV   A,H

0134 EE10            XRI   10H   ;MS HALF OF GEN. POLY

0136 67              MOV   H,A

0137 7D              MOV   A,L

0138 EE21            XRI   21H   ;LS HALF

013A 6F              MOV   L,A

013B 222001          SHLD  REM

013E C9              RET

013F                 END   100H

An   8080  routine  for  generating   a   cyclic-redundancy-check

character leaves that character in location REM.

END

########################################################################################################