Date: 21 November 1981 23:56-EST

From: Greg Heise

To:   Northstar Users

A  recent  conversation  brought to light the  fact  that

North Star has been putting out technical notes to dealers  which

haven't gone out to users.  To rectify that ...

1.   There  is a patch to the N* Z80 board that  improves

bus noise problems.  Here we go.

a.  Remove capacitor C8 and throw it away.

b.   Cut the trace between pin 8 IC 2B and pin 10

IC 3B on the solder side of the board.

c.   Connect #24 AWG wire from pin 8 IC 2B to one

of  the  two  feedthru holes at edge pins 50/100  of  the  board.

Solder both sides of the feedthru.

d.   Do  the  same with pin 10 IC 3B also to  the

edge  connector  pins  50/100.   Make  both  wires  as  short  as

possible.

The problem this fixes is a real one;  I have had  it

happen  to  me.   It apparently occurs whenever you have lots  of

boards installed in your computer.

2.  For hard disk users.  I will  quote this one exactly.

a.  This change corrects a potential problem with

the  phase lock loop.   The phase lock loop can unlock due  to  a

short  set  pulse.   This problem may exists in some Nevada  [the

Century  Data Controller] boards and is attributed  to  component

tolerance stack up.   Symptoms of the problem may be data errors,

ID errors,  CRC errors,  sector not found,  etc.   This condition

will  exist if the set and reset pulses were to occur at the same

time.  This change essentially widens the window in which the set

pulse occurs.

b.   [Follows  some 3 pages of  technical  how-to

documentation,  the  sum of which is:  IC A53 74S00 is changed to

7400]

c.   [Some more stuff which boils down  to:  Make

sure your Nevada board is now marked with Rev M.]

3.  The above change is in addition to the one that North

Star will give free to hard disk owners.   Basically,  there is a

sector encoder in the base of the hard disk drive that is open to

the  air;  dust and dirt can collect in the encoder slits causing

the system to never find a target sector.  North Star will supply

a  fan,  filter,  newer  revised diagnostic disk to  replace  the

HDTEST  program,  and instructions to anyone who can supply  them

with a hard disk serial number.  It's free.

4.  This is an old one, but I thought I's throw it in: If

you are having trouble with your disk drives,  it may not be  the

drives.   Capacitors C5,  C6,  C8, and C9 on the motherboard have

been found to cause oscillation problems in some TI regulators --

just clip out those caps and everything will be beautiful.

5.  If you should ever blow a N* board memory chip, don't

bother finding a perfect replacement: a prime 4116 will work fine

in any N* memory board -- even the 32K and 16K boards!

Greg

##################################################################################################################