*** SMODEX & SMODEM ***

CP/M HAYES-SMARTMODEM PROGRAMME

===============================

Version 3.8

by Walter Blady

*****

FOREWORD

Thi� programm� is a greatly revised version of MODEM7 b�

War� Christense�.  Many additional routines are copyrighted

(1980) b� M.Zeige� an� J.Mill� or are proprietary to

Walter Blady.  Permission is granted to use but NOT sell

these routines.

Man� othe� individual� contribute� t� th� developemen�

o� thi� programme�  bu� mentionin� the� an� thei� contribu-

tion� her� woul� mak� thi� fil� somewha� unmanagable�

Rightfu� credit� ar� documente� i� variou� version� o� MODEM�

M� appologies.

�����������������������������

All features and options marked with an asterisc '*'

apply to SMODEX version only.

*****

Programme features include...

*    - Four always-on status lines.

*    - Four individually selectable screen pages in terminal.

*    - Dump Terminal screen to printer.

- Directory listing also displays space remaining on disk

and total number of sectors for displayed file(s).

- Telephone directories are kept as separate files.

This allows fast editing without the need to recompile

the modem programme.  Also, many directories may be

kept on disk.

- Log in new drive.

- Kill a disk file.

- Terminal mode/Echo mode toggle.

- List device toggle, (terminal mode).

- Send/receive object-code informal file (terminal mode).

- CRC - (cycle redundancy check option).

- Auto line-feed in the Echo mode.

- 2 full-option menus.

- Display number of file sectors in decimal and hex�       during SEND/RECEIVE file.

- XON-XOFF recognition in transfer informal file mode.

- Auto-dial from internal directory to SMARTMODEM.

- Direct dial from programme intiation or from menu.

COMMANDS IN OFF-LINE MODE

*****

WRITE INFORMAL FILE TO DISK - 'WRT'

-----------------------------------

By definition, an 'Informal file' is one which is set-up

for recieving or transmitting data, while in the terminal mode,

without data validity checks.�

When data has been saved to an 'Informal file', the

file must be closed before continuing with other options.

The 'WRT' command writes all remaining buffered data to the

disk and then closes the file.  Information will be lost

if not done.

DELETE INFORMAL FILE - 'DEL'

----------------------------

This command will delete all buffered information saved

to the 'Informal file' and erase the file name.

RETURN TO TERMINAL MODE - 'RET'

-------------------------------

The 'RET' command is used to re-enter the terminal mode

with no data loss.

If information has been saved to an 'Informal file' and

the user re-enters the off-line mode, all saved data will be

lost if a return to the terminal mode is made with the 'T or

E' options. Using the 'RET' allows re-entry to the terminal

mode, preserving all saved information.

DISCONNECT (SMARTMODEM) - 'DSC'

-------------------------------

When Disconnect is initiated, the HAYES default escape

code '+++' is first sent to the modem to re-enter the

command state, then the 'on-hook' command 'ATH' is sent.

This causes the modem to hang up.  The programme then enters

the terminal mode.

AUTO-DIAL (SMARTMODEM) - 'CAL, C'

---------------------------------

The Auto-dial feature allows the user to call a number

automatically from one of several phone directories.

Auto-dial primary option may be used in three different

ways when initiating SMODEX...

eg. Initiate, goto Auto-Dial and wait for directory name.

SMODEX C

^

(primary option)

eg. Initiate and load the default directory 'PHONE.001', then

dial the number associated with the directory letter 'M'.

SMODEX C M

^  �      (directory-letter)

eg. Initiate and dial the given number.

SMODEX C 231-1898

^

(any number)

eg. Initiate, load and display the default directory.

SMODEX C :

^

(colon)

Auto-dial may also be used from the OFF-LINE mode with

'CAL', The commands are exactly the same except the code

'CAL' is used instead of the 'C' primary option.

If the command 'CAL' is used alone, SMODEX will ask for

a directory name.  If the 'colon' is included, the default

directory will be displayed.

When the phone directory is up, any number in

the directory may be dialed by entering a 'directory-letter

and pressing return.  Or any number may be typed in the

command line.  The dial command 'ATD' and the selected

number are automatically sent to the SMARTMODEM.

If you wish to cancel a call while the phone is ringing,

press return. The 'NO CARRIER' message will be sent and

SMARTMODEM will go to an on hook (hang-up) state.  To redial

the same number, give the command 'A/'.  All the standard

HAYES commands may be issued while in the terminal mode.

A second option for dialing a number is from the terminal

mode, using the standard SMARTMODEM dial command.  Simply type

ATD <phone number>

TELEPHONE DIRECTORY

-------------------

Directories may be given any name and kept on any disk,

but there must always be a 'Default directory' with the name

'PHONE.001' on the default drive.  SMODEX automatically loads

this file if a direct dial is called for.

Phone directories may be set-up using the actual directory

model on the last page.

EXPERT MODE - 'X, XPR'

-----------------------------

If the 'X' option is included in the command line when

SMODEX is initiated, it will come up with the menu display

turned off.  The menu may be toggled on and off during run-

time with the 'XPR' option.

DIRECTORY - 'DIR <d: and/or :>'�-------------------------------

This option prints the specified disk directory to the

screen and displays space remaining on disk and the total

number of (128 byte) sectors for the displayed file(s).

Remaining disk space may be displayed alone by typing a colon.

eg: >>DIR : or DIR d::

RE-ENTER CP/M - 'CPM'

---------------------

This option terminates the modem program and returns to

the cp/m operating system.

KILL A DISK FILE - 'K file.name'

--------------------------------

Files may be deleted from the disk without leaving the

modem programme.  CP/M rules apply.

eg. >>K file.name,  >>K *.ASM,  >>K B:file.A?M

DISPLAY OFF-LINE MENU - 'MEN'

----------------------------

Displays off-line menu while in EXPERT mode.

LOG IN NEW DRIVE - 'LOG d'

--------------------------

Changes the defalt drive for subsequent file operations.

No colon is needed.

RESET DISK DRIVES - '^C'

------------------------

Resets the disk parameter blocks and header information.

This command is used if there has been a media change.

PRIMARY AND SECONDARY OPTIONS

DETAILED EXPLANATION

*****

There are eight primary options in SMODEX.

Originate and Answer options are not used since

SMARTMODEM handles these automatically.

A primary option may be included in a command line when

initiating SMODEX, or while in the off-line mode, but only

ONE option is allowed.  The exception is with 'T', 'M' and 'E'.

They may be used as secondary options during 'send' or

receive'.

S... Send a file       R... Receive a file

T... Terminal mode     E    Echo mode

X    Expert mode       C... Auto-dial mode

M    Menu mode         K... Kill a CP/M file

There are twelve secondary options. These options may

be entered in a command line when initiating SMODEX or

while in the off-line mode.

More than one secondary option may be included in a

command line.  If the (CRC) option is used, then a maximum of

six may be included.  'T', 'M' and 'E' are used here as

secondary options.

...S Print to screen/sending  ...R Print to screen/receiving

...V Print send & receive     ...Q Don't print messages (SMODEM only)

...T Return to Terminal mode  ...E Return to Echo mode

�...D Disconnect               ...B Batch files mode

...C (CRC) request	      ...M Return to Menu mode

char.redundancy check

(option in receive only)

SENDING/RECEIVING WITH SMODEX

*****

(Sending/Receiving FORMAL Files)

The definition 'FORMAL' means that all transmitted

data are checked for validity.   There are two data-check

options available. (See information below.)

If a primary option is used in a command line when

initiating SMODEX, it must be the first character following

the name SMODEX, with one space preceeding it.  Secondary

options immediately follow the primary, with no spaces.

Secondary options may be entered in any order.

eg.  SMODEX SQT B:file.name

^

(primary option)

The above command line means:

SMODEX...Initiate SMODEX

S...Send a file

Q...Don't print any system messages on the screen

T...After the file is sent, go to the terminal mode

B:...The file is on drive B:

file.name...The name of the file to be sent

(File names may contain ambiguous references,

according to the rules of CP/M)

The 'B' batch file secondary option may ONLY be used

from the off-line mode.  It may not be included in a command

line during initiation of  SMODEX.

Command lines entered from the off-line mode are

virtually the same, except the word 'SMODEX' is not typed.

eg. >>RRD file.name

^

(primary option)

The above command line means:

R...Receive a file

R...Type the file to screen as it is being received

D...Disconnect the phone after the file is received

file.name...The name of the received file.

(The file.name doesn't have to match the sender's.�            Ambiguous reference in file.name is NOT allowed

with the receive option.)

When sending a batch of files, ambiguous references may

be used according to the rules of CP/M.

eg. >>SBST *.BAS

S...Send files

B...Batch mode

S...Print files to the screen as they're being sent

T...When through, enter the terminal mode

*.BAS...Send all the files on the default disk with the

extention of 'BAS'.

eg. >>SBM file.name1 file.name2...n

S...Send files

B...Batch mode

M...Enter the off-line mode after all files are sent

file.name...The list of files to be sent.

When receiving files in Batch mode, the file names are

NOT specified.  They are sent by the sender.  SMODEX

automatically receives and uses these names on the disk.

eg. >>RBCE

R...Receive files

B...Batch mode

C...Send a (CRC) character redundancy request to the

sender specifying a (CRC) check of data instead

of the default CHECKSUM method.

E...Go to the Echo mode after all files have been

received.

(Sending/Receiving INFORMAL Files)

Files may be sent or received from the Terminal mode.

The classification 'INFORMAL' simply means that data validity

checks are NOT made on transmitted data. (You takes your

chances.)

To save data to an informal file, a file.name must first

be specified in the off-line mode while giving the 'T'

terminal command.  The user is the asked if the received data

is object code.  (All data is either object code or ASCII.)

eg. >>T file.name

Once in terminal mode, any incoming data may be saved to

this file by simply pressing the '^R' receive toggle.  Data

may be intermitantly saved by toggling on or off.  As soon as

all desired data is saved, the file must be closed with the

'WRT' command, from the off-line mode.  Incoming material may

be temporarily stopped then started by issuing the '^S' XOFF'

and '^Q XON' commands.

To send an informal file, the '^T' transmit command is

given from the terminal mode.

eg. ^T

Following this command the user is asked if the file

to be sent is 'object code', then asked for the file.name.

The data is sent immediately following a carriage return, so

care must be taken that the receiver has enough time to

prepare.

OPTION SUMMARY

*****

PRIMARY OPTIONS

===============

SEND A FILE - 'S...'

--------------------

Send CP/M files individually or in batch mode using

CHECKSUMS or CRC checks.

RECEIVE A FILE - 'R...'

-----------------------

Receive CP/M files individually or in batch mode using

CHECKSUMS or CRC checks.

TERMINAL MODE - 'T <file.name>'

-------------------------------

Enter terminal mode (without echo.).  If <file.name> is

specified, then an informal file is set up and data may be

saved to this file using the (Receive) ^R' toggle.

ECHO MODE - 'E'

---------------

Enter echo mode.  Echo all characters back to the

sender.  Auto line feeds are sent following each carriage

return.  Both terminals may NOT be in echo mode at the same

time or SMODEX will go boobs up.

EXPERT MODE - 'X'

-----------------

This command surpresses the menu display when initiating �SMODEX.

AUTO-DIAL (SMARTMODEM) - 'C <command>'

--------------------------------------

Dial a number directly or display the telephone directory

and wait for command.

SECONDARY OPTIONS

=================

SHOW FILE AS SENT - 'S'

-----------------------

Selecting this option will display the file being sent,

on the screen.

SHOW FILE AS RECEIVED - 'R'

---------------------------

This option will display the file being received, on

the screen.

SHOW FILE - 'V'

---------------

Display sent or received file on the screen.

QUIET - 'Q' (Option in SMODEM only)

-----------------------------------

Do not display system messages at output device.

Primarily used with printing terminals.

TERMINAL MODE - 'T'

-------------------

Enter the terminal mode.  <file.name> may not be

specified when when using 'T' as a secondary option.

DISCONNECT (SMARTMODEM) - 'D'

-----------------------------

Issue the 'Escape' code to the HAYES SMARTMODEM, then

hang up the telephone and re-enter the off-line mode.

BATCH MODE - 'B'

----------------

This option is used to send multiple files.

File names can be individually specified in the command

line, or they can be specified using ambiguous file names

(CP/M convention).  Individual file names are sent to the

receiver first, then the file contents, until all files are �sent.

(CRC) cycle redundancy check - 'C'

----------------------------------

SMODEX defaults to a 'checksum' method of checking data

validity during send/receive.  Although quite accurate, it

is not virtually error free.  The secondary option 'C' may

be included in the command line to force a (CRC) check of

data. The (CRC) methode is 99.99% error proof.  The only

restrictions are:

- Both sender and receiver must use one of the MODEM series

programmes that have the (CRC) option.

- The (CRC) option may only be included in a RECEIVE

command line.

If SMODEX detects a request for (CRC) during a

SEND file, it automatically switches over and informs the

operater.

TERMINAL MODE OPTIONS

*****

* DUMP SCREEN TO PRINTER - ^U

-----------------------------

Dump displayed page to the CP/M list device.

* SELECT TERMINAL PAGE - ^Y (1 - 4)

-----------------------------------

Four selectable terminal screen pages may be individually

displayed.  Screen information is restored when switching

between pages.

TERMINAL MODE MENU - ^I

-----------------------

This command will clear the screen and display the

terminal mode options.

OFF-LINE - ^O

-------------

This command will allow re-entry to the off-line mode

without disconnecting the phone.

END OF FILE - ^Z

----------------

This command sends an end of file ASCII character.

XOFF - ^S

---------

The XOFF character tells the sending computer to stop

data transmission temporarily.  (The sender must recognize

XON/XOFF.)

XON - ^Q

--------

The XON character tells the sending computer to resume

data transmission.

PRINTER (TOGGLE) - ^P

---------------------

All incoming data may be sent to the printer while in

terminal mode. This feature also operates while receiving

a file using the 'R' (receive) primary option.

TRANSMIT INFORMAL FILE - ^T

---------------------------

This command will send a disk file informally (without

error checking).

After pressing ^T, the user is prompted for the

file.name, and if the file being sent is object code.

The file is immediately sent following a carriage return.

The transmission can be terminated with the cancel key ^X.

OBJECT CODE (SCREEN PROMT)

--------------------------

Object code files may be SAVED or SENT while in the

terminal mode.  The user is prompted during informal file

transfer/receive as to whether an object code file is being

transmitted.  There are no checks to ensure data validity,

therefore this feature should only be used when it is not

possible to use the 'R' (receive) or 'S' (send) primary

options.

RECEIVE INFORMAL FILE (TOGGLE) - ^R

-----------------------------------

The receive toggle may be used only if a <file.name> was

specified when entering the terminal mode.

eg.  T <file.name>

Alternately pressing 'R' turns the save feature ON and

OFF.  When the save function is ON, data is simultainiously

saved and printed on the screen, and each printed line is

preceded by a colon ':' (The colon is not sent to the file.)

Information may be saved and sent to the list device at the

same time.

ECHO MODE (TOGGLE) - ^E

-----------------------

Alternately pressing this key toggles between the

terminal mode and echo mode.

DISCONNECT (SMARTMODEM) - ^D�----------------------------

This key sends the SMARTMODEM default escape code '+++'

then issues the command 'ATH' causing the modem to go to

an ON-HOOK state (hang up).

CANCEL SEND/RECEIVE - ^X

------------------------

This key may be used to cancel files being sent or

received in the informal mode, or files being sent using the

primary option 'S'.  Files being received with the primary

option 'R' can't be cancelled by the receiver.  This ensures

that line noise doesn't inadvertantly stop transmission.

PHONE DIRECTORY

*****

The directory model below may be saved to disk and used

as the an actual directory.

The directory itself begins with the letter 'A' and

ends with the'$'.  'A' MUST BE THE FIRST CHARACTER IN THE

FILE. (No CR/LF pairs preceding it.)  Do not include the

spacing guide below.

(spacing guide)

12345678901234567890123456789012345678901234567890123456789012++

A Apple CBBS..........499-2908  B Tony Bagshaw........881-1532

C Conn TRS Pick.......839-4561  D Conn TRS Miss.......266-9110

E Conn TRS North Y....667-9981  F Frank Aylesworth....633-7497

G Bill Groskuth IRN...421-1922  H Heathkit BBS .......273-3011

I                               J K.Jeejeebhoy .......924-1517

K Microm CBBS.........226-7430  L Les Popelyak .......495-2086

M Missisaga CBBS......826-5394  N

O                               P P.M.I.CBBS(Punter)..624-5431

Q Queens BBSNY..1-212-441-3755  R Robert L. ..........621-8788

S Sorcerer BBS..1-313-535-9186  T Tony Loutenbach.....223-9238

U                               V

W Walter Blady........231-1898  X

Y                               Z Good time Zelda.....123-4567

$  	                       ^                              ^

^			       |			      |

|                      (two spaces)          (carriage return

|                                             line feed pair

(the $ MUST be the last character)        not seen, but there)

*****

CAUTION:

--------

All NUMBERS and COMAS in the directory line will be sent

to SMARTMODEM.  Only the numbers and comas to be dialed

should be present.  Each coma causes a one second delay in

the HAYES SMARTMODEM.

CONCLUSION

==========

This programme does not include some of the options

that were present in other versions:

- Quiet mode (SMODEX only)

- Baud rate change

- Originate mode

- Answer mode

- Parity select

- Auto dial source code for the PMMI modem

The Baud rate and parity select options may be included

in a future version.  Auto-dial, Originate and Answer

are built-in features of SMARTMODEM, and were not needed.

SMODEX must be assembled with Microsoft's M80 Macro

assembler.  The assembly commands to create a CP/M COM file

are:

M80 =SMODEX�     L80 /P:100,SMODEX,SMODEX/N/E

If anyone adds features to this programme, or has any

comments or suggestions, I would appreciate hearing them.

W.B. 1-(416) 231-1898.

17 Bermuda Ave.

Toronto, Ontario

Canada

M8Y 2P6

Or leave a message on Mississaga Can. RIBBS 1-(416) 826-5394.

END

###################