<!DOCTYPE HTML PUBLIC "-//W3C//DTD HTML 4.0 Transitional//EN"><!-- saved from url=(0075)http://www.hartetechnologies.com/manuals/Cromemco/CP_M%208-inch/CROBIOS.ASM -->

PAGE	60

TITLE	'Cromemco 4FDC/16FDC BIOS in 8080 Code'

;

; * * * * * * * * * * * * * * * * * * * * * * * * * * * * * *

;

;	modifications by Mike Goetz and Hank Kee - 9/2/81

;

;	corrected PIP A:=B:filename.ext on Persci 277/299

;		Persci's must be configured as A: and B:

;		and/or C: and D: since they have but one

;		arm to access two drives

;	modified sign-on message to properly generate size

;		from MEMSIZE definition during assembly

;	corrected current drive assignment on warm boot

;	inclusion of LIST serial driver for CCS 8250 (30 CPS)

;		option to support LA34/LA36

;

; * * * * * * * * * * * * * * * * * * * * * * * * * * * * * *

;

;

; This version modified for 4 drives, A, B, C, D, 2-8" and 2-5"

; In this first version, the boot drive is "A:" only, the 1st 8"

;

; MISC EQUATES

;

PERSCI	EQU	0		;ONE IF PERSCI DRIVE

LARGESW	EQU	1 OR PERSCI	;ONE IF MAXI DRIVE

NBRDRV	EQU	2		;ONE TO FOUR (MUST BE SAME TYPE)

BIGIO	EQU	1		;ZERO IF NO LIST, NO PUNCH

SERIAL	EQU	1		;SERIAL PRINTER OPTION W/8250

;

VERS	EQU	22		;VERSION 2.2

MEMSIZ	EQU	48		;48K SYSTEM

MEMT	EQU	MEMSIZ*1024

MEMONES:EQU	MEMSIZ-(MEMSIZ/10*10)

MEMTENS:EQU	MEMSIZ/10

;

; THE FOLLOWING PARAMETER VALUES FOR VERSION 2.X (INCL. 2.2)

;

BOOTSIZ	EQU	1*128

CCPSIZ	EQU	16*128

BDOSIZ	EQU	28*128

BIOSIZ	EQU	12*128		;7 ON DISK, EXCESS UNINITIALIZED RAM

STACK	EQU	MEMT

;

BIOS	EQU	MEMT-BIOSIZ

BDOS	EQU	BIOS-BDOSIZ

CCP	EQU	BDOS-CCPSIZ

BASE	EQU	0		;BASE OF USABLE RAM

IOBYTE	EQU	BASE+3		;I/O DEVICE ASSIGNMENT BYTE

LOGDSK	EQU	BASE+4		;WHERE CPM STORES DEFAULT DRIVE

FCB	EQU	BASE+5CH	;DEFAULT FILE CONTROL BLOCK

BUFF	EQU	BASE+80H	;DEFAULT DISK BUFFER

TPA	EQU	BASE+100H	;BASE OF TRANSIENT PGM AREA

;

; LOCATIONS AT SYSGEN TIME

;

SYSGEN	EQU	900H

BOOTGEN	EQU	SYSGEN

CCPGEN	EQU	BOOTGEN+BOOTSIZ

BDOSGEN	EQU	CCPGEN+CCPSIZ

BIOSGEN	EQU	BDOSGEN+BDOSIZ

BIOSEND	EQU	BOOTGEN+(52*128)-1

OFFSET	EQU	CCPGEN-CCP

;

;	CROMEMCO 4FDC I/O ASSIGNMENTS

;

CSTAT	EQU	00H		;3P + S, 4FDC, TU-UART, OR SCC

CONDATA	EQU	01H

CIMODE	EQU	02H		;(3P + S DOESNT HAVE THESE)

CIMASK	EQU	03H

PARPORT	EQU	04H		;NOT USED

;

FDCSTAT	EQU	30H		;4FDC OR CCS 2422 BOARD

FDCTRK	EQU	31H

FDCSECT	EQU	32H

FDCDATA	EQU	33H

FDCFLAG	EQU	34H

;

MBANKPT	EQU	40H		;MEMORY BANKING PORT

;

;	1771/179X EQUATES

;

HDLOAD	EQU	08H

VERIFY	EQU	04H

;

IF	BIGIO = 1

;

;	OTHER DEFINITIONS (OPTIONAL)

;

BAUDRATE	EQU	01H		;110 BAUD FOR READER ?

RFDCSTAT	EQU	10H

RBAUDPORT	EQU	RFDCSTAT	;TU-UART BOARD FOR PAPER TAPE

RFDCDATA	EQU	RFDCSTAT+1

PFDCSTAT	EQU	10H

PBAUDPORT	EQU	PFDCSTAT	;PAPER TAPE PUNCH ALSO

PFDCDATA	EQU	PFDCSTAT+1

LFDCSTAT	EQU	54H		;PARALLEL PRINTER POARD

LFDCDATA	EQU	LFDCSTAT

ENDIF

TRUE	EQU	-1

FALSE	EQU	0

NULLS	EQU	6		; NUMBER OF NULLS AFTER LF

LF	EQU	0AH		; LINE FEED

SDATA	EQU	20H		; DATA PORT

SINTEN	EQU	21H		; INTERRUPT REGISTER

SIDENT	EQU	22H		; INTERRUPT IDENTIFICATION

SLCTRL	EQU	23H		; LINE CONTROL REGISTER

SMDMCT	EQU	24H		; MODEM CONTROL REGISTER

SLSTAT	EQU	25H		; LINE STATUS REGISTER

SMDMST	EQU	26H		; MODEM STATUS REGISTER

SSTATP	EQU	25H		; SERIAL PRINTER STATUS PORT (INPUT)

STBE	EQU	20H		; SERIAL PRINTER TBE BIT

;

ORG	BIOS

;

;	CPM JUMP VECTOR ENTRY POINTS

;

JMP	CBOOT

JMP	WBOOT

JMP	CONSTAT

JMP	CONIN

JMP	CONOUT

JMP	LIST

JMP	PUNCH

JMP	READER

JMP	HOME

JMP	SELDSK

JMP	SETTRK

JMP	SETSEC

JMP	SETDMA

JMP	READ

JMP	WRITE

JMP	LISTAT

JMP	SECTRAN

;

;	CP/M 2.2 DISK CONTROL BLOCKS

;

DPBASE	DW	XLT8

DW	0000

DW	0000

DW	0000

DW	DIRBUF

DW	SD8PBLK

DW	CSV0

DW	ALV0

;

IF	NBRDRV > 1

DW	XLT8

DW	0000

DW	0000

DW	0000

DW	DIRBUF

DW	SD8PBLK

DW	CSV1

DW	ALV1

ENDIF

;

IF	NBRDRV > 2

DW	XLT8

DW	0000

DW	0000

DW	0000

DW	DIRBUF

DW	SD8PBLK

DW	CSV2

DW	ALV2

ENDIF

;

IF	NBRDRV > 3

DW	XLT8

DW	0000

DW	0000

DW	0000

DW	DIRBUF

DW	SD8PBLK

DW	CSV3

DW	ALV3

ENDIF

;

;	disk parameter blockS, common to all disks

;	this implements a "standard" CP/M directory

;	with 64 entries, and 1K blocks.

;

SD8PBLK	DW	26		;sectors per track

DB	3		;block shift factor

DB	7		;block mask

DB	0		;null mask

DW	242		;disk size-1

DW	63		;directory max

DW	00C0H		;INITIAL FREE SPACE MAP

DW	16		;check size

DW	2		;track offset

;

;	this implements the cdos standard 5" single density

;	18 sectors (128 bytes) per track, 40 tracks, 3 reserved

;	sectors, 83k of storage

;

SD5PBLK	DW	18		;sectors per track

DB	3		;block shift factor

DB	7		;block mask

DB	0		;null mask

DW	82		;disk size-1

DW	63		;directory max

DW	00C0H		;INITIAL FREE SPACE MAP

DW	16		;check size

DW	3		;track offset

;

;	sector translate vector

;

XLT8	DB	1,7,13,19,25,5,11,17,23,3,9,15,21

DB	2,8,14,20,26,6,12,18,24,4,10,16,22

;

XLT5	DB	1,6,11,16,3,8,13,18,5

DB	10,15,2,7,12,17,4,9,14

;

SIGNON	DB	0DH,0AH,0AH

DB	MEMTENS+30H

DB	MEMONES+30H

DB	'k CP/M version 2.2'

DB	0DH,0AH,0

;

SELDSK:	LXI	H,0

MOV	A,C		;C CONTAINS REQUESTED DRIVE NO.

CPI	NBRDRV

RNC			;IGNORE IF TOO HIGH

STA	DKNUMB

MOV	L,C		;L=disk number 0,1,2,3

DAD	H		;*2

DAD	H		;*4

DAD	H

DAD	H		;*16 (size of each header)

LXI	D,DPBASE

DAD	D		;HL=.dpbase(diskno*16)

RET

;

;TRANSLATE SECTOR IN C USING TABLE AT DE

;

SECTRAN:MVI	B,0

XCHG			;TABLE ADDR TO HL

DAD	B		;GET ADDRESS

MOV	L,M		;GET BYTE

MVI	H,0		;ANSWER IN HL

RET

;

SETSEC:	MOV	A,C		;JUST SAVE SECTOR NUMBER

STA	DKSECT

RET

;

SETDMA:	PUSH	H

MOV	L,C

MOV	H,B

SHLD	DKDMA

POP	H

RET

;

;	ERROR CHECKING READ AND WRITE RTNS FOR

;	CROMEMCO CBIOS

;

READ:	CALL	CLEAR

RTYRD:	CALL	RD4FDC		;READ SECTOR

RZ			;SUCCESSFUL READ. RETURN

CALL	ERROR		;INCREMENT RTYCNT

JNZ	RTYRD		;RETRY 20 TIMES

ORI	01H		;CP/M CONVENTION FOR PERMANENT ERROR

RET

;

CLEAR:	XRA	A

STA	TRKERCT	;ZERO OUT TRACK ERROR COUNTER

STA	RTYCNT	;ZERO OUT CRC ERROR COUNTER

RET

;

WRITE:	CALL	CLEAR

RTYWRT:	CALL	WR4FDC	;WRITE SECTOR

RZ			;SUCCESSFUL WRITE. RETURN

CALL	ERROR		;INCREMENT RTYCNT

JNZ	RTYWRT		;RETRY 20 TIMES

ORI	01H		;CP/M CONVENTION FOR PERMANENT ERROR

RET

;

ERROR:	PUSH	H

ANI	10H		;CHECK FOR NRF

JNZ	TRKERR

LXI	H,RTYCNT

INR	M		;INCREMENT RTYCNT

MOV	A,M

POP	H

SUI	20		;20 TRIES?

RET

;

TRKERR:	LXI	H,TRKERCT

INR	M		;INCREMENT NO OF TRACK ERRORS

MOV	A,M

SUI	10		;ALLOW ONLY 10 TRACK ERRORS

POP	H

RZ			;IF >10, RETURN A FAILURE

PUSH	B

CALL	HOME		;HOME THE HEAD

LDA	DKTRACK

MOV	C,A		;GET TRACK IN C

CALL	SETTRK		;RESEEK TO CORRECT TRACK

POP	B

ORI	0FFH		;RETRY

RET

;

;	RESTORE THE DISK TO TRACK ZERO

;

HOME:	SUB	A		;ZERO OUT TRACK COUNTER

STA	DKTRACK

CALL	DSKSEL		;NOW SELECT THE DISK

OUT	FDCFLAG

MVI	A,02H+HDLOAD+VERIFY	;USE SLOW STEPPING SPEED FOR ALL DISKS

OUT	FDCSTAT

RSTI:	IN	FDCFLAG		;NOW CHECK STATUS

RAR

JNC	RSTI		;LOOP BACK UNTIL DONE

JMP	SEEKTST

;

;	HERE, ACTUALLY DO THE READ OPERATION

;

RD4FDC:	PUSH	B

PUSH	H		;FIRST, SAVE REGS

PUSH	D

MVI	E,88H		;READ COMMAND (VALID FOR 1771, 179X)

CALL	INIT4FDC

RDI1:	IN	FDCFLAG		;NOW CHECK FLAGS

RAR

JC	RDI3		;IF PREMATURELY DONE, STOP

;	INI

IN	FDCDATA

MOV	M,A

INX	H

DCR	B

;

JNZ	RDI1		;READ ANOTHER BYTE INTO CORE UNTIL DONE

RDI2:	IN	FDCFLAG		;CHECK FLAGS

RAR

JNC	RDI2		;LOOP UNTIL READY

RDI3:	IN	FDCSTAT	;NOW CHECK STATUS

ANI	9CH

RDWREND:EI

POP	D		;RESTORE REGS

POP	H

POP	B

RET

;

;	ACTUALLY DO WRITE OPERATION

;

WR4FDC:	PUSH	B

PUSH	H		;SAVE REGS

PUSH	D

MVI	E,0A8H		;WRITE COMMAND (VALID FOR 1771, 179X)

CALL	INIT4FDC

WRI1:	IN	FDCFLAG		;CHECK FLAGS

RAR

JC	WRI3		;IF PREMATURELY DONE, STOP

;	OUTI

MOV	A,M

OUT	FDCDATA

INX	H

DCR	B

JNZ	WRI1		;WRITE DATA FROM MEMORY TIL DONE

WRI2:	IN	FDCFLAG		;CHECK FLAGS

RAR

JNC	WRI2		;LOOP TIL DONE WITH WHOLE OPERATION

WRI3:	IN	FDCSTAT	;NOW CHECK ERROR STATUS

ANI	0FCH

JMP	RDWREND

;

;	SET THE TRACK, AND MOVE DISK ARM THERE

;

SETTRK:	MOV	A,C

STA	DKTRACK		;STORE THE TRACK NUMBER

SUB	A

CALL	DSKSEL		;NOW SELECT THE DISK

OUT	FDCFLAG

MOV	A,C

OUT	FDCDATA		;TELL 1771 ABOUT TRACK WANTED

LDA	DKSECT

OUT	FDCSECT	;TELL IT ABOUT SECTOR WANTED

PUSH	H

PUSH	D

LXI	H,DRVTBL	;NOW, SEE WHERE DISK ARM IS NOW

LDA	DKNUMB

MOV	E,A		;LOOK UP IN TABLE

MVI	D,0

DAD	D		;GET BYTE

MOV	A,M

OUT	FDCTRK	;TELL 1771 WHERE ARM IS NOW

MOV	A,M

SUB	C

POP	D		;NOW CAN RESTORE THE REGS

POP	H

RZ			;IF ALREADY AT THAT TRACK, QUIT

MVI	A,12H-LARGESW*2+HDLOAD+VERIFY

OUT	FDCSTAT	;PERFORM THE SEEK THAT IS NEEDED

SKI:	IN	FDCFLAG		;CHECK FLAGS

RAR

JNC	SKI		;LOOP UNITL OPERATION DONE

SEEKTST:IN	FDCSTAT	;NOW CHECK ERROR STATUS

ANI	98H

RNZ			;ZERO IS ALL OK

PUSH	D

MVI	D,0		;UPDATE TRACK TABLE

LDA	DKNUMB

MOV	E,A		;WITH NEW POSITION

;

IF	PERSCI = 1

CALL	ADDSHFT		;PERSCI DRIVES HAVE ONLY ONE ARM

MOV	A,E

XRI	01H

MOV	E,A

CALL	ADDSHFT

POP	D

RET

ENDIF

;

ADDSHFT:			;IF NOT PERSCI, FALL THRU HERE

PUSH	H

LXI	H,DRVTBL	;GET CORRECT ADDRESS OF DISK

DAD	D

LDA	DKTRACK		;AND GET CURRENT TRACK NUMBER

MOV	M,A

SUB	A		;PUT IT IN

POP	H

;

IF	PERSCI = 0

POP	D		;AND MISC CLEANUP

ENDIF

;

RET

;

;	INITIALIZE THE 4FDC, BY SELECTING THE DISK AND

;	TURNING ON THE DISK MOTORS, AND INITIALIZE THE

;	REGISTERS FOR THE I/O OPERATION

;

INIT4FDC:

MVI	A,80H		;FIRST, SELECT THE DISK

CALL	DSKSEL

LHLD	DKDMA		;INITIALIZE HL AND BC REGS FOR I/O

LXI	B,8000H+FDCDATA	;80H IS 128 BYTE SECTORS

MOV	D,A		;SAVE THE A REG

DI			;DISABLE INTERRUPTS

LDA	DKSECT

OUT	FDCSECT	;SET THE SECTOR WANTED

IN	FDCFLAG

CMA			;SEE IF HEAD IS LOADED

ANI	20H

JZ	IN0

MVI	A,04H		;THIS IS THE HEAD LOAD BIT

IN0:	ADD	E		;ADD HEAD LOAD BIT TO COMMAND

MOV	E,A

MOV	A,D		;RESTORE THE A REG

OUT	FDCFLAG

MOV	A,E

OUT	FDCSTAT	;OUTPUT THE COMMAND

RET

;

;	FIGURE OUT WHICH DISK TO SELECT AND PUT BIT THERE

;

DSKSEL:	PUSH	B		;FIRST, SAVE THIS, ITS NEEDED

MOV	C,A

LDA	DKNUMB	;GET DISK NUMBER 0-3

MOV	B,A

INR	B		;ADD ONE

SUB	A

STC			;SET A BIT IN TO BE SHIFTED

SHFTBIT:RAL			;NOW, ROTATE BITS OVER

;	DJNZ	SHFTBIT

DCR	B

JNZ	SHFTBIT

;

MOV	B,A		;SAVE THE NEW VALUE

XRA	A

ORI	20H+LARGESW*10H ;CONDITION 4FDC FOR MOTOR ON AND MAXI

ORA	B

ORA	C		;OR IN DRIVE SELECT AND COMMAND WANTED

POP	B

RET

;

;	GETS CONTROL ON A WARM START

;

WBOOT:

MVI	A,01H		;FIRST, RESTORE DEFAULT BANKING

OUT	MBANKPT

;

LXI	SP,80H

LDA	DKNUMB

STA	CURDRV		;STORE SELECTED DRIVE

;

STARTBOOT:

MVI	C,0

CALL	SELDSK		;SELECT DRIVE A TO REBOOT

CALL	HOME

LXI	H,CCP-128	;WILL INCREMENT BY 128 LATER

SHLD	DKDMA

LXI	B,44*256+1	;SECTOR COUNT, FIRST SECTOR-1

RDSEC:	MOV	A,C		;C IS SECTOR NUMBER

CPI	18+LARGESW*8

JZ	NXTTRK

LXI	D,128		;INCREMENT DMA ADDRESS

LHLD	DKDMA

DAD	D

SHLD	DKDMA		;STORE DMA ADDRESS

INR	C		;INCREMENT SECTOR NUMBER

CALL	SETSEC

CALL	READ		;READ THE DATA

JNZ	WBOOT		;ON READ FAILURE TRY AGAIN

;	DJNZ	RDSEC

DCR	B

JNZ	RDSEC

JMP	BOOT		;READ CCP AND BDOS;

NXTTRK:	LDA	DKTRACK

CPI	2-LARGESW

JZ	BOOT		;STOP AT TRACK 2

MOV	C,A

INR	C		;SEEK NEXT TRACK IF NOT

CALL	SETTRK

MVI	C,0

JMP	RDSEC

;

; GET HERE AFTER A COLD BOOT

;

CBOOT:	LXI	SP,80H		;INITIALIZE STACK FOR ROUTINE

LXI	H,SIGNON

CALL	PRMSG		;PRINT MESSAGE

BOOT:	DI

MVI	A,0C3H		;SET UP PARAMETERS ON PAGE 0

STA	0

LXI	H,BIOS+3

SHLD	1

SHLD	7*8+1

STA	5

LXI	H,BDOS+6

SHLD	6

;

;		CROMEMCO INITIALIZATION HERE (READER, LIST NOT NEEDED)

;		NOTE .. THE CONSOLE TUART IS INITIALIZED BY RDOS

;

IF	BIGIO = 1

MVI	A,BAUDRATE

OUT	PBAUDPORT

ENDIF

;

IF	SERIAL = 1

CALL	L2INIT

ENDIF

;

LXI	B,80H		;SET DEFAULT DMA ADDR

CALL	SETDMA

;

LDA	CURDRV		;RESELECT THE DRIVE THAT WAS ACTIVE

MOV	C,A		;  BEFORE THE WARM START

EI

JMP	CCP

;

;

ERRMSG:

DB	13,10,'?? ERROR ??',13,10,0

;

;PRINT MESSAGE AT H,L UNTIL 0

;

PRMSG:	MOV	A,M

ORA	A		;ZERO?

RZ

MOV	C,A		;GO PRINT CHAR

CALL	CONOUT

INX	H		;GET NEXT CHAR

JMP	PRMSG

;

;

;HARDWARE UART CONSOLE ROUTINES

;

CONSTAT:

IN	CSTAT	;CHECK CONSOLE STATUS

ANI	40H

RZ			;ZERO MEANS NO INPUT BYTE READY

MVI	A,0FFH

RET			;FF MEANS INPUT

;

CONIN:

CALL	CONSTAT		;CHECK STATUS TIL GOT A BYTE

JZ	CONIN

IN	CONDATA	;READ THE BYTE AND KILL OFF 80H

ANI	7FH

;

;	THE FOLLOWING CODE REPLACES THE DEL KEY WITH A CTRL-U,

;	SO THAT DEL MEANS LINE DELETE. USE BACKSPACE TO DELETE

;	A SINGLE CHARACTER.

;

;	CPI	127		; DEL

;	RNZ

;	SUI	127-21		; CTRL-U

RET

;

;	CONSOLE OUTPUT ROUTINE

;

CONOUT:

IN	CSTAT	;FIRST, LOOP UNITL TRANSMITTER BUFFER

ANI	80H		;IS EMPTY

JZ	CONOUT

MOV	A,C		;NOW, OUTPUT CHARACTER TO CONSOLE

OUT	CONDATA

RET

;

;	LIST STATUS CHECK FOR CP/M 2.2

;

LISTAT:

IF	BIGIO = 1

IN	LFDCSTAT	;IF LIST TRANSMITTER IS BUSY,

CMA			;THEN RETURN WITH ZERO

ANI	20H

RZ

ORI	0FFH		;LIST TRANSMITTER BUFFER IS EMPTY

RET

ENDIF

;

LIST:

IF BIGIO = 1 AND SERIAL = 0

CALL	LISTAT		;CHECK IF PRINTER BUSY

JZ	LIST

MOV	A,C		;NOW OUTPUT CHARACTER

;	SET	7,A

ORI	80H

OUT	LFDCDATA,A	;WITH HIGH STROBE

;	RES	7,A

ANI	7FH

OUT	LFDCDATA,A	;NOW LOW STROBE

;	SET	7,A

ORI	80H

OUT	LFDCDATA,A	;NOW HIGH STROBE AGAIN

RET

ENDIF

;

IF	SERIAL = 1

MOV	A,C

JMP	L2OUT

ENDIF

;

PUNCH:

IF	BIGIO = 1

IN	PFDCSTAT	;CHECK IF PUNCH BUFFER EMPTY

ANI	80H

JZ	PUNCH		;LOOP UNTIL READY

MOV	A,C

OUT	PFDCDATA	;OUTPUT CHARACTER

RET

ENDIF

;

READER:

IF	BIGIO = 1

IN	RFDCSTAT	;SEE IF READER BUFFER FULL

ANI	40H

JZ	READER		;LOOP UNTIL FULL

IN	RFDCDATA	;AND READ IT

RET

ENDIF

;

IF	BIGIO NE 1

XRA	A,A		;DUMMY ROUTINE FOR LIST,PUNCH,READER

RET

ENDIF

;

;  Serial Printer Initialization Routine

;

L2INIT:	MVI	A,0FH		; SETUP THE 8250

OUT	SMDMCT

MVI	A,83H		; SET DIVISOR REGISTER ACCESS

OUT	SLCTRL

MVI	A,80H		; SET THE DIVISOR TO 384=300 BAUD

OUT	SDATA

MVI	A,01H		;

OUT	SINTEN

MVI	A,03H		; SET DATA REGISTER ACCESS

OUT	SLCTRL

MVI	A,0H		; DISABLE INTERRUPT

OUT	SINTEN

OUT	SLSTAT		; RESET ERROR FLAGS

RET

;

;  Get Serial Printer Output Status

;  Upon Exit:	A = -1 (FFH) and Z-flag is reset if ready for char.

;		A = 0 and Z-flag is set if not ready for character

;

L2RDY:	IN	SSTATP	; GET LIST-OUT STATUS

ANI	STBE		; CHECK PRINTER TBE FLAG

RZ			; PRINTER NOT READY FOR CHARACTER

MVI	A,-1		; PRINTER READY FOR CHARACTER

RET

;

;  Serial Printer Output Routine

;  Upon Entry:	A contains the character to be output

;

L2OUT:	PUSH	PSW		; SAVE CHARACTER FOR A MOMENT

L2OT30:	CALL	L2RDY		; GET LIST-OUT STATUS

JZ	L2OT30		; ZERO MEANS PRINTER BUSY

POP	PSW		; RESTORE CHARACTER

OUT	SDATA		; OUTPUT THE CHARACTER

CPI	LF		; CHECK FOR END OF LINE

RNZ			; RETURN IF NOT LINE FEED CHARACTER

MVI	A,NULLS+1	; IF LF, GET NUMBER OF NULLS

L2RTN:	DCR	A		; CHECK FOR 0 NULLS AT TOP OF LOOP

RZ			; RETURN IF ALL NULLS OUTPUT

PUSH	PSW		; SAVE NULLS COUNTER

SUB	A		; PRINT A SINGLE NULL

CALL	L2OUT		;	CHARACTER (RECURSIVE)

POP	PSW		; RESTORE NULLS COUNTER

JMP	L2RTN		; LOOP TO PRINT NEXT NULL

;

DKNUMB	EQU	4		;THIS IS WHERE THE DISK NUMBER IS

;KEPT IN NORMAL CP/M

CURDRV	DB	0		;THE SAVE AREA FOR THE DEFAULT DRIVE

DRVTBL	DB	2-LARGESW,0,0,0	;FOUR DRIVES MAX

DKSECT	DB	0		;SECTOR NUMBER

DKTRACK	DB	0		;TRACK NUMBER

DKDMA	DW	0		;DMA ADDRESS

TRKERCT	DB	0		;NUMBER OF TRACK ERRORS

RTYCNT	DB	0		;NUMBER OF READ ERRORS

;

;	FROM HERE ON, THE AREAS ARE INITIALIZED BY CP/M AS NEEDED

;

DIRBUF	DS	128		;SAVE AREA FOR DISK DIRECTORY OPERATIONS

ALV0	DS	32

;

IF	NBRDRV > 1

ALV1	DS	32

ENDIF

;

IF	NBRDRV > 2

ALV2	DS	32

ENDIF

;

IF	NBRDRV > 3

ALV3	DS	32

ENDIF

;

CSV0	DS	16

;

IF	NBRDRV > 1

CSV1	DS	16

ENDIF

;

IF	NBRDRV > 2

CSV2	DS	16

ENDIF

;

IF	NBRDRV > 3

CSV3	DS	16

ENDIF

;

;  Note:  The last assembled byte of this module MUST NOT be a Define

;  Storage (DS or DEFS) pseudo-op to assure proper operation with CDOSGEN

END

_______________________________________________________________________________________________