Breakthru 286 Manual Addendum

This note covers three topics: a replacement Appendix 7,

clock speeds for 80287 math coprocessors and more information

about turbo motherboards.

Appendix 7 - Software Utilities

Software Speed Switching

If you enable software speed switching with the * jumper (see Appendix

1), you will need to install the software that does the switch.  The

switching is done by two programs acting in concert, namely BSS.SYS

(Breakthru Speed Switch) and BRKTHRU.COM.

BSS.SYS

BSS.SYS must be copied into your boot directory (the same directory as

CONFIG.SYS) and specified in your CONFIG.SYS file with the following

command:

device = BSS.SYS

BSS.SYS can turn caching on and off at any time with a "hot key"

combination.  It is set up as the Left and Right shift keys pressed

together.  When both of these keys are pressed at the same time,

BSS.SYS will beep and change the speed of the cache.  It gives a long

low sound when it is turning the cache off, and a short high sound

when it is turning it on.

You can define a different hot key combination by adding "KEY =" and

then a combination of letters to the CONFIG.SYS entry.  Select two or

more letters from this list:

L	Left shift key

R	Right shift key

C	Control (Ctrl) key

A	Alt key

So for example if you wanted to switch Breakthru speed by using the

right shift key and the Ctrl key together, you would use the command:

device = BSS.SYS key=rc

You can also switch on caching at load time with BSS.SYS by adding the

parameter "CACHE ON" to the CONFIG.SYS command:

device = BSS.SYS cache on

This is useful if you want to cache your BIOS (put the R jumper on),

but your computer won't boot with the BIOS cached and caching on.

Simply put the jumper in the vertical position, so that speed

switching is enabled and the computer powers up slow.  The above

command will kick Breakthru into high speed to complete your boot.

Put this command in front of any EMS driver to avoid long waiting

during the power on self test.

BRKTHRU.COM

This program can change the hot keys, turn the cache on and off, and

turn off caching for floppy disk access.  It requires that BSS.SYS has

been installed.

You can define a different hot key combination with the command "KEY ="

followed by a combination of two or more letters from this list:

L	Left shift key

R	Right shift key

C	Control (Ctrl) key

A	Alt key

For example, you can change the hot keys to Alt plus left shift by

typing

BRKTHRU KEY=AL

BRKTHRU.COM can turn the cache on using the command

BRKTHRU CACHE ON

It will reply with one of the messages

BRKTHRU caching is now on

BRKTHRU caching is already on

BRKTHRU.COM can turn the cache off using the command

BRKTHRU CACHE OFF

It will reply with one of the messages

BRKTHRU caching is now off

BRKTHRU caching was not on

BRKTHRU.COM can show the state of the cache with the command

BRKTHRU CACHE

It will reply with one of the messages

BRKTHRU caching is currently on

BRKTHRU caching is currently off

The last option for BRKTHRU.COM is to turn off caching when accessing

disk drives.  This is sometimes necessary because the BIOS code

executes too fast for the disk controller, causing it to fail with a

message such as

General failure reading drive A:

Abort, Retry, Ignore?

If you see a message like this, you can recover by hitting the hot key

combination to turn off caching and then typing R to retry, but for a

permanent solution, type

BRKTHRU AB ON

This causes a portion of BRKTHRU.COM to remain memory-resident and to

turn caching off for drives A and B.

You can amend the drives for which slow-down is occurring by such

commands as

BRKTHRU A OFF

BRKTHRU B ON

and you can unload BRKTHRU.COM from memory by typing

BRKTHRU U

TEST286 and INVEN

To help verify that your Breakthru is running as it should, we have

included two utilities on your Lightning diskette.  They are called

TEST286.EXE and INVEN.EXE.

TEST286.EXE put your Breakthru board through its paces.  It is run on

every board before it is shipped, so it is very unlikely to register

an error.

INVEN.EXE explores your system and tries to figure out what's

installed.  It will give you a list of what it finds.  Try it out for

fun.  If it's wrong, we'd appreciate a call so we can update the

program.  Its main use is to pinpoint math coprocessor or floppy

diskette problems due to motherboard switch settings being faulty.

Clock speeds for the 80287 coprocessor.

The 80287 math coprocessor can be run at two speeds.  The two

speeds available depend on how your computer is set up, but they

are always:

1. The speed of the Breakthru 286

2. The speed of your system board.

On the 287 jumper of your Breakthru 286, the first setting is

labelled 8 and the second setting is labelled 5.  These would be the

megahertz speeds on a regular PC and an 8 MHz  Breakthru 286.

If you have a turbo motherboard that runs at 8 MHz, the setting

labelled 5 will run the 287 at 8 MHz.  If you have a Breakthru

286-12, the setting labelled 8 will run the 287 at 12 MHz.  It is

important to buy a 287 that is rated at the speed you intend to

run it, since unlike many chips, it is unlikely to work at a higher

speed than rated.

Turbo motherboards.

A conflict exists between some turbo motherboards and the

software speed switching of the Breakthru 286.  It can be

compensated for by careful setting of the * jumper.

If the turbo motherboard can switch between fast and slow with a

hot key, and it uses the same technique to switch speeds as does

Breakthru, it may accidentally switch the Breakthru speed.  This

will usually crash Breakthru, since a Breakthru speed switch can

only be properly done by the BSS or BRKTHRU.COM software.  There

are many ways around this which will be outlined below, but bear

in mind that the problem is caused by the motherboard switching

speeds, which can happen automatically at boot up, or when the

hot key combination is pressed.

The problem will not arise if you disable Breakthru software speed

switching by leaving the * jumper off altogether.  But this may

cause problems for you accessing your floppy disk if you desire to

cache your BIOS (R jumper on), and may sometimes stop your

computer booting.

If you allow Breakthru Speed Switching, but power up not caching,

(* jumper vertical), your computer will almost certainly crash at

some point in the boot up sequence when the motherboard

accidentally enables the Breakthru caching but does it wrong.

The alternative is to put the * jumper in the horizontal position,

so that when the motherboard switches on caching, it is already

on, and no harm is done.  The drawback of this technique is that

your computer may not boot properly in this configuration with the

R jumper on, so you may have to forego BIOS caching.

In any case, best results are usually achieved by jumpering your

motherboard to run at 8MHz, jumpering the Breakthru F/S jumper

to F, and never trying to switch motherboard speeds with the

motherboard hot key combination.  Always speed switch with the

Breakthru speed switch combination that you set in your

CONFIG.SYS file with the BSS 'key' command.