E2000plus-series Ethernet Adapters

Installation file for

\NDIS.30\WINNT40 directory

============================================================================

*********************************************

*  Driver Installation for Windows NT 4.0   *

*********************************************

1.  Logon as Administrator and select the Network icon from the

Control Panel.

2.  Select "Adapters" and click on "Add..." button. Highlight the NE2000

compatible driver to be installed. If Windows NT does not offer an NE2000

driver, choose the "Have Disk..." button.

3.  Insert the Windows NT system or E2000plus driver diskette into a

floppy disk drive.

4.  Follow the instructions appearing on the screen and select the

appropriate drive name when required to give the location for the

driver and the OEMSETUP.INF file.

For example :

A:\NDIS.30\WINNT40

5.  After pressing Continue, confirm the driver diskette, choose

IRQ and/or IO Address, and click on OK.

6.  Now, the driver has been installed. Select OK button from

the Network window. The protocols and driver will bind together.

7.  Reboot