USB Firmware Flashing Utility

OnSpec Electronic, Inc.

| REVISION HISTORY |  |  |
| --- | --- | --- |
| Date | Version | Reason for Change |
| Feb 05, 2000 | 1.00 | For V 1.00 release |
| May 08, 2000 | 1.01 | OnSpec Cable flash |

Document Title:  FLASH.DOC

Format: Word 97 for Windows 95

Date: May 05, 2000

Table of Contents

Function Description	3

USB1X-90C36LC1 Chip	3

Program Usage	3

Using the software for the first time	3

Open Issues	3

## Function Description

Firmware Flashing Utility is designed to help users to program their EEPROMs, examine the EEPROM, check the board for defects etc. While every effort has been taken to make this program as robust as possible, this utility may have a few limitations. It is important that the users read this manual first and get familiarized with its features.

## USB1X-90C36LC1 Chip

The USB1X-90C36LC1 chip is a 16-bit RISC processor, with an easy to use instruction set and also comes with a BIOS that needs to handle all the USB packets transactions as well as support these tools.

## Program Usage

When Firmware Flashing Utility is invoked, screen appears with one button to Flash LS-120 Firmware. The controls are explained below:

Flash OnSpec Cable: Use this option to flash the bridge with OnSpec Cable firmware.

.

Current Firmware: This provision will let you know about the Current Firmware loaded in the USB board like ATAPI, IDE, SCSI etc., If there is no Firmware -- it will display as “Blank (No Firmware). If you have connected a wrong device, it will display as “Not blank”.

Current Version: This will display the Current Firmware Version number.

## Using the software for the first time

- Click on A: drive and run SETUP. Default destination directory is C:\ONFLASH

- Start with a fully assembled USB-IDE interface board and apply power to it.

- Plug the USB cable to the board and the other end to a PC running Windows 98.

- The message, “OnSpec USB” will appear. You will be prompted to insert a diskette for the drivers. Choose Specify a location and type A:\ and follow the instructions.

## Open Issues

- When you exit this software, even if you do not program the flash, you must disconnect and re-connect the USB cable if your Drive Icon disappeared in your system. This will be fixed in the next release.