##### CardBus / PCMCIA IDE Card Quick Installation Guide for Windows and MacOS

Date: December 17, 2000

###### Windows95, 98 and Window98SE, ME Environments –

First time installation –

- Connect the device with device power, external power supply is required if Cardbus mode is selected.

- Locate the mode switch on the edge of the plastic cover of connection between PC Card and cable.

- Twig the CardBus/PCMCIA mode switch toward PC card for CardBus mode, away from it for PCMCIA mode by using pencil tip.

- Connect the PC card and device.

- Insert PC Card into socket.

- If CardBus mode selected,  Windows 9X pops up “New Hardware Find” & “PCI Mass Storage Controller” found messages , select “Recommend…” button to search for driver, enter “Specific Location” as “A:\Window95”, choose “pcmide95.inf” and hit “Enter” key.  Windows 9X finds the CardBus card specific driver and prompts for installation.  Enter “A:\Window95” for drivers search path.  Windows 9X copy necessary drivers into system and prompt for reboot.

- If PCMCIA mode selected,  Windows 9X pops up “New Hardware Find” & “Shining PMIDE-ASC Controller” found messages , select “Recommend…” button to search for driver, enter “Specific Location” as “A:\Window95”, choose “pcmide95.inf” and hit “Enter” key.  Windows 9X finds the PCMCIA card specific driver and prompts for installation.  Enter “A:\Window95” for drivers search path.  Windows 9X copy necessary drivers into system and prompt for reboot.

Hot-insertion

- Connect the device with device power.

- Locate the mode switch on the edge of the plastic cover of connection between PC Card and cable.

- Use pencil to twig the CardBus/PCMCIA mode switch toward PC card for CardBus mode, away from it for PCMCIA mode.

- Connect the PC card and device.

- Insert PC Card into socket.

- If CardBus mode selected,  a Windows 98 pop up “PCI Mass Storage Controller” found message and initializes the device.  Drive icon shows up in “MyComputer” folder and accessible.

- If PCMCIA mode selected,  a Windows 98 pop up “Shining PMIDE-ASC Controller” found message and initializes the device.  Drive icon shows up in “MyComputer” folder and accessible.

<<< See reverse side for MacOS Installation >>>

###### MacOS 8.5, 8.6, 8.7, 9.0 Environments –

First time installation –

- Un-stuff the driver’s file onto desktop.

- Drag and drop following files into “SystemFolder:Extensions”.

Files required for MacOS 8.5 to 8.7 are “ATAManager v4.0”, “PCCard AIM”( if PCMCIA mode is

Selected ) and “Shining Cardbus Driver”( if Cardbus mode is selected ).

File required for MacOS 9.0 is “Shining Cardbus Driver” if Cardbus mode selected.  ATAManager v4.0

And “PCCard AIM” are already included in MacOS 9.0.

- Reboot the computer.

Hot-insertion

- Connect the device with device power, external power supply is required if Cardbus mode is selected

- Locate the mode switch on the edge of the plastic cover of connection between PC Card and cable.

- Twig the CardBus/PCMCIA mode switch toward PC card for CardBus mode, away from it for PCMCIA mode by using pencil tip.

- Connect the PC card and device.

- Insert PC Card into socket.

- Hard disk icon shall pop up if it’s partitioned/formatted with file system recognized by MacOS; otherwise, use “Drive Setup” utility in the “Utility” folder to prepare the hardisk.

- CD-ROM type icon shall pop up if medium with disc format recognized by MacOS.  Other CD-ROM, DVD, or CD Recording software is now ready to use.

Precaution notes:

First time insertion installs driver that performs both CardBus and PCMCIA functions.

Always stop the PC Card before ejection by opening the “PC Card” icon in “Control Panel” folder,

selecting the  “Shining CardBus( PCMCIA ) IDE Controller” and “Stop” it if operating in Windows

enviroment.

Drag the icon to “Trash” can and eject the PC Card before removing the whole setup if operating in

Macintosh enviroment.

Twig the mode switch only when the “PC Card” is completely out of the socket to avoid damages.

Use “Drive Setup” utility that include in MacOS to prepare the hard disk format.

###### Windows 2000 Environment( Final Release Version Only )--

First time installation –

- Connect the device with device power, external power supply is required if Cardbus mode is selected.

- Locate the mode switch on the edge of the plastic cover of connection between PC Card and cable.

- Twig the CardBus/PCMCIA mode switch toward PC card for CardBus mode, away from it for PCMCIA mode by using pencil tip.

- Connect the PC card and device.

- Insert PC Card into socket.

Cardbus mode installation( skip to “PCMCIA mode installation” for PCMCIA-16bit installation )

- Windows 2000 pops up “New Hardware Find” & “PCI Mass Storage Controller” found messages , select “Next” to continue installation, select “Recomm…” button and specify path like “A:\Windows2K”.

- Select “pcmide2k.inf”, “Shining Cardbus IDE Win2k Controller” and “Next” to complete the installation

PCMCIA mode installation--

- Windows 2000 pops up “New Hardware Find” & “Shining PMIDE-ASC Controller” found messages , select “YES” to continue installation. Select “Recomm…” button and specify path like “A:\Windows2K”.

- Select “pcmide2k.inf”, “Shining PCMCIA IDE Win2k Controller” and “Next” to complete the installation

Update to Manufacturer Driver--

- Connect the device with device power.

- Locate the mode switch on the edge of the plastic cover of connection between PC Card and cable.

- Use pencil to twig the CardBus/PCMCIA mode switch toward PC card for CardBus mode, away from it for PCMCIA mode.

- Connect the PC card and device.

- Insert PC Card into socket.

Cardbus mode:

- Go to “Setting”, “Control Panel”, “System”, “Hardware”, “Device Manager” and open “IDE ATA/ATAPI Controller”.

- Select “Shining Cardbus IDE Controller Win2000”.  Double click on that object.

- Select “Driver”.  Select “Update Driver” or “Re-install Driver” button.  Select “Next” to continue.

- Specify path like “A:\Windows2K” for “pcmide2k.inf”.

- Select “Shining Cardbus IDE Controller Win2000” and “Next” or “Yes” to continue installation.

PCMCIA mode:

- Go to “Setting”, “Control Panel”, “System”, “Hardware”, “Device Manager” and open “IDE ATA/ATAPI Controller”.

- Select “Shining PCMCIA IDE Controller Win2000” with yellow mark in front of it.  Double click on that object.

- Select “Driver”.  Select “Update Driver” or “Re-install Driver” button.  Select “Next” to continue.

- Specify path like “A:\Windows2K” for “pcmide2k.inf”.

- Select “Shining PCMCIA IDE Controller Win2000” and “Next” or “Yes” to continue installation

Cardbus, PCMCIA/IDE Card Quick Installation for Windows 2000

Cardbus, PCMCIA/IDE Card Quick Installation for Windows 98’s

Cardbus, PCMCIA/IDE Card Quick Installation for MacOS’s