##### Firewire IDE Bridge Quick Installation Guide for Windows and MacOS

Date: August 20, 2000

###### Window98 Second Edition and Windows 2000 Environments –

First time installation –

- Connect the device with device power, external power supply is required if bus power is not supported.

- Skip this step if you have Firewire/IDE bridge built inside enclosure already; otherwise, attach external Firewire converter to your device.

- Plug Firewire connector into system.

- Windows 98 SE shall pop up “New Hardware Find” & “SBP2 Device” found messages and completes the installation by itself.

Note:  It’s suggested to update Windows 98 Second Edition Firewire driver to the latest version if user is

Running Windows98 Second Edition.   No need to include following patches for Window 2000.

Microsoft published the update, file named as “242975us8.exe”

"http://www.microsoft.com/windows98/downloads/contents/wurecommended/s_wufeatured/1394”.

By doing so,  a “Plug-n-Play” insertion icon just like the one in Win2000 will show up at bottom right

Corner, which allow user to stop Firewire devices before un-plugging the cable to prevent system

hangs.

Hot-insertion

- Connect the device with device power, external power supply is required if bus power is not supported.

- Skip this step if you have Firewire/IDE bridge built inside enclosure already; otherwise, attach external Firewire converter to your device.

- Plug Firewire connector into system.

- Windows 98 SE or Windows 2000 shall bring drive icons to the desktop.

Hot-removal

- Locate the “plug-n-play” icon at bottom right corner.

- To activate the icon by double-clicking mouse.

- Click on the device that you want to stop and click on “STOP”.

- Wait for Win98 or Windows 2000 to complete the stopping process.

- Win98 or Windows 2000 prompts “Safe to removal…” message, then, un-plug the Firewire connector.

Note:

No icon will show up in “My Computer” folder if  hard disk attached is brand new or un-recognizable partitions by Windows OS’s.

<<< See reverse side for MacOS Installation >>>

###### MacOS 8.5, 8.6, 8.7, 9.0 Environments –

First time installation –

- Un-stuff the drivers’ file onto desktop.

- Drag and drop following files into “SystemFolder:Extensions”.

Files required for MacOS 8.5 to 9.0 are “ATAManager v4.0”, “FirewireAIM” drivers.

“Firewire Enabler” and “Firewire Support” are also included.  If both files are not pre-installed by MacOS, simply drag both files into “SystemFolder:Extensions” as well.  If both files are already installed, do not install them.

- Reboot the computer.

Hot-insertion

- Connect the device with device power, external power supply is required if user is using internal bridge model.  However, bus power current is different in terms of the system and limited as well..

Following are configurations that do need external power supply to external devices:

Macintosh desktop systems that connect with 3.5” hard disk, CD-R, CD-RW device.

PowerBook without Firewire port built-in and a Cardbus/Firewire card is attached into PCMCIA slot.

- Connect the device with Firewire converter or cable.

- Turn on external device’s power supply if needed.

- Insert Firewire connector into system Firewire’s port.

- Hard disk icon shall pop up if it’s partitioned/formatted with file system recognized by MacOS; otherwise, use “Drive Setup” utility in the “Utility” folder to prepare the hardisk.

- CD-ROM type icon shall pop up if medium with disc format recognized by MacOS.  Other CD-ROM, DVD, or CD Recording software is now ready to use.

Precaution notes:

Always drag the icon to “Trash” can before removing the Firewire connector from system.

Firewire/IDE Bridge Windows 98SE and Windows 2000 Quick Installation Guide

Firewire/IDE bridges Quick Installation for MacOS’s