SUNIX EXTERNAL FLOPPY DISK INSTALLING

1. OVERVIEW

The Sunix Floppy Driver enables PC/XT/AT Systems to

handle up to 4 floppy drives. This floppy driver

handles all four popular formats:

1) 5.25"  360KB Floppy Drives

2) 5.25"  1.2MB Floppy Drives

3) 3.50"  720KB Floppy Drives

4) 3.50"  1.4MB Floppy Drives

Any combination of the above named drives can be support by

the Loadable Floppy Driver.

2. COMPATIBILITY

The Sunix Floppy disk controller card with floppy option, may be

used to provide connectivity of up to four floppy drives

with the use of the Sunix Floppy Driver.

3. INSTALLATION

HARDWARE INSTALLATION:

a) No Floppy Controller on the System Board or the Floppy

Controller on the System Board is disabled:

o-   Set option jumper on the Sunix floppy disk Future Domain Host

Adapter to "Primary Floppy Address" (set F1 to

OPEN). Refer to Users Guide.

o-   Note, with this option interrupt #6 (IRQ6) is

being used. Make sure no other controller uses

this interrupt.

IF YOU HAVE IBM STANDARD FLOPPY CABLES:

(The address signals 10 through 16 of the cable are

twisted. The primary floppy drive is the end-cable

connector and the secondary floppy drive is the

mid-cable connector.)

o-   Set all floppy drives to second drive select.

(i.e., Some floppy drives have the drive selects

DS0, DS1, etc. In this case the floppy  drive select

would be at DS1. Other floppy drives have the drive

selects DS1, DS2, etc. In this case the drive select

would be at DS2)

o-   Connect drive A: to the end of the cable.

o-   Connect drive B: to the middle of the cable.

o-   Connect the floppy drives designated as drives A:

and B: to the "PRIMARY DISKETTE" connector

o-   Connect the third floppy drive to the end

connector and the fourth floppy drive to the

middle connector of the second cable.

o-   Connect the third and fourth floppy drives to the

"SECONDARY DISKETTE" connector

o-   Stop here and go to software installation

section.

IF YOU HAVE NON-STANDARD FLOPPY CABLES:

(On the floppy cable address signals 10 through 16 are

not twisted. This is a straight through cable.)

o-   Set all floppy drives to first drive select.

(i.e., Some floppy drives have the drive selects

DS0, DS1, etc. In this case the drive select would

be at DS0. Other floppy drives have

drive selects labeled DS1, DS2, etc. In this

case the drive select would be at DS1.)

o-   Connect drive A: to the end of the cable.

o-   Connect drive B: to the middle of the cable.

o-   Connect the floppy drives designated as drives

A: and B: to the "PRIMARY DISKETTE" connector

o-   Connect the third floppy drive to the end

connector and the fourth floppy drive to the

middle connector of the second cable.

o-   Connect the third and fourth floppy drives to

the "SECONDARY DISKETTE" connector

o-   Stop here and go to software installation

section.

b) Floppy controller on the System Board is enabled.

(Floppy drive A:/B: already installed)

o-   Set option jumper on the Sunix floppy

controller card  "Secondary Floppy Address"

(set F1 to CLOSED). Refer to Users menu.

IF YOU HAVE IBM STANDARD FLOPPY CABLES:

(The address signals 10 through 16 of the cable are twisted.

The primary floppy drive is the end-cable connector and the

secondary floppy drive is the mid-cable connector.)

o-   Set all floppy drives to second drive select.

(i.e., Some floppy drives have the drive selects

labeled DS0, DS1, etc. In this case the floppy

drive select would be at DS1. Other floppy drives

have the drive selects labeled DS1, DS2, etc. In

this case the drive select would be at DS2.)

o-   Connect drive #N1 to the end of the cable.

o-   Connect drive #N2 to the middle of the cable.

o-   Connect the floppy drives designated as drives

#N1 and #N2 to the "PRIMARY DISKETTE" connector

o-   Connect floppy drive #N3 to the end connector

and floppy drive #N4 to the middle connector of

the second cable.

o-   Connect floppy drives #N3 and #N4 to the

"SECONDARY DISKETTE" connector

o-   Stop here and go to software installation

section.

IF YOU HAVE NON-STANDARD FLOPPY CABLES:

(On the floppy cable address signals 10 through 16 are not

twisted. This is a straight through cable.)

o-   Set all floppy drives to first drive select.

(i.e., Some floppy drives have the drive

selects DS0, DS1, etc. In this case the

floppy drive select would be at DS0. Other

floppy drives have the drive selects labeled

DS1, DS2, etc. In this case the drive

select would be at DS1.)

o-   Connect drive #N1 to the end of the cable.

o-   Connect drive #N2 to the middle of the cable.

o-   Connect the floppy drives designated as drives

#N1 and N2 to the "PRIMARY DISKETTE" connector

o-   Connect floppy drive #N3 to the end connector

and floppy drive #N4 to the middle connector of

the second cable.

o-   Connect floppy drives #N3 and #N4 to the

"SECONDARY DISKETTE" connector

o-   Stop here and go to software installation section.

SOFTWARE INSTALLATION:

The Floppy Driver is loaded into memory, while the

config.sys file is being processed. In order for the system

to load the Floppy Driver, the SDRIVE.SYS file should be

located in the boot device. The following statement should

be added to the config.sys file:

DEVICE=SDRIVE.SYS /u,t,a .... /u,t,a   /Ixx

where:

u - unit number (0,1,2,3)

Each floppy controller chip can handle up to four

drive units. The first drive addressed by this

controller is named unit number 0 and the fourth

drive addressed by this controller is named unit 3.

t - drive type - 0 = 180KB - 5 1/4"

1 = 360KB - 5 1/4"

2 = 1.2MB - 5 1/4"

3 = 720KB - 3 1/2"

4 = 1.4MB - 3 1/2"

a - controller address - P = Primary (FDC Host Adapter only)

- S = Secondary (AT/System and

FDC Host Adapter)

- R = Replace the A:/B: Driver

Depending on which controller the drive is connected

to the "a" parameter is set to "P" or "S" or "R". See the

examples.

SDRIVE.SYS defaults to using INT 7Eh & 7Fh. To change this

you can use the following command line syntax:

/Ixx

where:

xx are two hex digits representing the interrupt #. Use this

parameter only if you find that other software is

using these interrupts in your system.

EXAMPLES:

a)

DEVICE=SDRIVE.SYS /2,2,P /3,1,P

where:

/2,2,P

2 - unit number specifying third floppy drive in system.

2 - drive type - 2 = 1.2MB - 5 1/4"

P - controller address - P = Primary (FDC Host Adapter only)

/3,1,P

3 - unit number specifying the fourth floppy drive in system.

1 - drive type - 1 = 360KB - 5 1/4"

P - controller address - P = Primary (FDC Host Adapter only)

1. No other floppy controller is present in System.

2. FDC Floppy controller is configured at Primary Address.

3. The two floppy drives on the primary diskette connector is

by the System BIOS as drive A: and B:.

4. SDRIVE.SYS is supporting the third and fourth

floppy drive on the secondary diskette connector on

FDC Host Adapter.

DOS Internal Floppy Driver usually takes over the unit 0

and 1 on the primary controller as drives A: and B:. The

Internal Floppy Driver uses the System BIOS to communicate

with the floppy controller. Some System BIOS codes do not

expect another driver to handle units 2 and 3. Therefore,

it may be necessary for SDRIVE.SYS to take over the control

of the A: and B: drives, to avoid conflicts with the System

BIOS. The next example shows you how to control drives A:

and B:.

b)

DEVICE=SDRIVE.SYS /0,4,R /1,4,R /2,2,P /3,1,P

where:

/0,4,R

0 - unit number specifying first floppy drive in system.

4 - drive type - 4 = 1.4MB - 3 1/2"

R - controller address - R = Replace the A:/B: Driver

/1,4,R

1 - unit number specifying the second floppy drive in

system.

4 - drive type - 4 = 1.4MB - 3 1/2"

R - controller address - R = Replace the A:/B: Driver

/2,2,P

2 - unit number specifying third floppy drive in system.

2 - drive type - 2 = 1.2MB - 5 1/4"

P - controller address - P = Primary (FDC Host Adapter only)

/3,1,P

3 - unit number specifying the fourth floppy drive in

system.

1 - drive type - 1 = 360KB - 5 1/4"

P - controller address - P = Primary (FDC Host Adapter only)

1. No other floppy controller is present in System.

2. FDC Floppy controller is configured at Primary Adress.

3. SDRIVE.SYS is supporting all four floppy drives attached

to the primary and secondary connector on the FDC Host Adapter

Note: You do not have to take over the control of the drives A:

(Unit 0 in the above example) and B: (unit 1). You should

take control only if your System BIOS does not expect

another floppy driver on the primary floppy controller.

The way to detect this conflict, is to not to take control

at first. When system is up and operational, do "DIR" on the

drive A: and then perform "DIR" on unit 2 or 3 . Now, if the

next "DIR" command to the A: drive failed, you should let the

SDRIVE.SYS take over drives A: and B: .

c)

DEVICE=SDRIVE.SYS /0,2,S /1,1,S

Adds two additional drives which are connected to the

primary connector of the secondary controller.

#################################