LINUX

PCI I/O Cards

Linux

Installation manual

	Installation Guide for PCI Serial Ports	3

	Installation Guide for PCI Parallel Ports	8

- Installation Guide for PCI Serial Ports

This installation guide describes the procedures to install EXSYS PCI serial ports in Linux platform.

[Linux Platform]

- Operating System		: RedHat V6.1/V6.0 (Kernel 2.2.x / 2.0.x)

- Terminal Emulation AP	: minicom / xminicom

- Internet Dialer			: Kppp

[Installation Steps]

- Find the available serial ports

Since Linux only support 4 serial ports (ttyS0, ttyS1, ttyS2, ttyS3) under the default condition. Most likely, ttyS0 & ttyS1 are supported by mother board's built-in 16550 controllers and ttyS2 & ttyS3 are free for additional I/O card. (Note that ttyS2: S is upper case)

It could be checked by the following commands.

#setserial /dev/ttyS0 -a 		(COM1)

#setserial /dev/ttyS1 -a 		(COM2)

#setserial /dev/ttyS2 -a 		(COM3)

#setserial /dev/ttyS3 -a 		(COM4)

If COM1 is used by mouse, the response is similar to

/dev/ttyS0 : Device or resource busy

If the COM1 does not attach any device, the response is similar to

/dev/ttyS0, Line 0, UART: 16550A, Port: 0x3f8, irq: 4

Baud_base: 115200, clos_delay: 50, divisor: 0

closing_wait: 3000, closing_wait2: infinite

Flags: spd_normal skip_test

In case ttyS2 (COM3) is free, the response for command

# setserial /dev/ttyS2 -a is shown below.

/dev/ttyS2, Line 2, UART: unknown, Port: 0x3e8, irq: 4

Baud_base: 115200, clos_delay: 50, divisor: 0

closing_wait: 3000, closing_wait2: infinite

Flags: spd_normal skip_test

(note that UART: unknown)

In case ttyS3 (COM4) is free, the response for command # setserial /dev/ttyS3 -a is shown below.

/dev/ttyS3, Line 3, UART: unknown, Port: 0x2e8, irq: 3

Baud_base: 115200, clos_delay: 50, divisor: 0

closing_wait: 3000, closing_wait2: infinite

Flags: spd_normal skip_test

(note that UART: unknown)

Finally, the /dev/ttyS2 & /dev/ttyS3 are free for PCI serial ports.

- Find the PCI card resource ( IO port address & IRQ) for the serial ports

Please enter the command "#more /proc/pci".

The response is similar to the following

…………………………………………………………………..

…………………………………………………………………..

Bus 0, Device 11, function 0:

^^

Serial controller : Unknown vendor Unknown device (rev 1).

Vendor id=1409, Device id=7168

Medium devsel. Fast back-to-back capable. IRQ 10

^^

I/O at 0xef80 [0xef81]

^^^^  ^^^^

…………………………………………………………………..

(note : ^^ means it could be different from the above.

They are varied with the different PC.)

From the /proc/pci file, it is possible to find the PCI card's IO port address and IRQ. Especially, the EXSYS card always shows

"Vendor id=1409, Device id=7168".

- Configure the parameters for ttyS2 & ttyS3

for EXSYS 41051 card (PCI 1S), please enter (if ttyS2 is free)

# setserial /dev/ttyS2 port 0xef80 UART 16550A

irq 10 Baud_base 921600

for EXSYS 41092 card (PCI 2S), please enter (if ttyS2 & ttyS3 are free)

# setserial /dev/ttyS2 port 0xef80 UART 16550A

irq 10 Baud_base 921600

# setserial /dev/ttyS3 port 0xef88 UART 16550A

irq 10 Baud_base 921600

- Check the setting for ttyS2 & ttyS3

Please enter # setserial /dev/ttyS2 -a

The response look likes below

/dev/ttyS2, Line 2, UART: 16550A, Port: 0xef80, irq: 10

Baud_base: 921600, clos_delay: 50, divisor: 0

closing_wait: 3000, closing_wait2: infinite

Flags: spd_normal skip_test

(5) Then the ttyS2 & ttyS3 are ready for application

(eg. minicom -s or xminicom -s or Kppp ...)

(6) In case more than 4 serial ports are needed

If there are more than 4 serial ports to be supported by Linux system, the first step is to add more tty device nodes into system.

Inquire the system tty device nodes,

#ls -al /dev/ttyS*

crw-------   1  root  tty   4, 64 Jan 8 11:40 /dev/ttyS0

crw-------   1  root  tty   4, 65 Jan 8 11:40 /dev/ttyS1

crw-------   1  root  tty   4, 66 Jan 8 11:40 /dev/ttyS2

crw-------   1  root  tty   4, 67 Jan 8 11:40 /dev/ttyS3

Add tty device node one by one

#mknod  /dev/ttyS4  c  4  68		(for ttyS4)

#mknod  /dev/ttyS5  c  4  69		(for ttyS5)

#mknod  /dev/ttyS6  c  4  70		(for ttyS6)

#mknod  /dev/ttyS7  c  4  71		(for ttyS7)

………………………….

Please add all tty device nodes accordingly

Configure the parameters for all new ttyS*

Please repeat step (2) (3) (4) to inquire and change the I/O address for each tty device. Because all the new added tty device nodes are still invalid by default.

For PCI 4S card (41094), it allocate 2 I/O resources. You could inquire it according to step(2). E.g.

# more /proc/pci

……………………………….

Vendor id=1409, Device id=7168

Medium devsel. Fast back-to-back capable. IRQ 10

I/O at 0xd000 [0xd001]	 1st port=0xd000, 2nd port=0xd008

I/O at 0xb800 [0xb801]	 3rd port=0xb800, 4th port=0xb808

For PCI 8S card (41098), it allocate 6 I/O resources. You also need to inquire its allocated address

# more /proc/pci

……………………………….

Vendor id=1409, Device id=7168

Medium devsel. Fast back-to-back capable. IRQ 10

^^

I/O at 0xd000 [0xd001]	 1st port=0xd000, 2nd port=0xd008

I/O at 0xb800 [0xb801]	 3rd port=0xb800, 4th port=0xb808

I/O at 0xb400 [0xb401]	 5th port=0xb400

I/O at 0xb000 [0xb001]	 6th port=0xb000

I/O at 0xa800 [0xa801]	 7th port=0xa800

I/O at 0xa400 [0xa401]	 8th port=0xa400

Re-Inquire the system tty device nodes,

#ls -al /dev/ttyS*

crw-------   1  root  tty   4, 64 Jan 8  11:40 /dev/ttyS0

crw-------   1  root  tty   4, 65 Jan 8  11:40 /dev/ttyS1

crw-------   1  root  tty   4, 66 Jan 8  11:40 /dev/ttyS2

crw-------   1  root  tty   4, 67 Jan 8  11:40 /dev/ttyS3

crw-r--r--   1  root  root  4, 68 Jan 18 11:40 /dev/ttyS4

crw-r--r--   1  root  root  4, 69 Jan 18 11:40 /dev/ttyS5

crw-r--r--   1  root  root  4, 70 Jan 18 11:40 /dev/ttyS6

crw-r--r--   1  root  root  4, 71 Jan 18 11:40 /dev/ttyS7

………………………….

Important Notes :

- Since all serial ports on EXSYS PCI card are using only one interrupt pin, you must set them the same IRQ number with setserial command.

- Un-installation,

e.g.#rm /dev/ttyS4 (remove ttyS4 device)

- Installation Guide for PCI Parallel Ports

This installation guide describes the procedure to install EXSYS PCI parallel ports in Linux platform.

[Linux Platform]

- Operation System:	RedHat V6.1/V6.0 (Kernel 2.2.x / 2.0.x)

- "AnotherLevel menu -> administration -> printtool" in X windows.

[Installation steps]

Linux kernel provide a 'parport' code to support parallel port (/dev/lp0,  /dev/lp1, /dev/lp2). This code provides the ability to share one port between multiple devices. And it is loadable when kernel is running.

The 'parport' code is split into two parts : generic (which deals with port sharing) and architecture-dependent eg. X86, SPARC. (which deals with actually using the port). Thus, please take the following steps to install 'parport' module!

- Check the on-board parallel port's hardware resources

Most likely each motherboard has one built-in parallel port. And its hardware resources are settable with BIOS utility.

(for example, on-board parallel is in 0x378 port, IRQ=7 )

- Find the PCI resource ( IO port address & IRQ) for the parallel ports

please enter the command "# more /proc/pci"

The response will look like the following

…………………………………………………………………..

…………………………………………………………………..

Bus 0, Device 11, function 0:

^^

Parallel controller : Unknown vendor Unknown device (rev 1).

Vendor id=1409, Device id=7268 (or 7168)

Medium devsel. Fast back-to-back capable. IRQ 10

^^

I/O at 0xe000 [0xe001]

I/O at 0xd800 [0xd801]

I/O at 0xd400 [0xd401]

I/O at 0xd000 [0xd001]

^^^^  ^^^^

…………………………………………………………………..

…………………………………………………………………..

(Note : ^^ means it could be different from the above.

They are varied with the different PC.)

From the /proc/pci file, it is possible to find the PCI card's I/O port address and IRQ. Especially, the EXSYS card always shows

"Vendor id=1409, Device id=7268"

or

"Vendor id=1409, Device id=7168"

- Install 'parport' module

<step I> to load the generic 'parport' code

# insmod parport.o

<step II> to load the architecture-dependent code to tell 'parport' code

# insmod parport_pc.o  io=0x378,0xe000,0xd400

irq=7,none,none

that you have three PC-style ports, one at 0x378 with IRQ 7, one at

0xe000 with no IRQ, one at 0xd400 with no IRQ.

Note :

1. Please enter "#find /lib -name parport.o" to find out the correct file path. (it is varied with the different kernel version)

2. For the detailed description, please refer to /Documentation /parport.txt in Linux kernel

- Check the attached printer device [optional]

once the architecture-dependent part of the paraport code is loaded

into the kernel. You could enter the following command

# insmod parport_proble.o

to check any attached devices and log a message similar to

parport0: Printer, BJC-210 (Canon)

- Then /dev/lp0, /dev/lp1 and /dev/lp2 are ready for service

Now parallel port is available, please connect a correct type of printer for your printing applications.

(eg. AnotherLevel menu -> administration -> printttool in X windows)

Note :

- Type "# cat /dev/printcap" to inquire the printer connection.

- It's possible to print a text file to the printer for verification

(#lpr -Pprinter_name  textfile_name)

- In case more than 3 parallel ports ( eg. /dev/lp3, /dev/lp4 …) are needed for service

Please get the root privilege and enter the following command

#mknod  /dev/lp3  6  3

#chmod  660  /dev/lp3

#chgrp  daemon  /dev/lp3

to add  /dev/lp3  into kernel

#mknod  /dev/lp4  6  4

#chmod  660  /dev/lp4

#chgrp  daemon  /dev/lp4

to add  /dev/lp4  into kernel

………

………

then please enter the following command to check /dev/lp devices

#ls -al  /dev/lp*

crwxrwxrwx  1 root  daemon  6, 0  may 5 1998  /dev/lp0

crwxrwxrwx  1 root  daemon  6, 1  may 5 1998  /dev/lp1

crwxrwxrwx  1 root  daemon  6, 2  may 5 1998  /dev/lp2

crwxrwxrwx  1 root  daemon  6, 3  dec 9 1999  /dev/lp3

crwxrwxrwx  1 root  daemon  6, 4  dec 9 1999  /dev/lp4

- and then repeat step (3) to load parport module for /dev/lp*.

- Note :  #rm  /dev/lp3  to remove it