Banyan Vines installation with NE2000 Compatible NDIS driver

------------------------------------------------------------

for

NIC-2002/UK series LAN Adapter

1. Installation Introductions for Banyan Vines

----------------------------------------------

a. Type PCCONFIG.  Press Enter.

"Configuring A PC for the Network" appears.

b. Select "1 -  Network Card Setting", press Enter.

"The possible hardware configuration option are" appears.

c. Choose "NDIS Ethernet".  Press ENTER.

d. Key in specified interrupt number, and UK000x to the PROTOCOL bindings = .

e. Press 'F10' when done.  Press ESC to exit this screen.

f. Select "2 - Login Environment Settings."  Press ENTER.

g. Select "1 - Select Default Communications Driver."  Press ENTER.

h. Choose "NDIS Ethernet".  Press ESC to exit this menu.

i. Press 'F10' when done.

"Please wait ...." appears.

k. Modify the CONFIG.SYS and PROTOCOL.INI files, see the example.

Sample configuration files:

2. CONFIG.SYS

-------------

device=c:\banyan\protman.dos/i:c:\banyan

device=c:\banyan\ethne.dos

3. Protocol.ini file for LAN Manager

------------------------------------

[protocol manager]

DriverName = PROTMAN$

[UK000x]

DriverName = ethne$