Microsoft LanManager installation with NE2000 Compatible NDIS driver

--------------------------------------------------------------------

for

NIC-2002/UK series LAN Adapter

1. Installation Introduction for LAN Manager V2.0

--------------------------------------------------

a. Type 'SETUP' in the LanManager Directory.  Press ENTER.

b. Press ENTER.

c. Choose "Import Network Drivers..." in "Action".  Press Enter.

d. Type the specific source drive for the Driver Disk (A: or B:).

e. Insert the Driver Disk in the specific drive (A: or B:).  Press ENTER.

"Import Network Drivers" appears.

f. Press the SPACEBAR to select Network Adapter Card, "NE2000 Compatible

Ethernet Adapter Card".  Press Enter.

g. Press NO button.

h. Press Enter three times.

Network Driver appears.

i. Choose "NE2000 Compatible Ethernet Adapter Card" in Network Adapter Cards

select menu, then press <Add driver> button, and press <Add protocol>,

press <OK>. Workstation Primary parameters appear, press <OK>.

k. Press <OK> three times.

l. Choose "Save" in "Action".

m. Press F3 to Exit.

*******************************************************************************

2. Installation Introduction for LAN Manager V2.1

--------------------------------------------------

a. Type 'LMSETUP' in LanManager Directory.  Press ENTER.

Microsoft LAN Manager Setup appears.

b. Choose "Network Drivers..." in "Configuration".  Press ENTER.

c. Choose <Add New Configs>.  Press ENTER.

Network Adapter Driver appears.

d. Choose <Other Driver>.  Press ENTER.

e. Insert the Driver Disk in the specified drive (A: or B:), key in the

specified drive (A: or B:), then push the <ok> button.

f. Select an available Network Adapter Driver, NE2000 Compatible

Ethernet Adapter Card. Push the <OK> button.

g. Press the SPACEBAR to select the protocol that you want.

h. Push <OK> button.

Configuration Complete appears.

i. Push <Save> button.

j. Press "F3" to exit.

k. Press Enter.

Sample configuration files:

---------------------------

3. CONFIG.SYS

a.<V2.0>

LASTDRIVE=Z

BUFFERS = 35

FILES = 35

SHELL=C:\DOS50\COMMAND.COM C:\DOS50\e:1000 /p /E:1024

DEVICE=C:\HIMEM.SYS

DEVICE=C:\DOS50\SETVER.EXE

DOS=HIGH,UMB

DEVICE=C:\LANMAN.DOS\DRIVERS\PROTMAN\PROTMAN.DOS /i:C:\LANMAN.DOS

DEVICE=C:\LANMAN.DOS\DRIVERS\ETHNE\ACCNE\ETHNE.DOS

DEVICE=C:\LANMAN.DOS\DRIVERS\PROTOCOL\NETBEUI\NETBEUI.DOS

b.<V2.1>

LASTDRIVE=Z

BUFFERS = 35

FILES = 35

SHELL=C:\DOS50\COMMAND.COM C:\DOS50\e:1000 /p /E:1024

DEVICE=C:\HIMEM.SYS

DEVICE=C:\LANMAN.DOS\DRIVERS\DOSUTILS\EMM386.EXE NoEMS

DEVICE=C:\DOS50\SETVER.EXE

DOS=HIGH,UMB

DEVICE=C:\LANMAN.DOS\DRIVERS\PROTMAN\PROTMAN.DOS /i:C:\LANMAN.DOS

DEVICE=C:\LANMAN.DOS\DRIVERS\ETHNE\ACCNE\ETHNE.DOS

4. PROTOCOL.INI file for LAN Manager.

a. V2.0

[PROTOCOL MANAGER]

DRIVERNAME = PROTMAN$

[NETBEUI_XIF]

Drivername = netbeui$

BINDINGS = "ACCNE_NIF"

[ACCNE_NIF]

; PROTOCOL.INI for NE2000 Ethernet Aadapter

drivername = ETHNE$

;   irq = 3

;   iobase = 0x300

; End of this sample file

b. V2.1

[PROTMAN]

DRIVERNAME = PROTMAN$

DYNAMIC = YES

PRIORITY = NETBEUI

[NETBEUI_XIF]

Drivername = netbeui$

SESSIONS = 6

NCBS = 12

BINDINGS = "ACCNE_NIF"

LANABASE = 0

[ACCNE_NIF]

; PROTOCOL.INI for NE2000 Compatible ethernet adapter

drivername = ETHNE$

;   irq = 3

;   iobase = 0x300

; End of this sample file

5. Autoexec.bat file for LAN Manager.

a. V2.0

@echo off

PROMPT $P$G

PATH=c:\PE2;C:\DOS50;C:\NC;

APPEND=C:\PE2;C:\DOS50;

REM ==== LANMAN 2.0 == DO NOT MODIFY BETWEEN THESE LINES === LANMAN 2.0 ====

SET PATH=C:\LANMAN.DOS\NETPROG;%PATH%

C:\LANMAN.DOS\DRIVERS\PROTMAN\NETBIND

NET START WORKSTATION

REM ==== LANMAN 2.0 == DO NOT MODIFY BETWEEN THESE LINES === LANMAN 2.0 ====

b. V2.1

@echo off

PROMPT $P$G

PATH=c:\PE2;C:\DOS50;C:\NC;

APPEND=C:\PE2;C:\DOS50;

@REM ==== LANMAN 2.1 == DO NOT MODIFY BETWEEN THESE LINES === LANMAN 2.1 ===

SET PATH=C:\LANMAN.DOS\NETPROG;%PATH%

NET START WORKSTATION

LOAD NETBEUI

@REM ==== LANMAN 2.1 == DO NOT MODIFY BETWEEN THESE LINES === LANMAN 2.1 ===