Novell Netware installation with NE2000 Compatible driver

---------------------------------------------------------

for

NIC-2002/UK series LAN Adapter

1. Installation for Netware Versions Below 3.11 using IPX

---------------------------------------------------------

a. Using the correct shell Generation program (shgen, wsgen)

Select the NE2000 option

2. Installation for All Netware Versions supporting ODI

-------------------------------------------------------

1.   Things You Need

---------------

Before you start the installation  process  make

sure you have the following:

a)    Hardware Driver Disk

b)    Novell DOS ODI WORKSTATION SERVICES diskette.

The following drivers must be loaded in the exact order

LSL     - Found on the Novell DOS ODI WORKSTATION SERVICES diskette

NWMLID  - Found on the NIC-2002 Driver Disk

IPXODI  - Found on the Novell DOS ODI WORKSTATION SERVICES diskette

NETX    - Found on the Novell DOS ODI WORKSTATION SERVICES diskette

If the NIC driver, NWMLID executed successfully, it will inform you of

the adapter mode and related configurations.  Otherwise you will

see error messages telling you the reason the driver did not load.

Note: The ODI driver will always parse your NET.CFG file first.  If the

related  adapter's configuration  parameters are found, they

will be defaulted to IRQ 3 BASE 300 even though they

conflict with parameters stored in the EEPROM of adapter, thus

it will fail to initialize the adapter.

Below is a sample NET.CFG file:

LINK DRIVER NWMLID

INT   5

PORT  320

:

:

You can refer to the Netware Manuals for more information about NET.CFG

Novell has changed the Ethernet  default frame  type to 802.2

starting with the release of NetWare v4.0. The move has been made to

enable it to better support future protocols and networking features.

Some of these features are:  IPX with checksum, security enhancements,

and  protocol independence.  Following this change, the default frame

type of the DOS ODI driver is Ethernet 802.2 when there  is no protocol

binding in NET.CFG file.  With this situation, if  your server  is not

NetWare v4.0, please make sure the Ethernet 802.2 frame type has been

bound before logging in to it.

3. Novell NetWare V4.0x Server Driver Installation

--------------------------------------------------

Things you Need

---------------

Make sure you have the following:

1. Driver Diskette

2. Novell DOS ODI Workstation Services Diskette

Installation Procedure

----------------------

1. Insert the Driver Diskette into drive A: and check the contents of the

subdirectory \NETWARE\ODI\4.00. It should have the following files:

ACC40.LAN    ----> your Novell NetWare v4.0 Server Driver

ACC40.LDI    ----> your Novell NetWare v4.0 Server Driver

configuration file

2. At the NetWare prompt (indicated by the Server name), run

the INSTALL.NLM program by typing:

server name: LOAD INSTALL     <Enter>

3. Follow the instructions on screen to complete the process.

Press <Enter> to confirm each choice:

Maintenance/Selective Install                <Enter>

LAN Driver Options (Configure/Load/...)      <Enter>

4. Press the <Ins> key to insert other drivers you wish to install.

5. Press <F3> to display the driver loading path as follows:

A:\NETWARE\ODI\4.00          <Enter>

6. The ACC40.LAN will appear as an item under the

"Select a LAN Driver" menu.  Select it to load the driver

and start the binding process.  This will allow you to

load and bind all 4 frame types supported by NetWare.

7. Next time you reboot your NetWare 4.0x server, do so by

following steps 2 to 4 above.

Notes about the Novell v4.0x and the v3.1x drivers

--------------------------------------------------

1. The NetWare v4.0x and v3.11 drivers have different

architectures.  When using NetWare v4.x, you should

load the MSM.NLM and ETHERTSM.NLM modules

before loading server driver ACC40.LAN.

2. ACC40.LAN is your server driver for NetWare v4.0.  When

loading ACC40.LAN, it will check whether or not

these two modules (MSM.NLM and ETHERTSM.NLM) have been

loaded.  If not, it will load them automatically.

3. After the ACC40.LAN is loaded, type the "MODULES" command to

view and confirm that the additional modules, MSM.NLM and

ETHERTSM.NLM loaded.

4. The default frame type for ACC40.LAN binding in NetWare v4.0x

is Ethernet_802.2 and not Ethernet_802.3.