Wollongong PathWay Access installation with NE2000 Compatible NDIS driver

-------------------------------------------------------------------------

for

NIC-2002/UK series LAN Adapter

1. INSTALLATION INSTRUCTIONS

----------------------------

a. NDIS driver Installation Procedure

1. Insert your copy of the PathWay Drivers diskette into drive A.

2. To copy the software onto the system, enter :

A:\> pwinstal C:

where C: is the disk drive that you intend to use for the installation

of your PathWay software.  The PWINSTAL program automatically installs

the driver software in a new directory named \PATHWAY on the drive you

specified.

The diskette is finished copying when you receive the following

message:  "The PathWay Drivers have been copied successfully. Refer to

documentation to complete driver installation."

3. Insert the NIC-2002 Driver diskette into drive A: and copy the NDIS

driver to the subdirectory where you want to install the PathWay

software:

A:\> Copy A:\NDIS\ETHNE.DOS C:\PATHWAY

4. Edit the C:\CONFIG.SYS file to contain the following entries:

DEVICE=C:\PATHWAY\PWTCP.SYS

DEVICE=C:\PATHWAY\PROTMAN.EXE/I:C:\PATHWAY

DEVICE=C:\PATHWAY\ETHNE.DOS

5. Use any text editor to generate a PROTOCOL.INI file in C:\PATHWAY

which must have the following section:

[UK000x]

DriverName = ETHNE$

6. The startup batch file - AUTOEXEC.BAT should have the following entries

in the order shown:

path=c:\pathway

ndis [-d:#]

pwbind

where the -d option is used when accessing a specific module number

within PROTOCOL.INI.  In your PROTOCOL.INI file, a line enclosed by

square brackets designates the beginning of a new module.  Each module

is referred to by a hexadecimal number.  The first module is module

number one.  The second module is module two, and so on.

b. PathWay Kernel Installation Procedure

1. Insert your copy of the PathWay Kernel diskette into drive A

2. To copy the software onto the system, enter:

A:\> pwinstal C:

where C: is the disk drive that you intend to use for the installation

of your PathWay software.  The PWINSTAL program automatically installs

the driver software in a new directory named \PATHWAY on the drive you

specified.

The diskette copying is complete when you see the following message:

"PathWay Kernel files have been copied successfully.  Reboot and refer

to the documentation to complete kernel installation."

3. Remove the diskette from drive A: and restart your computer

4. Change the current directory to PATHWAY.

5. Run the CUSTOM program in order to set or change the following settings:

. The Internet address

. The subnet bits (if your network includes subnetworks)

. The gateway address (if you are communicating with hosts outside your

local network )

6. Remove the diskette from drive A: and restart your computer

7. Enter the following command to load the Kernel:

C:\PATHWAY> PWTCP

8. Test your PC Using PING program

C:\PATHWAY> PING hostname

Where hostname is the official name, alias, or Internet address of a

host on your network that you expect to be able to communicate with.

9. According to your needs, you can install the Character-based

application or the Windows-based application of PathWay.