DEC Pathworks installation with NE2000 Compatible NDIS driver

-------------------------------------------------------------

for

NIC-2002/UK series LAN Adapter

1. INSTALLATION INSTRUCTIONS

----------------------------

a. Insert the DEC PathWorks Client Setup diskette #1 into your

diskette drive (A: or B:)

b. Type 'NETSETUP' at A:> or B:> prompt, then press ENTER.

c. Unless you have an LK250 keyboard, Select "NO".

d. Select the DECNET transport option and press ENTER.

e. Insert the DNET TRNSPRT diskette #1 into the diskette drive (A: or B:).

f. Specify the destination drive letter (C or D)

g. Are you creating an initial workstation diskette? Select "NO".

h. Type in the node name and address (i.e node number).  Press ENTER.

i. Type in the server node name and address (i.e node number).  Press ENTER.

j. Select "Ethernet" when prompted by "Select the network adapter type".

k. Select "(Other NDIS)" when prompted by "Select the network adapter".

l. Enter full drive, path and filename for NDIS driver:

A:\ or B:\MSLANMAN.DOS\DRIVERS\ETHNE\ACCNE\ETHNE.DOS

m. Enter full drive and path for the PROTOCOL.INI file:

A:\or B:\MSLANMAN.DOS\DRIVERS\ETHNE\ACCNE\

n. Select "WRITE KEY DISK" to start installation.

When prompted to copy files ETHNE.DOS and PROTOCOL.INI, insert

NIC-2002 driver diskette into the drive specified in step l & m

o. Follow the prompts until complete.

p. At the Configuration screen, press F10 to exit to C:\

q. Using a text editor to edit PROTOCOL.INI at DECNET sub-directory

remove the semicolon (;) from the statement "NI_IRQ = 5"

under [DATALINK], then check if the IRQ value in

the new [ETHNE.DOS] IRQ statement matches the NI_IRQ value

in the [DATALINK] statement and Adapter IRQ value when set.

Sample configuration files:

2.  CONFIG.SYS

shell=c:\command.com /P /E:526

device=c:\decnet\hiMEM.sys

device=C:\dos\EMM386.EXE ram 1024

dos=high, UMB

devicehigh=c:\decnet\protman.sys /I:D:\DECNET

devicehigh=c:\decnet\ethne.dos

rem device=c:\decnet\nddrv.sys

rem device=c:\decnet\npdrv.sys

FILES=50

BUFFERS=50

LASTDRIVE=v

STACKS=9,256

3.  PROTOCOL.INI file for Pathwork v 4.1

; PROTOCOL.INI file create by DNETOMO.V41 V4.1.060

; 			via DNETWIK.V41 V4.1.047

[protocol manager]

DRIVERNAME = PROTMAN$

[IPX4MAC]

DRIVER = IPX$MAC

BINDINGS	   = ETHNE.DOS

[DATALINK]

DRIVERNAME   = DLL$MAC

LG_BUFFERS   = 16

SM_BUFFERS   = 6

OUTSTANDING  = 32

HEURISTICS   = 0

BINDINGS	   = ETHNE.DOS

;Specify IRQ level used by workstations network adapter

NI_IRQ	   = 3

[ETHNE.DOS]

; PROTOCOL.INI for NE2000 Compatible Ethernet adapter

drivername = ETHNE$

; irq = NI_IRQ = Adapter IRQ Setting

irq = 3

iobase = 0x300

; End of this sample file

4.  AUTOEXEC.BAT

echo off

break on

cls

prompt $p$g

REM Insert any keyboard internationalization and character set information here.

if not exist \decnet\STARTNET.BAT goto nostartup

call \decnet\STARTNET

goto end

:nostartup

echo ** WARNING ** STARTNET.BAT file not found.  Network functions not performed

:end