Windows for Workgroups 3.10 installation with NE2000 Compatible NDIS driver

---------------------------------------------------------------------------

for

NIC-2002/UK series LAN Adapter

1. Installation Introduction for Windows for Workgroups

-------------------------------------------------------

a. Select "Control Panel" icon from the Main group in Windows.

b. Select "Network" icon.

c. Select "Adapters" item in "Option" dialog.

d. Press the  Add button.

e. Select "Unlisted or Updated Network Adapter".

"install driver" appears.

f. Key in the drive and path name for the driver diskette. (A:\ndis or B:\ndis)

g. Insert your Driver Disk in specified drive.  Press ENTER.

h. Press <OK>.

i. Press <close>.

j. Press <OK> twice.

k. Restart computer.

2. Example of CONFIG.SYS file

FILES=30

buffers=30

STACKS=9,256

DEVICE=c:\windows\HIMEM.sys

device=C:\WINDOWS\protman.dos /i:C:\WINDOWS

device=C:\WINDOWS\ethne.dos

device=C:\WINDOWS\workgrp.sys

LASTDRIVE=y

3. Example of PROTOCOL.INI file in Windows for Workgroup

[network.setup]

version=0x3100

netcard=uk000xd,1,UK000XD

transport=ms$netbeui,MS$NETBEUI

lana0=uk000xd,1,ms$netbeui

[protman]

DriverName=PROTMAN$

PRIORITY=MS$NETBEUI

[UK000XD]

DriverName=ETHNE$

[MS$NETBEUI]

DriverName=netbeui$

SESSIONS=10

NCBS=32

BINDINGS=UK000XD

LANABASE=0

4. Example of AUTOEXEC.BAT

C:\WINDOWS\net start

C:\WINDOWS\SMARTDRV.EXE

@ECHO OFF

PROMPT $p$g

PATH C:\WINDOWS;

SET TEMP=C:\WINDOWS\TEMP