Windows for Workgroups 3.11 installation with NE2000 Compatible NDIS driver

---------------------------------------------------------------------------

for

NIC-2002/UK series LAN Adapter

1. Installation Introduction for Windows for Workgroups

-------------------------------------------------------

a. Select "Network Setup" icon in Network group in Windows enviroment.

b. Press the "Drivers Button"

c. Press the "Add Adapter" Button in the Network Drivers window.

d. Select "Unlisted or updated Network Adapter" and press the OK button.

e. Insert your Driver Disk in specified drive, then press Enter.

f. Key in the specified drive and path name. (A:\wfw311 or B:\wfw311)

g. Select 'NE2000 Compatible EtherCombo/Coax/Pair -16'

h. Press <OK>.

i. Press <OK>.

j. Restart the computer.

Sample Configuration Files

--------------------------

1. CONFIG.SYS File

FILES=30

BUFFERS=30

LASTDRIVE=Z

DEVICE=C:\WFW311\HIMEM.SYS

DEVICE=C:\WFW311\IFSHLP.SYS

STACKS=9,256

2. AUTOEXEC.BAT File

C:\WFW311\SMARTDRV.EXE

C:\WFW311\net start

PATH C:\WFW311;

SET TEMP=C:\WFW311\TEMP

3. PROTOCOL.INI File on Windows for Workgroups 3.11

[network.setup]

version=0x3110

netcard=ms$uk000x,1,MS$UK000X,2

transport=ms$netbeui,NETBEUI

lana0=ms$uk000x,1,ms$netbeui

[MS$UK000X]

DriverName=MS2000$

INTERRUPT=3

IOBASE=0x300

[UK000X]

dapters=MS$UK000X

[NETBEUI]

BINDINGS=MS$UK000X

LANABASE=0

SESSIONS=10

NCBS=12

4. SYSTEM.INI

[386Enh]

netcard=acc311.386

===End===