******************************************************************************

*            Advanced NE2000 Compatible Card IBM LAN Server driver           *

******************************************************************************

A. Diskette Introduction :

==========================

This sub-directory contains the following structures:

LANSRVR

�

�

��� DOS         : Contains IBM LAN SERVER driver for DOS.

�    �

�    ��� 23     : Conatins IBM LAN SERVER V2.x & V3.x driver and

�    �            installation file.

�    ��� 40     : Contains IBM LAN SERVER V4.0 driver and installation

�

��� OS2         : Contains IBM LAN SERVER driver for OS/2.

B. Installation Guildline:

==========================

FOR DOS:

--------

1.  Insert the Advanced NE2000 Compatible Card driver diskette

into floppy drive.

2.  In the directory the IBM LAN Server program located, press

"INSTALL" to install the driver for Advanced NE2000 Compatible

Card.

3.  In the dialog box, select "Network card" and press ENTER

to continue.

4.  In the dialog box, select "Change driver for network card."

and press ENTER to continue.

5.  In the dialog box, select "Network card not shown in list

below ..." and press ENTER to continiue.

6.  To specify the drive and path to Advanced NE2000 Compatible Card

driver diskette.

(e.g.,"A:\LANSRVR\DOS\40").

7.  The "Advanced NE2000 Compatible Card" is shown in the

listed dialog box.

8.  If you need to change the configuration, select "Edit settings

for network card driver." to change configuration such as IOBASE

or INTERRUPT number.

9.  After you installed completely, in DOS prompt copy the driver,

PNPND.DOS, to the hard disk from the directory the driver

located.

FOR OS/2:

---------

1.  Insert the Advanced NE2000 Compatible Card driver diskette into

floppy drive.

2.  In IBM LAN Server, select or run the MPTS (Multi Protocol

Transport Service) program. refer to the IBM LAN Adapter and

Protocol Support Configuration Guide for more information on

this utility and for more details on how to install additional

adapters and device drivers.

3.  From the IBM MPTS dialog box, choose the "INSTALL" button to

install a Advanced NE2000 Compatible ethernet card for IBM LAN

Server environment.If the NE2000 Compatible ethernet card has

already been installed, go to setp 6 below to continue.

4.  Specify the drive and path (e.g., "LANSRVR") to the Advanced

NE2000 Compatible Card driver diskette (e.g., "A:\LANSRVR\OS2").

5.  After installing/copying the appropriate NIF files from Advanced

NE2000 Compatible Card driver diskette, IBM MPTS dialog box will

appear again.

6.  From the IBM MPTS dialog box, select the "Configure".

7.  In the Configuration dialog box, choose the "Configure LAN

transports"(default) option and select "Continue".

8.  When the Configure Workstation dialog box is displayed, remove

the current configuration from the Current Configuration list

box.

9.  Choose the "Advanced NE2000 Compatible Card" from the Network

Adapters list box, by highlight the entry. Then choose "Add" the

Advanced NE2000 Compatible Card to the Current Configuration list

box.

10. Highlight the appropriate protocols in the Protocols list box.

Then choose "Add" to add them to Current Configuration list box.

11. Choose "OK" to finish installation.

======================================================================