******************************************************************************

*             Advanced NE2000 Compatible Card -- NDIS DOS Driver             *

******************************************************************************

A. Diskette Introduction :

==========================

This sub-directory contains the following structures:

MSLANMAN.DOS

|

|

|---DRIVERS

|

|--NIF         : Contains the Network Information File (NIF)

|                that setup needs.

|--ETHERNET

|

|--GE3ND  : Contains the Advanced NE2000 Compatible Card NDIS

DOS driver and protocol.ini file that could be

used for setup.

B. Installation Guildline:

==========================

1. To change directory to LANMAN.DOS and run Microsoft Lan Manager

SETUP program. In the DOS prompt key in:

C:\LANMAN.DOS>SETUP

2. Press Alt+C to select the "Configuration".

3. Press "D" or move the cursor to "Network Driver" item and press

ENTER.

4. Press Alt+A to select <Add New Config> .

5. In the "Network Adapter Drivers" menu, press Alt+D to select other

driver.

6. Insert the Advanced NE2000 Compatible Card driver diskette to the

floppy disk A, and press ENTER.

7. In "Other Network Adapter Drivers" menu, select "Advanced NE2000

Compatible Card" and press ENTER.

8. Select the network protocol that you want and press ENTER.

9. To verify the workstation configuration is correct and press ENTER.

10. Select "SAVE" to save the Lan Manager configuration. It will update

the AUTOEXEC.BAT and CONFIG.SYS files.

11. Exit Lan Manager setup program and reboot the system.

======================================================================