******************************************************************************

*              Advanced NE2000 Compatible Card -- NetWare Driver             *

******************************************************************************

A. Diskette Introduction :

==========================

This sub-directory contains the following structures:

NETWARE

�

�

���-DOSODI      : Contains the .COM execution file for NetWare ODI

�                Workstation for V3.x, V4.x.

�

���-NW31x       : Contains the NetWare V3.11 and 3.12 server driver

�

���-NW4XX       : Contains the NetWare V4.XX server driver

B. Installation Guildline:

==========================

The installation procedure is like as below:

NOVELL Workstations:

---------------------

* NetWare 386 V3.XX :

Copy the .COM and NET.CFG files in sub-directory "DOSODI" to the

working diskette or directory and execute the .COM file follow

the command listing :

LSL.COM

GE3ODI.COM

IPXODI.COM

NETX.EXE

Therefore, you had attached the server and you must

LOGIN the file server.

Where the LSL.COM, IPXODI.COM, NETX.EXE are all supported

by Novell.

* NetWare V4.X :

1. Copy the .COM and .INS files in sub-directory "DOSODI" to the

Wetware client directory \CLIENT\DOSWIN\DOS>

2. Run NetWare Client Installation file INSTALL.BAT under DOSWINDOW

sub-directory \CLIENT\DOSWIN>

3. Follow the installation instruction from Step 1.

4. Select "Advanced NE2000 Compatible Card" on the menu bar for LAN

card driver.

5. Execute the following commands under \NWCLIENT> directory.

LSL.COM

GE3ODI.COM

IPXODI.COM

VLM.EXE

Therefore, you had attached the server and you must

LOGIN the file server.

Where the LSL.COM, IPXODI.COM, VLM.EXE are all supported

by Novell.

NOVELL Server:

--------------

* NetWare 3.11

1. Insert the Advanced NE2000 Compatible Card driver diskette into

fllopy disk drive.

2. Copy all files from the \NETWARE\NW31x subdirectory on driver

diskette to the directory the NetWare 3.11 systm files located.

3. Boot NetWare by excuting SERVER.EXE. Type the following command

at the file server console:

LOAD LSLENH.NLM

LOAD MSM31X.NLM

LOAD ETHERTSM.NLM

LOAD GE3SRV

BIND IPX TO GE3SRV

* NetWare 3.12

1. Insert the Advanced NE2000 Compatible Card driver diskette into

fllopy disk drive.

2. Copy all files from the \NETWARE\NW312 subdirectory on driver

diskette to the directory the NetWare 3.11 systm files located.

3. Boot NetWare by excuting SERVER.EXE. Type the following command

at the file server console:

LOAD MSM31X

LOAD ETHERTSM

LOAD GE3SRV

BIND IPX TO GE3SRV

* NetWare 4.XX

1. Insert the Advanced NE2000 Compatible Card driver diskette into

fllopy disk drive.

2. Copy all files from the \NETWARE\NW4XX subdirectory on driver

diskette to the directory the NetWare 4.XX systm files located.

3. Boot NetWare by excuting SERVER.EXE. Type the following command

at the file server console:

LOAD GE3SRV

BIND IPX TO GE3SRV

======================================================================