******************************************************************************

*            Advanced NE2000 Compatible Card -- SunSoft PC-NFS driver        *

******************************************************************************

Installation Guildline:

=======================

1.  Be sure PC-NFS is installed on your system and configured for an

NDIS 2.01 driver. See the PC-NFS Installation Procedure Manuals

for more imformation.

2.  Run NFSCONF. Select "Adapter" Menu. Select "Ethernet." Select "NDIS."

3.  Insert the PC-NFS QUIKNDIS diskette into floppy drive. Change to the

drive containing the QUIKNDIS diskette (e.g., "A:\").

4.  At the prompt type:

QUIKNDIS

Then press <Enter>.

5.  A message will appear asking for current boot drive. Enter the current

boot drive and then press <Enter>.

6.  Press "Y" to save the current PROTOCOL.INI file as PROTOCOL.OLD.

Ignore this step if this dialog box does not appear.

7.  In the Adapter Card List box, select the "Custom" option to install

the NDIS driver for PC-NFS.

8.  Remove the QUIKNDIS disk from the floppy drive. Insert the driver

diskette into the floppy drive.

9.  Follow the dialog boxes and enter the correct driver and path on the

driver diskette to install the NDIS driver (e.g., A:\PCNFS").

10. Enter "GE3ND" for the NIF file name and press <Enter>.

11. Make the appropriate changes to the CONFIG.SYS and AUTOEXEC.BAT files.

12. Reboot your system.

The installation of the NDIS driver for PC-NFS is now complete.