Sampled Sounds for Musical Use

README.DOC

Copyright (c) 1993 Wall of Sound Productions

Introduction:

Various discussions in the MIDI Forums on CompuServe have dealt with the

new sample playback sound cards available from increasing number of

vendors.  These cards are shipped with a General MIDI set of sampled

sounds either in ROM or loadable into onboard RAM.  The RAM based cards

include the Gravis Ultrasound card (GUS) and the soon-to-be released

MAUI card (which has both ROM and RAM--up to 8 meg of RAM) from Turtle

Beach.  In addition there are ROM only cards and many 16-bit sound cards

that will play .WAV files including the Turtle Beach Multisound and

future Tahiti cards, PAS16, ARIA, SBPRO16 and others.

As a test, as well as an example of good quality samples, we have

included with this README.DOC file various sound and support files to be

used with the RAM based sound cards and other 16-bit sound cards.  Note

that due to the inherently large size of certain sample files we have

prepared two ZIP files named WOSPT1.ZIP and WOSPT2.ZIP.  This allowed us

to split the file sizes in half.  The list of files are below:

Name:              Size:   Description:

-----              -----   ------------

WOSPT1.ZIP

CELLOE1.CFG           18   Configuration file for a GUS MIDI file

CELLOE1.MID          706   MIDI file to demonstrate GUS Cello patch

CELLOE1.PAT        64015   Looped Cello patch in Gravis GUS format

CELLOE1.SMP        64331   Looped Cello patch in Sample Vision format

CELLOE1.WAV        63724   Cello sample in .WAV file format

808KICK.CFG           92   Configuration file for GUS MIDI file

808KICK.MID          622   MIDI file to demonstrate GUS Kick Drum patch

808KICK.PAT        64379   TR808 Kick Drum patch in Gravis GUS format

808KICK.SMP        69713   TR808 Kick Drum patch in Sample Vision format

808KICK.WAV        69428   TR808 Kick Drum sample in .WAV file format

README.DOC          7638   This file

WOSPT2.ZIP

DANCE.CFG             22   Configuration file for GUS MIDI file

DANCE.MID            636   MIDI file to demonstrate GUS Dance percussion patch

DANCE.PAT          64287   Dance percussion patch in Gravis GUS Format

DANCE.SMP         157713   Dance percussion patch in Sample Vision format

DANCE.WAV         157428   Dance percussion sample in .WAV file format

README.DOC          7638   This file

Discussion:

The intent of this exercise was to show the close relationship between

sound samples in three different, but usable formats, i.e., GUS Patch,

Sample Vision Patch and .WAV sample.  The basic building block in all

cases was the original .WAV file.  We sampled these sounds with the

Turtle Beach Multisound card because of its high quality sampling

capabilities.  We then used various commercial and custom programs to

convert and edit the files producing the final GUS .PAT patch files and

Turtle Beach .SMP Sample Vision patch files.  Note:  Both .WAV and .SMP

files will be usable with the Turtle Beach Maui sound card when it is

released.

All samples originally were 16-bit, 44,100 Hz sound files.  We were

forced to downsample the DANCE.WAV file to 18,140 Hz before conversion

to a .PAT file.  The reason for the 18,140 Hz rate was to keep the file

under the 64K size limit imposed by the GUS software for any one sample

within a larger patch file.  Yes, you can have up to 16 separate samples

in a GUS patch file but no individual sample can be over 64K.

Notes about various sample files:

The DANCE.PAT file is a percussion loop that plays back at 134 beats per

minute (BPM) at note F3 on a MIDI keyboard.  Obviously if played at

higher notes, both the BPM and pitch will increase accordingly, and

decrease for lower notes.  If played at A#3 the new BPM will be 179 (F3

= 174.614 Hz, A#3 = 233.081 Hz) with said pitch increase.  A software or

hardware pitch shifter can be used to return the pitch to is original

frequency while playing at a faster rate.  Playing octaves is a quick

way to find BPMs that are multiples of two greater or less than the

original for purposes of layering the loop or extending the beat with a

new sound.  Laying in a synth pad or hook over the top of a loop such as

this one might be all that needs to be done to cut vinyl at a level many

people are now producing (although most of this is of dubious musical

value, there are exceptions).  Note:  this sample can be looped, but to

maintain rhythmic fidelity in your sequencer, it should be retriggered

at the beginning of every measure or where desired with the appropriate

BPM setting on your sequencer.

The CELLOE1.PAT file is from an acoustic string Cello and was sampled at

E1 (root note).  The usable range for this sample in the PAT file was

set to be from C1 through E3.  This is stretching the range and gives

the patch a cheesy sound in the higher note ranges.  Preferably, the

Cello should be multi-sampled across its usable range to sound

realistic.  We have the full set of samples across the usable range of a

Cello to provide a smooth transition between note ranges.

The 808KICK.PAT, 808KICK.SMP and 808KICK.WAV files contain the classic

analog bass drum sound of the Roland TR808 drum machine.  This is the

sound (along with its close cousins) that has driven dance music of the

80s and 90s, in particular techno and house.

Who is Wall of Sound Productions?:

We are a new company formed to support the new generation of RAM based,

sample playback, sound cards for the PC compatible platform.  Our

initial offerings are custom sample patches developed to expand the

General MIDI wave-sample patches available on these cards, stored in ROM

or loaded into RAM.  We currently have many megabytes of samples in our

library, all 16-bit sampled initially at 44.1KHz to be used in the

creation of the sounds you want.

Format & Pricing:

Packaging is currently on 720K diskettes formatted for PC-DOS and other

formats on request.  Current file types supported are Microsoft Wave

.WAV files, Ultrasound .PAT files, Sample Vision .SMP files and Sound

Designer .DIG files.  Pricing is $12 per diskette; each disk is as full

as possible.  All necessary information regarding the disk contents,

supported note ranges for each sample, whether the sample is looped and

availability of other sample sets are all in a separate Readme file.

Quantity pricing is available.  More ordering information will be

released in the near future.  For a current price list and availability,

contact Craig or Todd at:

Wall of Sound Productions

P.O. Box 20301

Columbus, OH  43220-0301

(614) 848-6052

Now available:

>>>>Percussion<<<<

* TR808

* TR707

* TR909

* Techno-1

* Ambient-1

* ClassicElectronic-1

* OrchestralPercussion-1

>>>>>>Bass<<<<<<<

* TB303

* MoogBass

>>>>>>Loops<<<<<<

* Superloops-1

>>>>>Analog<<<<<<

* SH101

* Minimoog-1

* Juno106

* SuperJupiter-1

>>>>Acoustic<<<<<

* String-1

* Trumpet

* Trombone

* Clarinet

* Flute

* Piano-1

* OrchestralPercussion-1

>>>>>Vocal<<<<<<<

* Videogame-1

* JapaneseFemale-1

* ChineseFemale-1

>>Sound Effects<<

* VideoGame-2