This is the documentation file for TESTREC2.ZIP.  TESTREC2.ZIP contains

two stereo test recordings from the Gravis UltraSound soundcard.  These

stereo test recordings differ from the mono test recordings of TESTRECS.ZIP.

I made these recordings to compare the quality of two different recording

programs: PLAYFILE and GUSDELAY (a new, unreleased version).  This new,

soon-to-be released version of GUSDELAY uses a revolutionary new recording

technique that I developed (so I may be a little biased when I say

"revolutionary") which allows jitter-free and dropout-free recording with

the GUS.  (I also made some stereo test recordings with USS8, but they were

so bad that I didn't bother to include one.)

Each recording was made using the same test conditions.  The only difference

in the three tests was the software used for recording.  Here is a breakdown

of my test setup.

Computer: 25 MHz 386DX clone w/ 32 KB cache and 4 MB ram.

Bus Speed: 8 MHz

Hard Drive: Seagate ST-1096N (I think). 80 MB SCSI

SCSI Adapter: Seagate ST-01 (or ST-02, I can't remember). 8-bit only!

GUS DMA Setup: Record DMA on channel 5, Playback DMA on channel 6

Test Signal Source: (don't laugh) Casiotone MT-800's flute sound.  The note

used is the C above middle C, but the keyboard is probably way out

of tune.

Test Signal Level: Line level.

Sampling Frequency: 44100 Hz

Channels: 2 channels (i.e. stereo recording)

Notes: I fully defragmented the hard drive before each recording.  The test

recordings are stored in the headerless SND format.  This makes it easier

to play the sounds back at varying rates.  Playing the sounds back at

frequencies slower than (i.e. half) the sampling frequency makes any

imperfections in the recording stand out more.  The recordings are not all

the same length, but they are all approximately 5 seconds in length.

Recording in stereo uses twice as many samples per second as recording in

mono.  This fact makes the differences in quality of the three recording

programs much more apparent in the stereo recordings than in the mono

recordings.  For these recordings, I used the flute sound instead of the

pipe organ sound.  The flute sound of this keyboard has some vibrato in it

that can be heard in these recordings.  To play these sound files using

PLAYFILE at 44100 Hz, use the following command:

PLAYFILE -s -f<nnnnn> xx44100S.SND

Replace <nnnnn> with either 44100 or 22050.  Replace xx with PF or GD.

US44100S.SND was recorded using USS8.  This came out so badly that I didn't

even bother to include it.  If you really want to hear how it compares, let

me know and I'll arrange to get a copy of this recording to you.  Many loud

clicks can be heard in rapid succession.  This recording was MUCH noisier

than the USS8 mono recording.

PF44100S.SND was recorded using PLAYFILE.  This sounds better than

US44100S.SND, but many clicks can still be heard (although they are not so

loud or frequent as the clicks in US44100S.SND).  I assume these are due to

missed samples and/or "jitter".  This recording was MUCH noisier than the

PLAYFILE mono recording.

GD44100S.SND was recorded using GUSDELAY.  This is the best recording of the

three.  As far as I can tell, there are no clicks whatsoever.  This is due

to the revolutionary new recording technique that I developed.  This

recording sounds as clean and clear as the GUSDELAY mono recording (which

also had no clicks).

There are a few other files in TESTREC2.ZIP.  They are included to provide

the listener with a baseline test.  When listening to GD44100S.SND, I would

occasionally notice clicks.  Believing that these clicks (which were *very*

small and infrequent compared to the clicks in PF44100S.SND) were not in the

recording itself, I started to develop suspicions about the playback side of

my setup.  Maybe PLAYFILE has a bug in it that manifest itself when playing

large (>100K) files...maybe having my GUS on a card extender with

oscilloscope probes attached to it was introducing some noise...maybe my $7

"CD to cassette" adapter wasn't seated well in my boom box...you get the

idea.  In order to test these theories, I wrote the QBASIC program

SINE1.BAS.  Running SINE1.BAS will produce a file (SINE1.SND) that contains

one cycle (in stereo) of a sine wave.  This sine wave contains 44 samples.

BIGSINE.BAT is a batch file that uses the DOS copy command to create other

SND files that contain more than one cycle of this sine wave.  The biggest

of the SND files created, SINE8192.SND, contains 8192 cycles of this sine

wave.  Playing these SND files back at 44100 Hz (in stereo) produces a tone

that is very close to 1 kHz.  Well, I haven't been able to isolate any

problems that occur, but when I hear clicks in GD44100S.SND, I also hear

clicks in SINE4096.SND and SINE8192.SND.  This leads me to believe that the

problem is somewhere in the playback side.  In fact, when listening to

GD44100S.SND at 22050 Hz, I never hear any clicks.  When listening to

PF44100S.SND at 22050 Hz, however, I still hear the click (they're just

twice as long).

Please let me know what you think about the recordings.  I am interested in

the opinions of unbiased ears.

I can be reached via internet e-mail.  My address is davidm@marcam.com.

Dave

David MacMahon

Systems Administrator

davidm@marcam.com